3GPP TSG-RAN WG4 Meeting # 108bis		R4-2316230

Xiamen, China, October 09 – October 13, 2023

Agenda Item:	5.21.3

Source: 	OPPO

Title: 	On testability issues for two-sided AIML model

Document for:	Approval

- Introduction

In 3GPP RAN#94e meeting, a new study item (SI) on AI/ML for NR air interface in Rel-18 [1] was agreed. According to the SID, the study will focus on the general framework, evaluations for three typical use cases and other aspects relate to specification impacts.

RAN4 scope in the SID is listed as below:

- Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition

In the recent RAN1 meetings,

- A basic R18 (also targeting future 6G) AI/ML framework is considered, including

- Discussion on life cycle management (LCM), performance monitoring, data collection, model/data ID, AI capability, model privacy, etc.

- Six representative sub use cases have been identified, including

- CSI: CSI compression (two-sided model), Time domain CSI prediction

- Beam management: Spatial domain beam prediction, Temporal domain beam prediction

- Positioning: Direct AI/ML positioning, AI/ML assisted positioning

- Performance evaluation methodologies(EVM) are confirmed for the identified sub use cases, including

- Performance evaluation methodologies for different sub use cases

- Quantitative simulation results under different EVM assumptions for different sub use cases

In this contribution, we will discuss the RAN4 related issues, especially the testability of two-sided AI/ML models.

- Discussion

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

For example, a corresponding encoder/decoder model would be needed to cooperate with the decoder/encoder under test in order to monitor the decoding/encoding effect of the PMI. Only by well matching the models deployed on UE&NW can the performance of the AI/ML based two-sided solutions be guaranteed. Matching at different levels may result in different performance loss.

Proposal 1: Regarding the testability of two-sided model, should introduce test encoder(s) to collaborate with the  NW decoder under test.

Proposal 2: Regarding the testability of two-sided model, should introduce test decoder(s) to collaborate with the UE encoder under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

According to our understanding, the pros and cons of different options for test decoder are listed below,

Table 1 pros and cons of different options for test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | UE | NW | RAN4 spec | RAN4 spec and [TE] |
| Source of decoder training data | UE | NW | RAN4 spec | RAN4 spec and [TE] |
| DUT vendor knowledge of the test decoder | Full knowledge | No, need assisted indication of the test decoder knowledge | Full knowledge | Partially, may need assisted indication of the test decoder knowledge |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type1 and Type3, For example, UE first trains its encoder and a corresponding decoder, and then transmits the decoder to NW, or transmits a dataset that can assist NW in training its decoder to NW. Under these conditions, when testing UE side encoder, the NW side decoder provided by the UE can be used as the test decoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, For example, NW first trains its decoder and a corresponding encoder, and then transmits the encoder to UE, or transmits a dataset that can assist UE in training its encoder to UE. Under these conditions, when testing UE side encoder, the NW side decoder provided by the NW can be used as the test decoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. |
| Test decoder verification procedure at TE and/or DUT | No consensus | No consensus | No consensus | No consensus |
| Feasibility of test decoder verification procedure | No consensus | No consensus | No consensus | No consensus |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Depends on training data | Depends on training data | Depends on training data | Depends on training data |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | High, TE needs to support various test decoders specific to different devices, particularly when there are a multitude of test decoder providers(e.g. UEs). And also depends on the granularity of test decoder. | Relatively high, TE needs to support different test decoders from NW vendors. And also depends on the granularity of test decoder. | Medium, TE needs to provide support for a limited number of test decoders that specified and captured in RAN4 spec. And also depends on the granularity of test decoder. | Medium, TE needs to provide support for a limited number of test decoders that specified and captured in RAN4 spec. And also depends on the granularity of test decoder. |
| Specification Effort (e.g. test decoder) | Medium, The test may involve test model/data indication and delivery. | Medium, The test may involve test model/data indication and delivery. | High, Consensus of a test model(s) in RAN4 is a challenging task. | High, Consensus of a test model(s) in RAN4 is a challenging task. |
| Confidentiality/IP issues | With test decoder exposure in training collaboration type1. No model exposure in training collaboration type3. | With test decoder exposure in training collaboration type1. No model exposure in training collaboration type3. | No model exposure. | No model exposure. |
| Applicability to different scenarios/conditions/ configurations | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) |
| Complexity of actual testing procedure for the ecosystem | No consensus | No consensus | No consensus | No consensus |
| Friendly to STOA(state of the art) model test | Yes | Yes | No | No |
| Whether model transfer/delivery is needed during the test procedure | Yes | Yes | No | No |

Observation 1: Pros and cons for different options on test decoder are shown in table 1.

Regarding the test encoder, 4 corresponding options are listed as below.

- Option 1: test encoder is provided by the vendor of the decoder

- Option 2: test encoder is provided by the vendor of the encoder

- Option 3: The test encoder(s) are fully specified and captured in RAN4 spec

- Option 4: The test encoder(s) are partially specified and captured in RAN4 spec.

The pros and cons of different options for test encoder are listed below,

Table 2 pros and cons of different options for test encoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test encoder | NW | UE | RAN4 spec | RAN4 spec and [TE] |
| Source of encoder training data | NW | UE | RAN4 spec | RAN4 spec and [TE] |
| DUT vendor knowledge of the test encoder | Full knowledge | No, need assisted indication of the test encoder knowledge | Full knowledge | Partially, may need assisted indication of the test encoder knowledge |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type1 and Type3, For example, NW first trains its decoder and a corresponding encoder, and then transmits the encoder to UE, or transmits a dataset that can assist UE in training its encoder to UE. Under these conditions, when testing NW side decoder, the encoder provided by the NW can be used as the test encoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, For example, UE first trains its encoder and a corresponding decoder, and then transmits the decoder to NW, or transmits a dataset that can assist NW in training its decoder to NW. Under these conditions, when testing NW side decoder, the encoder provided by the UE can be used as the test encoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. |
| Test encoder verification procedure at TE and/or DUT | No consensus | No consensus | No consensus | No consensus |
| Feasibility of test encoder verification procedure | No consensus | No consensus | No consensus | No consensus |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Depends on training data | Depends on training data | Depends on training data | Depends on training data |
| TE requirements to deploy the encoder (e.g. training, complexity, interoperability) | High, TE needs to support different test encoders provided by different NW vendors. And also depends on the granularity of test encoder. | Relatively high, TE needs to support various test encoders specific to different devices, particularly when there are a multitude of test encoder providers (e.g. UEs). And also depends on the granularity of test encoder. | Low, TE only need to provide support for a limited number of test encoders that specified and captured in RAN4 spec. And also depends on the granularity of test encoder. | Low, TE only need to provide support for a limited number of test encoders that specified and captured in RAN4 spec. And also depends on the granularity of test encoder. |
| Specification Effort (e.g. test encoder) | Medium, The test may involve test model/data indication and delivery. | Medium, The test may involve test model/data indication and delivery. | High, Consensus of a test model(s) in RAN4 is a challenging task. | High, Consensus of a test model(s) in RAN4 is a challenging task. |
| Confidentiality/IP issues | With test encoder exposure in training collaboration type1. No model exposure in training collaboration type3. | With test encoder exposure in training collaboration type1. No model exposure in training collaboration type3. | No model exposure. | No model exposure. |
| Applicability to different scenarios/conditions/ configurations | Depends on the granularity of test encoder and the test data set(s) | Depends on the granularity of test encoder and the test data set(s) | Depends on the granularity of test encoder and the test data set(s) | Depends on the granularity of test encoder and the test data set(s) |
| Complexity of actual testing procedure for the ecosystem | No consensus | No consensus | No consensus | No consensus |
| Friendly to STOA(state of the art) model test | Yes | Yes | No | No |
| Whether model transfer/delivery is needed during the test procedure | Yes | Yes | No | No |

Proposal 3: Further study the pros and cons for test decoder and test encoder in RAN4.

In addition to the test model, the dataset that used for test model training/inference need to be considered as well. Options are listed below:

Option1: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”.

Option2: Field dataset

In RAN1, option1(data set based on TR 38.901) is utilized by companies to propose their evaluation results. For example, in CSI cases, the UMa channel model based on TR 38.901 is the data source for training and testing. As for the field data, companies can optionally provide their results based on it.

In RAN4, especially for the construction of test models and tests, evaluation assumptions in RAN1 can be considered. Data sets based on TR 38.901 can be utilized as the starting point.

Proposal 4: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, should be considered in RAN4 as the starting point.

Regarding the testability of AI/ML features, different AI/ML capabilities should be considered, e.g. capabilities that support different use cases, different scenarios and different models. In RAN4 AI/ML tests, following aspects need to be considered:

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities (e.g. AI capability being influenced by computing resources, transmission resources, and storage resources)

Proposal 5: Regarding the AI/ML capabilities, following aspects should be considered

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities

- Conclusions

In this contribution, we discussed the Rel-18 AI/ML impacts to RAN4 and got following proposals

Proposal 1: Regarding the testability of two-sided model, should introduce test encoder(s) to collaborate with the  NW decoder under test.

Proposal 2: Regarding the testability of two-sided model, should introduce test decoder(s) to collaborate with the UE encoder under test.

Observation 1: Pros and cons for different options on test decoder are shown in table 1.

Table 1 pros and cons of different options for test decoder

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | UE | NW | RAN4 spec | RAN4 spec and [TE] |
| Source of decoder training data | UE | NW | RAN4 spec | RAN4 spec and [TE] |
| DUT vendor knowledge of the test decoder | Full knowledge | No, need assisted indication of the test decoder knowledge | Full knowledge | Partially, may need assisted indication of the test decoder knowledge |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | Type1 and Type3, For example, UE first trains its encoder and a corresponding decoder, and then transmits the decoder to NW, or transmits a dataset that can assist NW in training its decoder to NW. Under these conditions, when testing UE side encoder, the NW side decoder provided by the UE can be used as the test decoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, For example, NW first trains its decoder and a corresponding encoder, and then transmits the encoder to UE, or transmits a dataset that can assist UE in training its encoder to UE. Under these conditions, when testing UE side encoder, the NW side decoder provided by the NW can be used as the test decoder. Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. | Type1 and Type3, Type2 is deprioritized in RAN1#111 meeting. |
| Test decoder verification procedure at TE and/or DUT | No consensus | No consensus | No consensus | No consensus |
| Feasibility of test decoder verification procedure | No consensus | No consensus | No consensus | No consensus |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | Depends on training data | Depends on training data | Depends on training data | Depends on training data |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | High, TE needs to support various test decoders specific to different devices, particularly when there are a multitude of test decoder providers(e.g. UEs). And also depends on the granularity of test decoder. | Relatively high, TE needs to support different test decoders from NW vendors. And also depends on the granularity of test decoder. | Medium, TE needs to provide support for a limited number of test decoders that specified and captured in RAN4 spec. And also depends on the granularity of test decoder. | Medium, TE needs to provide support for a limited number of test decoders that specified and captured in RAN4 spec. And also depends on the granularity of test decoder. |
| Specification Effort (e.g. test decoder) | Medium, The test may involve test model/data indication and delivery. | Medium, The test may involve test model/data indication and delivery. | High, Consensus of a test model(s) in RAN4 is a challenging task. | High, Consensus of a test model(s) in RAN4 is a challenging task. |
| Confidentiality/IP issues | With test decoder exposure in training collaboration type1. No model exposure in training collaboration type3. | With test decoder exposure in training collaboration type1. No model exposure in training collaboration type3. | No model exposure. | No model exposure. |
| Applicability to different scenarios/conditions/ configurations | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) | Depends on the granularity of test decoder and the test data set(s) |
| Complexity of actual testing procedure for the ecosystem | No consensus | No consensus | No consensus | No consensus |
| Friendly to STOA(state of the art) model test | Yes | Yes | No | No |
| Whether model transfer/delivery is needed during the test procedure | Yes | Yes | No | No |

Proposal 3: Further study the pros and cons for test decoder and test encoder in RAN4.

Proposal 4: Dataset based on TR 38.901, e.g. UMa channel, UMi channel, CDL channel, “legacy approach”, should be considered in RAN4 as the starting point.

Proposal 5: Regarding the AI/ML capabilities, following aspects should be considered

- Definition of basic AI/ML capability and corresponding testing metrics

- Definition of different AI/ML capability levels and different testing metrics for different levels

- Dynamic AI/ML capabilities

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#94-e, Dec. 2021