3GPP TSG-RAN WG4 Meeting # 108-bis 	R4-2316234

Xiamen, China, October 09th – 13th, 2023

Agenda Item: 5.9.2.3

Source: MediaTek Inc.

Title: Discussion on case 2 requirements (NCSG and concurrent MG)

Document for: Discussion

# Introduction

In existing Rel-15/16 NR, two measurement gaps have been identified, which are per-UE and per-FR measurement gap. Later in Rel-17 NR, three measurement gap enhancement have been considered, which are: (i) pre-configured MG pattern(s) per configured BWP (fast MG configuration), (ii) multiple concurrent and independent MG patterns, and (iii) network controlled small gap (NCSG). Now in Rel-18, further work objective to enhance the existing measurement gap is agreed on, which is ‘Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG’, given in the revised work item description (WID) [1] as below:

- Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Define requirements only for the following two cases for UE configured with: - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: This WID does not include any inter-working with MUSIM gaps

In the previous RAN4 107 meeting, the issues are captured in the way forward (WF) [2]. The analysis and discussion on the issues from the WF are provided in the next section.

# Discussion

From the WF, the issues are for: (i) gap combination, (ii) UE capabilities, and (iii) other Rel-17 rules.

## Discussion on gap combinations

The open issues are given as:

### Issue 4-1-1: [Case 2] Whether per FR PRS gap should be considered in case 2 Concurrent gaps with NCSG? < Way forward >: - FFS: Yes, a Per-FR Type-2 gap can be used to perform PRS measurements in [case 2] Concurrent gaps with NCSG provided that the UE supports independentGapConfigPRS-r17. NCSG cannot be used to perform PRS measurements. ### Issue 4-1-2: [Case 2] is subject to UE capability ncsg-MeasGapPerFR-r17? < Way forward >: - FFS: the support of gap combinations in TS 38.133 table 9.1.8-1 should be subject to UE capability ncsg-MeasGapPerFR-r17.

Issue 4-1-1: RAN4 support the following scenarios: (i) Type-2 MG + NCSG, (ii) NCSG + NCSG. Given that NCSG requirements doesn’t support measuring PRS then scenario (ii) is not supported. Yet, for scenario (i) the UE can still use Type-2 MG (but not NCSG) for measuring PRS gap as per FR.

Issue 4-1-2: the proposed way forward is agreeable.

- a Per-FR Type-2 gap can be used to perform PRS measurements in [case 2] Concurrent gaps with NCSG provided that the UE supports independentGapConfigPRS-r17. NCSG cannot be used to perform PRS measurements.

- the support of gap combinations in TS 38.133 table 9.1.8-1 should be subject to UE capability ncsg-MeasGapPerFR-r17.

## Discussion on UE capabilities

The open issues of UE capabilities are given as:

## Sub-topic 4-2: UE capabilities for Concurrent gaps with NCSG and NCSG + NCSG ### Issue 4-2-1: [Case 2] Whether to consider a new capability for Concurrent gaps with NCSG in an FR? < Agreement >: - Yes, a Rel-18 UE capability. ### Issue 4-2-2: [Case 2] Whether to consider an additional capability for NCSG + NCSG in an FR? < Way forward >: - FFS the following options: - Option 1: - No, without additional UE capability. - Option 2: - Yes, with additional UE capability

Furthermore, on issue of UE capability for NCSG + NCSG: In the meeting RAN4#105, RAN4 agreed to consider the scenario of NCSG + NCSG in an FR. However, it was also mentioned during the discussion that the necessity to introduce this scenario is not high. This is because NCSG can be seen as a supplement to traditional MG and it is expected to have gap as common case. Besides, it is common in RAN4 to introduce a UE capability when there is a feature with tight requirements that normal UE cannot achieve or a feature that is not typical for all NR UE. Therefore, RAN4 should support this with UE capability (i.e. it is left to the UE implementation). Furthermore, the use of NCSG + NCSG scenario is different than the scenario of NCSG + type-2 MG, hence, RAN4 should define two separate capabilities to indicate the support of these two scenarios.

- RAN4 shall support two separate UE capabilities for the scenarios of (i) NCSG + NCSG in an FR, and (ii) Type-2 MG + NCSG in an FR.

## Discussion on other Rel-17 rules to be revisited

The agreements from previous meetings are given as:

### Previous agreements on potential changes for NCSG upon SCell activation: < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

The open issues are captured as:

### Issue 4-3-1: [Case 2] Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG? (Clarify Rel-17 understanding) < Way forward >: - This issue was handled in a different email thread in the Rel-17 MG maintenance #202 in meeting RAN4#108. ### Issue 4-3-2: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18) < Way forward >: - FFS the following options: - Option 1: - Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association. - Option 3: - A new indication shall be introduced enable support of NCSG for deactivated SCell only. - Option 3a: - If UE supports the new indication of NCSG for deactivated SCell, then skip gap association, i.e., the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. Otherwise, still follow the gap association, i.e., deactivated Scell MO associated with NCSG is measured within NCSG and deactivated Scell MO not associated with NCSG is measured outside NCSG. - Option 4: - The Rel-18 UE behavior (assume SMTC partially overlapped with NCSG) can follow the gap association, i.e., deactivated SCell MO associated with NCSG is measured within NCSG and the MO not associated with NCSG is measured outside NCSG. ### Issue 4-3-3: [Case 2] Whether a new UE capability is needed for the support of NCSG for deactivated SCell? < Way forward >: - FFS the options: - Option 1: No - Option 2: Whether to introduce additional UE capability in Rel-18 is dependent to whether deactivated Scell MOs could be measured within NCSG regardless intraFreq-needForNCSG report in Rel-17.

Issue 4-3-1/4-3-2/4-3-3:

Given that this issue is currently handled in the maintenance phase of Rel-17 the issues 4-3-1/4-3-2/4-3-3 shall be postponed until the Rel-17 understanding is clarified.

- RAN4 shall wait for the outcome of the issue on whether all deactivated Scell  will be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG from Rel-17 maintenance on Rel-17 MGE.

# Summary

In this contribution, discussion on NCSG and concurrent MG is provided, and we have the following proposals:

Proposal 1: a Per-FR Type-2 gap can be used to perform PRS measurements in [case 2] Concurrent gaps with NCSG provided that the UE supports independentGapConfigPRS-r17. NCSG cannot be used to perform PRS measurements.

Proposal 2: the support of gap combinations in TS 38.133 table 9.1.8-1 should be subject to UE capability ncsg-MeasGapPerFR-r17.

Proposal 3: RAN4 shall support two separate UE capabilities for the scenarios of (i) NCSG + NCSG in an FR, and (ii) Type-2 MG + NCSG in an FR.

Proposal 4: RAN4 shall wait for the outcome of the issue on whether all deactivated Scell  will be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG from Rel-17 maintenance on Rel-17 MGE.

# References

- RP-231477, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, June 2023, Taipei Taiwan.

- R4-2314323, “WF on NR MG enhancements (Part 1)”, MediaTek Inc, Toulouse, Fr, August 2023.