3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316247

Xiamen, China, 9 – 13 October 2023

Source:	Ericsson, Rogers

Title:	TP for TR 38.718-03-01: to include band combination CA_n12A-n71A-n77A

Agenda item:

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.718-03-01to add CA_n12A-n71A-n77A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.x	CA_n12A-n71A-n77A

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n12+n71+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n71A-n77A | CA_n12A-n77A CA_n71A-n77A | n12 | 5, 10, 15 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n12-n71-n77, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n12-n71-n77 | 1 | 1 | 0.8 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n12-n71-n77 | 0.8 | 0.8 | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations: CA n12-n77and CA_n71-n77 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings:

–   5th order IMD generated by dual uplink of Band n77 + Band n12 may fall into own Rx of Band n71.

–   5th order IMD generated by dual uplink of Band n77 + Band n71 may fall into own Rx of Band n12.

#### 5.x.2.2	REFSENS requirements

Table 5.x.2.2-1 lists the MSD required for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5A-n12A-n77A are reused.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n12-n71-n77 | n12 | N/A | 5 | 25 | 732 | 4.4 | FDD | IMD5 |
|  | n71 | 693 | 5 | 25 | 647 | N/A | FDD | N/A |
|  | n77 | 3504 | 10 | 50 | 3504 | N/A | TDD | N/A |
|  | n12 | 711 | 5 | 25 | 741 | N/A | FDD | N/A |
|  | n71 | N/A | 5 | 25 | 646 | 3.9 | FDD | IMD5 |
|  | n77 | 3490 | 10 | 50 | 3490 | N/A | TDD | N/A |

---End of changes---

# Reference

[1]		RP-231860, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2), ZTE