3GPP TSG-RAN WG4 #108bis	R4-2316266

Xiamen, China, October 9th – 13th, 2023

Agenda item:	4.1.1.1

Source: 	Qualcomm France

Title: 	MSD for UL CA_n5B

Document for:	Approval

# Introduction

Analysis results for CA_n5B are presented in this contribution

# Discussion

WF for was agreed in RAN4#108 [1]:

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | [FFS] | [FFS] | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

We analysed MSD according to the WF. We used both PA measurements and quantitative methods to construct the analysis.  For PCC, the dominating factor is IMD3. For SCC, the dominating factor is IMD5. We note that the MSD for SCC is significantly smaller not only because of IMD3 vs IMD5, but also because standalone 20MHz CC has reduced baseline REFSENS due to TX impairments.

# Conclusions

Analysis results and considerations on UL CA_n5B were provided with the following proposals.

Proposal 1: Consider the following MSD evaluations when specifying MSD for UL CA_n5B

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5Bx | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 43.4 | 20.6 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# Reference

[1]	R4-2314855, WF on CA_n5B DL/UL BCS1 MSD test point, Skyworks