3GPP TSG-RAN WG4#108bis	R4-2316267

Xiamen, China, October 9th – 13th, 2023

Agenda item:	4.1.1.1

Source: 	Qualcomm France

Title: 	MSD for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A

Document for:	Approval

# Introduction

Analysis and proposals on CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A MSD are provided in this contribution.

# Considerations

The following was agreed in WF [1]:

- The following option is presented as a WF.

  - Option 1: Use UL configuration 1RB+ 1RB (Equal PSD) to align with existing specification in 38.101-1

    - Finalize IMD order and DL frequency test point if necessary. Adjust RB start position if necessary to center IMD noise. Determine MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | [743.5] | TBD | FDD | [IMD9] |
| CA_n5-n14 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | [765.5] | TBD | FDD | [IMD7] |
| CA_n5-n29 | n5 | 829 | 10 | 1 (RBSTART=[0]) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=[52]) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | [725.5] | TBD | FDD | [IMD11] |

  - Option 2: Other options not precluded for further information.

RBstart=[52] is not applicable, instead RBstart=[51] is applicable when 1RB is placed at uppermost corner of 10MHz CC.

With 1+1 RB UL allocations, the location of the IMD product including it’s adjacent channel impacts has a huge impact to MSD; If fully contained, then MSD is very high, but if only partially contained the respective MSD is a lot smaller.

For all the cases above, if we still assume 1+1 RB allocation, shifting UL RB start positions or victim DL frequency would not result in worse case; hence we used the following test points in our analysis:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | TBD | FDD | IMD9 |
| CA_n5-n14 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | TBD | FDD | IMD7 |
| CA_n5-n29 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | TBD | FDD | IMD11 |

We used the conventional 3GPP assumptions for the TX for the PA calibration.

- 20 MHz 100RB DFT-s-OFDM

- 30dBc ACLR

- 4dB Post-PA loss

- Image -28dB

- LO -28dBc

- CIM3 -60dBc

- CIM5 -70dBc

The parameters and MSD results are shown below. We measured the TX noise at PA output and added 3dB margin due to many uncertainties associated.

For comparison, we also did analyse the tests points with 10+10 UL RB’s [2], with the following results:

-9.2dB n12

-11.7dB n14

-7.5dB n29

The reason for big differences between 1+1 and 10+10 RB are related to the amount of IMD overlap and width of the IMD bump including ACLR effects. For 10+10RB case, the IMD bump reaches further than respective 1+1RB IMD bump because of the ACLR impacts.

For n14, 1+1 RB surely is the worst case as the narrow IMD7 is entirely within the 5MHz BW. For n12 and n14, 1+1 RB is not the worst case, as e.g. 10+10 RB results in larger MSD’s. Big question is whether RAN4 should seek for the ultimate worst case, given that in practice it would be very rare case when Intra-band contiguous UL CA is scheduled using non-contiguous RB allocations which effectively destroy also own DL (n5 PCC, SCC in this case). In our view, it is not meaningful to seek for the ultimate worst case, e.g. by choosing random number of UL RB’s. Initially we think that MSD for these cases should either be specified using 1+1 or 10+10 RB, preferably using 10+10RB.

Proposal 1: Further discuss in RAN4 if reverting back to 10+10 RB UL allocation for CA_n5B would be most meaningful approach given 1+1 RB gives very inconsistent results.

# Conclusion

MSD analysis was conducted for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A

Proposal 1: Further discuss in RAN4 if reverting back to 10+10 RB UL allocation for CA_n5B would be most meaningful approach given 1+1 RB gives very inconsistent results.

Proposal 2: Use the following analysis results when discussing requirements for CA_n5B-n12A, CA_n5B-n14A, and CA_n5B-n29A if 1+1RB allocation is eventually used.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n12 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n12 | N/A | 5 | N/A | 743.5 | 4.4 | FDD | IMD9 |
| CA_n5-n14 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n14 | N/A | 5 | N/A | 765.5 | 34.3 | FDD | IMD7 |
| CA_n5-n29 | n5 | 829 | 10 | 1 (RBSTART=0) | 874 | N/A | FDD | N/A |
|  |  | 839 | 10 | 1 (RBSTART=51) | 884 | N/A |  |  |
|  | n29 | N/A | 5 | N/A | 725.5 | 6.4 | FDD | IMD11 |

# Reference

[1] R4-2314856, WF on CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29 MSDs with CA_n5B UL

[2] R4-2311269, CA_n5B ULCA MSD due to IMD for CA_n5B-n12 CA_n5B-n14 and CA_n5B-n29