3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316302

Xiamen, China, October 9-13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	TP to TR 38.858: Feasibility of FR2 BS aspects

Agenda item:	5.19.2.2.2

Document for:	Discussion

Introduction

In RAN4 #108, a draft TR 38.858 was endorsed in [1], containing companies’ views on the feasibility of SBFD for FR1 and FR2. In this contribution, we include Nokia’s views on the feasibility of SBFD for FR2 BS, including a TP to TR 38.858.

TP to TR 38.858

< Start of TP >

## 9.5	Feasibility of FR2-1 BS aspects

### 9.5.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability and analysis results.

### 9.5.1.1	Summary table for self-interference analysis

The self-interference analysis from companies’ inputs for FR2 SBFD-capable gNB are summarized in Table 9.5.1.1-1. Both self-interference leakage in gNB RX sub-band due to non-ideal TX and Self-Interference signal in gNB RX sub-band caused by non-ideal RX selectivity are studied in the analysis framework. The self-interference cancellation techniques including spatial isolation, frequency isolation, beam nulling and digital/RF cancellation are considered.

Table 9.5.1.1-1: FR2-1 BS Self-interference Analysis Summary

|  |  |  |  | Samsung | Huawei | Qualcomm | Ericsson |  |  | Nokia |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | FR2-1 BS | FR2-1 BS | FR2-1 BS | FR2-1 BS |  |  | FR2-1 BS (preliminary) |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm | 35 dBm | 30 dBm | 40 dBm | 35 dBm | 30 dBm | 37 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc | 28 dBc |
|  |  | Frequency isolation techniques used |  | Without DPD | DPD | DPD | Digital Filtering, CFR |  |  | Without DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 87 dBc | 85-95 dBc | 85-95 dBc | 80 dBc | 80 dBc | 80 dBc | 80 dBc |
|  |  | Spatial isolation techniques used |  | TX/RX panel separation and RF barrier structure | Spatial separation between TX/RX panel, with absorbing material and choke structure. | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation | A combination of spatial isolation, chokes, absorption, mushroom EBG. |  |  | Separate TX and RX panels with absorptive RF barriers, physical walls and chokes |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dB | 10 dBc | 5-10 dBc | 10 dBc | 10 dBc | 10 dBc | 10 dBc |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Limited, ~0dB | Less than 0.5 dB loss |  | Up to 5dB EIRP loss, depending on beam direction |  |  | < 0.5 dB |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -95 dBm Note: provided by ①-②-③-⑨ dBm | -94~-104 dBm | -88 dBm | -78 dBm | -83 dBm | -88 dBm | -87 dBm Note: provided by ①-②-③-④-10*log10(160/40) dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  |  | RF IC techniques and other tech. (before LNA) |  | Not applicable | N/A |  |  |  |  | Not applicable |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | No impact | N/A |  | 0 dBc | 0 dBc | 0 dBc | Not applicable |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -67 dBm Note: provided by ①-③-④-⑤dBm | -60 ~ -70 |  | -50 dBm | -55 dBm | -60 dBm | -53 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 24 dBc | N/A | 15 dBc |  |  |  | Not applicable |
|  |  | Frequency isolation techniques |  | Filtering | - | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |  |  |  | Not Applicable Filtering would not prevent in-band blocking in any case |
|  |  | RX IMD | Rx IIP3 capability (dBm) | IM3 contribution is Neglectable | negligible | Similar conclusion as FR1 (i.e., IIP3 and IM3 are not dominant). | -35 dBm | -35 dBm | -35 dBm | -35 dBm |
|  |  |  | Rx IM3 contribution (dBm) |  | negligible |  | -80 dBm | -95 dBm | -110 dBm | Negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | N/A | negligible | Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model. | Reciprocal phase noise mixing will add noise at around -95dBm. | Reciprocal phase noise mixing will add noise at around -100dBm | Reciprocal phase noise mixing will add noise at around -105dBm |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -91 dBm Note: provided by ①-③-④-⑤-⑥dBm | negligible |  | -80 dBm | -94 dBm | -104 dBm | -77 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 10 dBc | 10 | 5-10 dBc | TBC dBc |  |  | TBC |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Limited, ~0dB | Less than 0.5 dB loss |  |  |  |  | - |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dBc | - | 10 dB | 10 dBc | 10 dBc | 10 dBc | - |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 129.5 dBc | 129 ~ 139 | 128 dB | 119 dBc | 125,5dBc | 127 dBc | 114 dBc |
| Noise floor ⑩dBm |  |  |  | -83 dBm/100MHz | -88 dBm/40 MHz | -88 dBm/40 MHz | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/CBW | -87 dBm/40 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -89 dBm | -94 dBm | -94 dBm | -93 dBm | -93 dBm | -93 dBm | -93 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc | 129 | 124 dBc | 133 dBc | 128 dBc | 123 dBc | 130 dBc |
| SBFD configuration |  |  |  | DU (100MHz-100MHz) | DUD [80,40,80] | DUD | 75-50-75 |  |  | DUD [80,40,80] |
| Guardband assumption (if exist) |  |  |  | 5PRB | Existing SU | 5 PRBs | 3 RB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | Several GHz | 200MHz | Several GHz |  |  |  |

< Unchanged sections are omitted >

##### 9.5.1.2.5	Nokia

Nokia’s views on the self-interference analysis for FR2-1 base stations are presented in Table 9.5.1.1-1.

#### 10.5.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

< End of TP >

References

- R4-2314005, “Draft TR 38.858”, CMCC, August, 2023