3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316303

Xiamen, China, October 9-13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on BS RF requirements for SBFD

Agenda item:	5.19.2.3

Document for:	Approval

Introduction

RAN has agreed a new Rel-18 Study Item on Evolution of NR Duplex Operation with the following objectives [1]:

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The agreements on the BS RF requirements of SBFD in RAN4 #108, and list of topics that require further studies are captured in the way forward in [2]. In this document, we discuss some of the items listed as FFS in RAN4 #108:

Issue 3-2-2: Output power dynamics - Agreement: - RE power control dynamic range: Same requirements can be applied. - Total dynamic range: Requirements applicable for SBFD slots - FFS for the requirements limit and conformance testing Issue 3-2-6: Transmitter intermodulation - Agreement: - FFS whether Tx IMD requirements applicable for SBFD slots/symbols Issue 3-3-2: Dynamic range - Agreement: Dynamic range requirements applicable for SBFD symbols/slots - FFS for IoT level, and wanted signal power level Issue 3-4-2: In-channel adjacent subband leakage ratio, In-channel adjacent subband Blocking and adjacent subband selectivity - Agreement: - FFS whether new requirements need to be specified for co-site inter-sector gNB and inter-site gNB with candidate list as following - in-channel adjacent subband leakage ratio, - in-channel adjacent subband blocking and - in-channel adjacent subband selectivity.

Discussion

Transmitter intermodulation

In the last meeting, there were diverged views if existing Tx IMD requirements are still applicable in DL-only symbols/ slots. As there was no agreement reached, it was left for further studies whether new requirements for the SBFD symbols/ slots are needed or not.

The transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the antenna, radio distribution network and antenna array. Here, we note that the existing TDD requirements are defined considering a coupling loss of 30 dB between the co-located gNBs, which are more stringent than the coupling loss assumed for SBFD capable gNBs.

The SBFD Tx IMD performance might be able to be guaranteed by the legacy Tx intermodulation requirements.

Dynamic range

The dynamic range is specified as a measure of the capability of the receiver to receive a wanted signal in the presence of an interfering signal. As already agreed in the last meeting, dynamic range requirements are applicable for SBFD symbols/slots. This should be also tested during SBFD symbols/slots due to the additional interference.

Dynamic range requirements are applicable for SBFD symbols/slots. Existing requirements can be re-used.

Dynamic range should be tested during SBFD symbols/slots. Existing requirements can be re-used.

In-channel adjacent subband leakage ratio, In-channel adjacent subband blocking and adjacent subband selectivity

SBFD introduces new types of interference, some of which are highlighted in the figure below. From the BS perspective, the new cases of interference come from: 1) self-interference, 2) inter-sector interference, and 3) gNB-gNB interference.

Figure 1: Types of interference in SBFD deployment

In the last RAN4 meeting, it has been agreed to have new OTA sensitivity requirements in SBFD time slot with self-interference only. Also, in the last RAN4 meeting, it has been discussed whether new requirements should be defined for the adjacent subbands, such as:

- In-channel adjacent subband blocking and selectivity

- In channel adjacent subband leakage

Below, we discuss the need to introduce the different requirements to ensure the SBFD operation.

In our view, new requirements are needed to ensure interoperability between base stations of different vendors in the same network. They cannot be implicitly guaranteed by OTA sensitivity requirement. It is easier to cancel the effects of interference inside your base station, whereas from other base stations there might be no cancellation options. Inter-site and inter-sector BS interference should be taken into consideration in the definition of the requirements for SBFD operation.

The OTA sensitivity requirement does not capture the effects from inter-sector and inter-gNB interference.

In RAN4#107, some companies argued that, since RAN4 has not agreed on a reference implementation for the gNB, it is difficult to define additional requirements. Though we agree that depending on the gNB implementation the adjacent channel leakage, for example, can be different, in our view, RAN4 is responsible to define minimum requirements which should be achievable for reasonable implementations to guarantee the minimum performance.

In channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to ensure proper operation considering self-interference, inter-site and inter-gNB interference.

In-channel adjacent sub-band leakage ratio requirements

As shown in Figure 1, the leakage from the transmission in the DL subband can cause self-interference, inter-sector interference and gNB interference. The effects of the adjacent sub-band leakage into the gNBs own receiver can be cancelled to some extent, and we agree that this requirement can be implicitly guaranteed by the OTA sensitivity when considering self-interference only. However, the OTA sensitivity requirement does not guarantee that the sub-band leakage to other nodes in the network. Therefore, we believe that a new requirement is needed. The exact value of the requirement can be discussed in the work item phase.

RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering inter-sector interference and inter-site interference.

In-channel adjacent sub-band selectivity and blocking

Currently, the adjacent channel selectivity (ACS) is defined as a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

In SBFD operation, there is also the impact of adjacent sub-band interference, either caused by self-interference or inter-sector or inter-gNB interference.

Again, it can be argued that the in-channel adjacent sub-band selectivity can be captured in the OTA sensitivity requirements. However, as in the in-channel adjacent sub-band leakage ratio case, this test does not capture the selectivity in the presence of inter-sector and inter-gNB interference. Furthermore, there might be base stations of different vendors in the same network. That is why these requirements are needed to ensure interoperability between these base stations.  Therefore, we propose that sub-band selectivity requirement is also defined by RAN4.

RAN4 to define in-channel adjacent sub-band selectivity requirements considering inter-sector interference and inter-site interference.

Conclusion

This contribution presents our further views on the SBFD BS RF requirements aspects. The following observations and proposals were made:

- Observation 1: The SBFD Tx IMD performance might be able to be guaranteed by the legacy Tx intermodulation requirements.

Observation 2: Dynamic range requirements are applicable for SBFD symbols/slots. Existing requirements can be re-used.

Proposal 1:  Dynamic range should be tested during SBFD symbols/slots. Existing requirements can be re-used.

Observation 3: The OTA sensitivity requirement does not capture the effects from inter-sector and inter-gNB interference

Observation 4: In channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

Observation 5: Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to ensure proper operation considering self-interference, inter-site and inter-gNB interference.

Proposal 2: RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots.

Proposal 3: RAN4 to define in-channel adjacent sub-band selectivity requirements.

- References

- RP-223041, “Study on Evolution of NR Duplex Operation”, CMCC

- R4-2313867, “WF for feasibility and BS requirements impact on SBFD operation”, Samsung, August 21-25, 2023.

- R4-2314242, “Topic summary for [108][306] FS_NR_duplex_evo_Part1”, Samsung, August 21-25, 2023