3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316309
Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.16.3

Source:	CAICT

Title:	TP to TR 38.761 on channel model validation for n41 and n78

Document for:	Approval

1	Introduction

At RAN#101 plenary, the revised WID RP-232679 is approved and an internal TR38.761 is allocated to capture measurement of MIMO OTA performance for UEs. Several companies/labs successfully performed FR1 MIMO OTA channel model validation for bands n41 and n78 during Rel-17. In this contribution, we provide a TP to record the channel model validation results from all the labs for n41 and n78.

2	References

- RP-232679, Revised WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs, CAICT, 3GPP RAN#101

- R4-2207303, “Views on PDP reference and pass/fail limits for FR1 MIMO OTA channel model validation,” CAICT, 3GPP RAN4#102-e

- R4-2201591, “FR1 MIMO OTA channel model validation results and views on PDP pass/fail limits,” CAICT, 3GPP RAN4#101-bis-e

- R4-2119558, “FR1 MIMO OTA channel model validation results,” CAICT, 3GPP RAN4#101-e

- R4-2204570, “PDP validation results for CDL-C UMa and reference values for UMi,” CMCC, BUPT, 3GPP RAN4#102-e

- R4-2200832, “FR1 channel model validation results for CMCC & BUPT joint lab,” CMCC, BUPT, 3GPP RAN4#101-bis-e

- R4-2200573, “Further results on FR1 MIMO OTA Channel model validation,” Huawei, HiSilicon, 3GPP RAN4#101-bis-e

- R4-2119541, “Initial results on NR UE FR1 MIMO OTA channel model validation,” Huawei, HiSilicon, 3GPP RAN4#101-bis-e

- R4-2200576, “FR1 MIMO OTA channel validation,” MediaTek, 3GPP RAN4#101-bis-e

- R4-2201494, “Validation results and limits for FR1 CDL-C UMa channel model,” Xiaomi, 3GPP RAN4#101-bis-e

- R4-2207689, “FR1 MIMO OTA Lab Alignment, Channel Model Validation update,” Apple Hungary Kft., MVG, 3GPP RAN4#103-e

3	Text Proposal to TR 38.761

--------------Start of text proposal 1-------------

# 6	Channel Model Validation

## 6.1	General

This clause describes the FR1 and FR2 MIMO OTA channel model validation measurements. The purpose of channel model validation is to ensure that the channel models are correctly implemented and hence capable of generating the propagation environment, as described by the model, within the test zone.

### 6.2	Verification of Channel Model implementation of FR1

### 6.2.1	Framework

This clause describes how to proceed Channel Model Validation for FR1 MIMO OTA with MPAC system.

- The channel model validation measurements shall be performed according to the procedures in Annex C.3 of TS 38.151, including:

  - Power Delay Profile (PDP)

  - Doppler/Temporal correlation

  - Spatial correlation

  - Cross-polarization

  - Power validation

- Channel model: FR1 UMa CDL-C and UMi CDL-C, as specified in Annex C.1 of TS 38.151

- Test frequency: as specified in Tables C.3.1-1 and C.3.1-2 of TS 38.151

- Pass/fail limits: as defined in Annex C.4 of TS 38.151

### 6.2.2 Channel Model Validation Results

This clause presents channel model validation results from different labs with different types of equipment and setup vendors. All the validation results are included for comparison. Table 6.2.2-1 lists equipment and setup vendors of different labs.

Table 6.2.2-1: Equipment and setup vendors of different labs

| Labs | Channel Emulator | BS Emulator | System |
| --- | --- | --- | --- |
| Lab 1 | Vender 1 | Vender 1 | Vender 1 |
| Lab 2 | TBA | TBA | TBA |
| Lab 3 | TBA | TBA | TBA |
| Lab 4 | TBA | TBA | TBA |
| Lab 5 | TBA | TBA | TBA |
| Lab 6 | TBA | TBA | TBA |
| Lab 7 | TBA | TBA | TBA |

#### 6.2.2.1 Power Delay Profile (PDP)

The PDP should be validated in a Beam-Specific manner. It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

#### The PDP measurement results of UMa CDL-C for bands n41 and n78 are presented in Figures 6.2.2.1-1~5.

(a) Band n41

(b) Band n78

Figure 6.2.2.1-1: Lab 1: PDP measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2. CE bandwidth: 100MHz

(a) Band n41

(b) Band n78

Figure 6.2.2.1-2: Lab 2: PDP measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2

(a) Band n41

(b) Band n78

Figure 6.2.2.1-3: Lab 3: PDP measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2. CE bandwidth: 40MHz

(a) Band n41

(b) Band n78

Figure 6.2.2.1-4: Lab 4: PDP measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2.

(a) Band n41

(b) Band n78

Figure 6.2.2.1-5: Lab 6: PDP measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2.

#### 6.2.2.2 Doppler/Temporal correlation

The Doppler should be validated in a Beam-Specific manner. It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

The Doppler measurement results of UMa CDL-C for bands n41 and n78 are presented in Figures 6.2.2.2-1~6.

(a-1) Beam 1, H11                                                 (a-2) Beam 2, H11

(a-3) Beam 1, H12                                                    (a-4) Beam 2, H12

(a-5) Beam 1, H12                                                          (a-6) Beam 2, H12

(a-7) Beam 1, X2V                                                       (a-8) Beam 2, X2V

(a) Band n41

(b-1) Beam 1, H11                                                  (b-2) Beam 2, H11

(b-3) Beam 1, X2V                                                    (b-4) Beam 2, X2V

(b) Band n78

Figure 6.2.2.2-1: Lab 1: Doppler measurement results for CDL-C UMa, (a) Band n41 with different beams and different polarizations; (b) Band n78 with different beams and different polarizations

(a) Band n41

(b) Band n78

Figure 6.2.2.2-2: Lab 2: Doppler measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2

(a) Band n41

(b) Band n78

(c) Band n79

Figure 6.2.2.2-3: Lab 3: Doppler measurement results for CDL-C UMa, (a) Band n41, beam 1, beam 2, and combined beams; (b) Band n78, beam 1 and beam 2; (c) Band n79, beam 1 and beam 2

(a) Band n41

(b) Band n78

Figure 6.2.2.2-4: Lab 4: Doppler measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2

(a) Band n41

(b) Band n78

Figure 6.2.2.2-5: Lab 5: Doppler measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2

(a) Band n41

(b) Band n78

Figure 6.2.2.2-6: Lab 6: Doppler measurement results for CDL-C UMa, (a) Band n41, beam 1 and beam 2; (b) Band n78, beam 1 and beam 2

#### 6.2.2.3 Spatial correlation

The Spatial correlation should be validated in a Beam-Combined manner. It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

-	Combined beam for CDL-C UMa: Input 1 + Input 2 + Input 3 + Input 4

-	Combined beam for CDL-C UMi: Input 1 + Input 2

#### The Spatial correlation measurement results of UMa CDL-C for bands n41 and n78 are presented in Figures 6.2.2.3-1~6.

(a) Band n41

(b) Band n78

Figure 6.2.2.3-1: Lab 1: Spatial correlation measurement results for CDL-C UMa, (a) Band n41, beam 1, beam 2, and combined beams; (b) Band n78, combined beams

(a) Band n41                                                                (b) Band n78

Figure 6.2.2.3-2: Lab 2: Spatial correlation measurement results for CDL-C UMa, (a) Band n41 with combined beams; (b) Band n78 with combined beams

(a) Band n41

(b) Band n78

Figure 6.2.2.3-3: Lab 3: Spatial correlation measurement results for CDL-C UMa, (a) Band n41, beam 1, beam 2, and combined beams; (b) Band n78, combined beams

(a) Band n41

(b) Band n78

Figure 6.2.2.3-4: Lab 4: Spatial correlation measurement results for CDL-C UMa, (a) Band n41, combined beams; (b) Band n78, combined beams

(a) Band n41

(b) Band n78

Figure 6.2.2.3-5: Lab 5: Spatial correlation measurement results for CDL-C UMa, (a) Band n41, combined beams; (b) Band n78, combined beams

(a) Band n41

(b) Band n78

Figure 6.2.2.3-6: Lab 6: Spatial correlation measurement results for CDL-C UMa, (a) Band n41, combined beams; (b) Band n78, combined beams

#### 6.2.2.4 Cross-polarization

The Cross-polarization should be validated in a Beam-Specific manner. It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

The Cross-polarization measurement results of UMa CDL-C for bands n41 and n78 are presented in Table 6.2.2.4-1~6.

Table 6.2.2.4-1: Lab 1: Cross-polarization verification results for CDL-C UMa, bands n41 and n78

| Frequency | Beam | Reference | Measurement result |
| --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) | Beam 1 | Input 1+2: V/H = 0 dB | 0.44 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB | 0.55 dB |
| fc > 2.5 GHz (n78) | Beam 1 | Input 1+2: V/H = 0 dB | 0.49 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB | 0.58 dB |

Table 6.2.2.4-2: Lab 2: Cross-polarization verification results for CDL-C UMa, bands n41 and n78

| Frequency | Beam | Reference | Measurement result |
| --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) | Beam 1 | Input 1: V/H = -0.5 dB Input 2: V/H = 0.6 dB | Input 1: V/H = -0.96 dB Input 2: V/H = 0.94 dB |
|  | Beam 2 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB | Input 1: V/H = -0.11 dB Input 2: V/H = 0.71 dB |
| fc > 2.5 GHz (n78) | Beam 1 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB | Input 1: V/H = -0.51 dB Input 2: V/H = 1.19 dB |
|  | Beam 2 | Input 1: V/H = -0.7 dB Input 2: V/H = 0.8 dB | Input 1: V/H = -0.41 dB Input 2: V/H = 0.47 dB |

Table 6.2.2.4-3: Lab 3: Cross-polarization verification results for CDL-C UMa, bands n41 and n78

| Frequency | Beam | Reference | Measurement result |
| --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) | Beam 1 | Input 1: V/H = -0.5 dB Input 2: V/H = 0.6 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -0.98 dB Input 2: V/H = 0.63 dB Input 1+ 2: V/H = -0.21 dB |
|  | Beam 2 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -1.1dB Input 2: V/H = -0.07 dB Input 1+ 2: V/H = -0.60 dB |
| fc > 2.5 GHz (n78) | Beam 1 | Input 1: V/H = -0.6 dB Input 2: V/H = 0.7 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = -0.45 Input 2: V/H = 1.32 dB Input 1+ 2: V/H = 0.39 dB |
|  | Beam 2 | Input 1: V/H = -0.7 dB Input 2: V/H = 0.8 dB Input 1+ 2: V/H = 0 dB | Input 1: V/H = 0.25 Input 2: V/H = 1.61 Input 1+ 2: V/H = 0.91 dB |

Table 6.2.2.4-4: Lab 4: Cross-polarization verification results for CDL-C UMa, bands n41 and n78

| Frequency | Beam |  | Reference |  |  | Measurement result |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) |  |  | Beam 1 |  |  | Input 1: V/H = -0.5 dB |  |  | -0.48 dB |  |  |
|  |  |  |  |  |  | Input 2: V/H = 0.6 dB |  |  | 0.57 dB |  |  |
|  |  |  | Beam 2 |  |  | Input 1: V/H = -0.6 dB |  |  | -0.69 dB |  |  |
|  |  |  |  |  |  | Input 2: V/H = 0.7 dB |  |  | 0.71 dB |  |  |
| fc > 2.5 GHz (n78) |  |  | Beam 1 |  | Input 1: V/H = -0.6 dB |  |  | -0.48 |  |  |
|  |  |  |  |  | Input 2: V/H = 0.7 dB |  |  | 0.99 |  |  |
|  |  |  | Beam 2 |  | Input 1: V/H = -0.7 dB |  |  | -0.34 |  |  |
|  |  |  |  |  | Input 2: V/H = 0.8 dB |  |  | 1.13 |  |  |

Table 6.2.2.4-5: Lab 5: Cross-polarization verification results for CDL-C UMa, band n41

| Frequency | Beam | Reference | Measurement result |
| --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) |  | Beam 1 |  | Input 1: V/H = -0.5 dB |  | -0.48 dB |  |
|  |  |  |  | Input 2: V/H = 0.6 dB |  | 0.61 dB |  |
|  |  | Beam 2 |  | Input 1: V/H = -0.6 dB |  | -0.64 dB |  |
|  |  |  |  | Input 2: V/H = 0.7 dB |  | 0.68 dB |  |

Table 6.2.2.4-6: Lab 6: Cross-polarization verification results for CDL-C UMa, bands n41 and n78

| Frequency | Beam | Reference | Measurement result |
| --- | --- | --- | --- |
| fc ≤ 2.5 GHz (n41) | Beam 1 | Input 1+2: V/H = 0 dB | -0.6555 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB | -0.0676 dB |
| fc > 2.5 GHz (n78) | Beam 1 | Input 1+2: V/H = 0 dB | 0.5064 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB | 0.6414 dB |

#### 6.2.2.5 Power validation

The Power validation results of UMa CDL-C for bands n41 and n78 are presented in Table 6.2.2.5-1~3.

Table 6.2.2.5-1: Lab 1: Power validation results for CDL-C UMa, bands n41 and n78 (Unit: dBm/30kHz)

| CDL-C UMa, n41, 2592.99 MHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Measured V power | Measured H power | Measured total power | Expected | Delta |
| -79.6244 |  | -79.5959 |  | -76.5999 |  | -77 |  | 0.4001 |  |
| CDL-C UMa, n78, 3549.99 MHz |  |  |  |  |
| Measured V power | Measured H power | Measured total power | Expected | Delta |
| -79.9988 |  | -79.6618 |  | -76.8168 |  | -77 |  | 0.1832 |  |

Table 6.2.2.5-2: Lab 4: Power validation results for CDL-C UMa, bands n41 and n78 (Unit: dBm/30kHz)

| UMa CDL-C 2450 MHz | Dipole |  | S=-50.6dBm |  |
| --- | --- | --- | --- | --- |
|  | Loop |  | S=-50.4dBm |  |
| UMa CDL-C 3600 MHz |  | Dipole |  | S=-45.6dBm |  |
|  |  | Loop |  | S=-46.68dBm |  |

Table 6.2.2.5-3 (a): Lab 6: Power validation results for CDL-C UMa, band n41 (Unit: dBm/30kHz)

Table 6.2.2.5-3 (b): Lab 6: Power validation results for CDL-C UMa, band n78 (Unit: dBm/30kHz)

--------------End of text proposal 1 -------------