# 3GPP TSG-RAN WG4 Meeting # 108bis	                              R4-2316325

# Xiamen, China, Oct. 9 – Oct. 13, 2023

# Title: 	Further discuss Lower MSD for inter-band CA/EN-DC/DC combinations

# Source: 	ZTE Corporation

# Agenda item:	5.3.1.4.2

# Document for:	Approval

# 1 Introduction

The lower MSD topic was extensive discussed in the previous RAN4 meetings. In this contribution, we continue to discussed some issues about the lower MSD issues based on the WF [1].

# 2	Discussion

The conditions to indicate lower MSD capability were agreed in the following:

#### Sub-topic 1-1: Conditions to indicate Lower MSD capability - Agreement - For the purpose of low-MSD capability, if the minimum requirement for a given REFSENS exception case falls into the interval of MSD ≤ Thi dB, the actual MSD should be at least one-level lower (i.e., actual MSD ≤ Thi-1 dB) in order for the UE to report the low-MSD capability. - If the actual MSD is larger than the maximum threshold ThM-1 (i.e. out of range), the UE cannot report low-MSD capability for this REFSENS exception case. Note: the above two bullets should be reflected in the specification.

An another issue is whether to define at least [x]dB MSD improved. In terms of the discussion in the past, part of companies didn’t want to define it. Consequently, only the at least one-level lower approach was adopted. However, it implies that the lower-MSD capability is allowed to be reported if very small MSD like 0.1dB is improved.

Observation 1. The current conditions imply the lower MSD capability is allowed to be reported if very small MSD like 0.1dB is improved.

As we discussed in [3], we think it is meaningless to report lower MSD capability with much smaller amount of MSD improvement, for example, <1 dB MSD improved would be meaningless which may cause burden for UE or NW scheduling. So we think RAN4 should discuss the at least [x] dB MSD improved value.

Proposal 1. To discuss the at least [x] dB MSD improved value.

We think x=3dB is reasonable.

For the MSD order, there was an agreement in RAN4#107 meeting, but an new MSD index approach is .

- Issue 1-2-3: MSD order - Proposals - Option 1: previous agreements on MSD order - No need to report order for harmonic/ harmonic mixing/cross band isolation - IMD order up to 5 in Rel-18 - Option 2: Instead of MSD order, an MSD index is used (Skyworks) - For Harmonic, harmonic mixing and cross band isolation, index 0 is the worst-case MSD test point, index 1 is the optional second test point for the 1UL 1CC UL configuration type - For IMDs, index 0 correspond to the lowest IMD order (largest MSD) and index 1 to the next higher IMD order: - Within IM2 to IMD5 for the 2UL 2CC UL configuration type related IMDs - Within IMD3 to IMD9 for the 1UL 2CC UL configuration type related IMDs (but maximum 2 order possible) - IMD3 for triple beat related MSD for 2UL 3CC UL configuration type. - WF FFS in next meeting.

However, in RAN4 #104 meeting, there was an agreement saying:

<Agreement>: One common capability report scheme should apply for all band combinations rather than only example BC.

In current release 18 spec, there were several band combinations who have IMD issue with IMD order larger than 5.In general, these IMD issue are usually for the band combination with 3CC configured in 2UL. Therefore, if IMD order is limited up to 5, then the lower MSD capability cannot be applied for all band combination, especially for those band combination who has only IMDx (x>5) issue cannot be allowed to report the lower MSD capability.

Observation 2. The lower MSD capability cannot be applied for all band combination if IMD order is limited up to 5.

For the MSD types, the following option 2 reflects the current status of the all the band combinations.

#### Issue 1-2-2: MSD type - Proposals - Option 1: As long as the order is within 5, the IMD could be considered in Rel-18 regardless of the IMD mechanism (which means intra-band UL CA and intra+ inter UL CA are also taken into account in Rel-18) in terms of lower MSD capability reporting. (Samsung) - Option 2: (Skyworks) - MSD types are communicated per UL configuration and victim band - 4 UL configuration types are needed as of today: - 1UL 1CC - 1UL 2CC - 2UL 2CC - 2UL 3CC - 8 MSD types per UL configuration is sufficient (currently it is 4 per MSD type) - UL harmonic, Harmonic mixing and cross band isolation MSD types for 1UL 1CC UL configuration type - IMD type for 1UL 2CC UL configuration type - IMD type for 2UL 2CC UL configuration type - Triple beat type for 2UL 3CC UL configuration type - Option 3: Six different low MSD types signaling for R18 + 3 in R19 (MTK) - Option 4: If RAN4 can keep the existing UL/DL configuration for the MSD test, then RAN4 don’t need to report the detail UL/DL configuration information (# of CC, # of bands of each UL/DL) and RAN4 can merge the MSD types based on the actual MSD values from UE (Meta)

Also, Option 1 above seems reasonable. However, there exits the same IMD order for different UL configurations. For example for CA_n5-n77 as below, IMD4 issues are for both 2UL CC and 3UL CC configurations in 2 UL bands.

Although the MSD value are close (indeed there are other combinations have the similar situation), it is unclear which one can meet the lower MSD capability report condition above, and NW may not know which MSD value is improved for the same IMD order for different UL configuration when UE reports the lower MSD capability report. Therefore, we think IMD MSD type for different UL configuration type might be needed to when UE report the lower MSD capability.

Proposal 2. IMD MSD type for different UL configuration type might be needed when UE report the lower MSD capability.

# 3 Conclusion

In this contribution, we give some discussion on the lower MSD for inter-band NR CA/ENDC based on the latest agreements. The conclusions are:

Observation 1. The current conditions imply the lower MSD capability is allowed to be reported if very small MSD like 0.1dB is improved.

Observation 2. The lower MSD capability cannot be applied for all band combination if IMD order is limited up to 5.

Proposal 1. To discuss the at least [x] dB MSD improved value.

Proposal 2. IMD MSD type for different UL configuration type might be needed when UE report the lower MSD capability.

# 4	Reference

[1] R4-2314932, WF on lower MSD, Huawei, HiSilicon

[2] R4-2310499, WF on the lower MSD, Huawei, HiSilicon

[3] R4-2308173, On lower MSD for inter-band CA/ENDC, ZTE