3GPP TSG-RAN WG4 Meeting #108bis	R4-2316349

Xiamen, China, 9 – 13 October 2023

Agenda Item:	5.31.2

Source:	Ericsson

Title:	Discussions on enhanced eDRX for release 18 RedCap

Document for:	Discussions

# 1	Introduction

RAN4 discussed the remaining open issues of defining enhanced eDRX requirements for release 18 RedCap UEs, and good progress was reached which were summarized in a way forward document in [1]. There is, however, one remaining issue related to transition requirements which need to be discussed and resolved. In this contribution we discuss and provide our view on this topic.

# 2	Discussions

Transition requirements

The gNB can configure the UE with both Rel-17 INACTIVE eDRX and Rel-18 INACTIVE eDRX which allows the UE to switch to Rel-17 eDRX when the Rel-18 INACTIVE eDRX is not supported in a cell. Transition requirements for transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec) was discussed at last meeting and following two options were identified [1]:

Transition requirements - Define requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX (20.48sec) for at least the following cases - Case 1: transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec) within same cell - Case 2: UE moves from a cell that supports and configures Rel-18 INACTIVE eDRX to a cell that supports only Rel-17 INACTIVE eDRX and vice versa - FFS whether to reuse legacy requirements or introduce new requirements

It is noted that the legacy eDRX requirements defined in clause 4.2B.2.2, 4.2B.2.3, 4.2B.2.4 and 4.2B.2.5 in [2] already contain general requirements covering the UE transitions containing any of eDRX IDLE cycle, eDRX IDLE length, PTW configuration, see below:

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

Observation 1: Legacy eDRX requirements contain general transition requirements covering changes in eDRX cycle, eDRX cycele length and PTW configuration.

However, the release 18 RedCap UE can be configured with both eDRX_IDLE and eDRX_INACTIVE and the current requirements do not cover the transitions between those. Therefore we propose to revise the legacy transition requirements to cover the case of transition when any of of eDRX parameters of eDRX_IDLE and eDRX_INACTIVE are changed. The actual requitements can remain the same which is that the UE shall meet the less stringent requirement of the two requirements corresponding to the first and second state.

RAN4 to revise the legacy transition requirements to cover the transition when any of the eDRX parameters of eDRX_IDLE and eDRX_INACTIVE are changed.

# 3	Summary

In this contribution we have discussed the remaining issue of introducing enhanced eDRX requirements in RRC_INACTIVE mode for release 18 RedCap UE. Based on the discussions, we have made following observation and proposals:

Observation 1: Legacy eDRX requirements contain general transition requirements covering changes in eDRX cycle, eDRX cycele length and PTW configuration.

- RAN4 to revise the legacy transition requirements to cover the transition when any of the eDRX parameters of eDRX_IDLE and eDRX_INACTIVE are changed.

# References

R4-2314372, “WF on R18 NR RedCap RRM requirements”, Moderator (Ericsson)

TS 38.133 V18.2.0