3GPP TSG-RAN WG4 Meeting #108bis                                        R4-2316367

Xiamen, China, October 09 – 13, 2023

Source: 	Huawei, HiSilicon

Title: 	On UE RF requirement for FR2 multi-Rx chain DL reception

Agenda Item:	5.7.1

Document for:	Approval

Introduction

In previous RAN4 meeting, steady progress has been made for this WI as recorded in WF [1]. Still, final decision on core requirements is pending for another round of simulation result collection. Some of remaining issues are excerpted as below.

- Combining method to compute Pdirectional in metric Background: Baseline Metric from WF R4-2306604 (106-Bis): For UEs required to fulfil a requirement on the probability for 2AoA reception, the metric for a given AoA separation is the spatial average: Poverall= θ1φ1Pdirectional (θ1,φ1).AW(θ1) Pdirectional(1,1) is given by: Option 1 – arithmetic mean; Pdirectional(θ1,φ1)= {PFAoApair#1+PFAoApair#2+…PFAoApair#nn}. Option 2 – OR (*); Pdirectional(θ1,φ1)= OR{PFAoApair#1,PFAoApair#2,…PFAoApair#n}. Recommended WF: Companies are encouraged to provide analysis of pros & cons for each combining method. - AoA offsets for the UE RF requirement Recommended WF: The UE only needs to meet the requirement for 1 AoA offset. Options: - Define a requirement for each candidate AoA offset. - The requirement is defined for just 1 AoA offset. Companies are encouraged to provide the simulation results of at least 3 UE implementations (panels facing opposite directions, panels in adjacent sides, panels in same side). The results of both OR combing and arithmetic mean are expected to be provided. The results of each AoA offset should be based on the best UE orientation, the best UE orientation can be different for each AoA offset. The following table is used as template: 30; 60; 90; 120; 150; 180. Impementation#1. Impementation#2. Impementation#3. ……. - Alternative calibration for a UE model Besides the previously agreed calibration condition (UE meets REFSENS as well as EIS spherical coverage), an alternative calibration method for UE models is below: - The DL power level used in simulation will be adjusted to UE’s EIS CCDF@50%-xile. - This means the simulation results for PC3 provided by companies should not exceed 50% for any AoA offset. - The performance of UE model must meet the legacy REFSENS and EIS spherical coverage requirement

In this contribution, we would like to share our views regarding the remaining issues.

Discussion

On the core requirement

In previous meeting, it was agreed that only one AoA offset will be tested but the combination method should be further evaluated. Thus we would like to enjoy one more round of simulation considering the agreed alternative calibration method. Particularly, we consider all three types of antenna module location and traverse all three specified UE orientations in order to pick best performance which is aligned with the previous agreement.

Table 1. Pass ratio for PC3 UE based on ‘OR combining’

|  | AIP location AoA offset | left+right | back+left | back+right | back+back |
| --- | --- | --- | --- | --- | --- |
| UE facing positive direction of X-axis | 30 | 14.03% | 14.51% | 23.72% | 27.87% |
|  | 60 | 20.81% | 21.74% | 28.48% | 25.43% |
|  | 90 | 19.06% | 15.65% | 26.54% | 29.08% |
|  | 120 | 29.12% | 8.58% | 19.60% | 16.88% |
|  | 150 | 28.11% | 7.52% | 6.87% | 7.91% |
|  | 180 | 21.02% | 6.34% | 1.19% | 3.31% |
| UE facing positive direction of Y-axis | 30 | 13.40% | 14.10% | 23.43% | 31.18% |
|  | 60 | 19.57% | 20.64% | 27.45% | 25.46% |
|  | 90 | 17.80% | 14.62% | 26.47% | 29.89% |
|  | 120 | 30.64% | 8.82% | 19.10% | 16.73% |
|  | 150 | 29.67% | 7.87% | 6.51% | 11% |
|  | 180 | 22.94% | 6.93% | 1.66% | 4.58% |
| UE facing positive direction of Z-axis | 30 | 7.78% | 17.96% | 21.51% | 20.47% |
|  | 60 | 18.35% | 30.62% | 33.96% | 29.18% |
|  | 90 | 34.38% | 35.66% | 38.78% | 35.37% |
|  | 120 | 45.19% | 30.91% | 35.35% | 30.97% |
|  | 150 | 42.53% | 17.25% | 17.17% | 16.62% |
|  | 180 | 22.55% | 6.29% | 1.50% | 4.26% |

Table 2. Pass ratio for PC3 UE based on ‘arithmetic combining’

|  | AIP location AoA offset | left+right | back+left | back+right | back+back |
| --- | --- | --- | --- | --- | --- |
| UE facing positive direction of X-axis | 30 | 7.34% | 8.26% | 14.15% | 17.25% |
|  | 60 | 10.58% | 11.42% | 15.36% | 14.36% |
|  | 90 | 10.95% | 7.9% | 14.9% | 15.45% |
|  | 120 | 16.75% | 4.28% | 10.12% | 8.46% |
|  | 150 | 20.36% | 4.48% | 3.45% | 4.18% |
|  | 180 | 21.02% | 6.34% | 1.19% | 3.31% |
| UE facing positive direction of Y-axis | 30 | 6.91% | 8.03% | 13.98% | 19.97% |
|  | 60 | 9.89% | 10.76% | 15.03% | 14.31% |
|  | 90 | 10.15% | 7.46% | 14.78% | 15.8% |
|  | 120 | 17.37% | 4.41% | 9.81% | 8.38% |
|  | 150 | 21.73% | 4.72% | 3.27% | 5.77% |
|  | 180 | 22.94% | 6.92% | 1.65% | 4.58% |
| UE facing positive direction of Z-axis | 30 | 3.97% | 10.65% | 13.88% | 12.35% |
|  | 60 | 9.62% | 16.46% | 18.25% | 15.77% |
|  | 90 | 17.24% | 17.95% | 19.68% | 17.81% |
|  | 120 | 25.1% | 15.45% | 17.68% | 15.49% |
|  | 150 | 25.33% | 8.62% | 8.58% | 8.31% |
|  | 180 | 22.55% | 6.29% | 1.50% | 4.26% |

We try both “OR combining” and “arithmetic method” for the evaluation of pass ratio and attached full set of results. In addition to the value difference, it was agreed that the requirement and verification will be applied to both two AoAs rather than single AoA. To our understanding, this is aligned with the intention of this feature, which is to standardize the RF requirements and test method that can accommodate feasible implementations for 2 AoAs simultaneous reception. Follow this logic, the “OR combining” can properly reflect the UE capability under simultaneous reception mode while the “arithmetic method” would be over stringent currently but may be considered if more AoA and/or advanced test framework will be studied in the future. Thus we have the following proposal.

Proposal 1: Adopt the “OR combining” for the 2 AoA simultaneous reception spherical coverage requirement’s metric computation.

As for the existing options for core requirement definition, we think only specifying single AoA offset is the least attractive one because it cannot accommodate all different UE implementation choices that have been considered behind inputs from all companies. Before jumping into the particular value, we propose following consideration to move forward.

Proposal 2: Adopt one of the below definitions for the 2 AoA simultaneous reception spherical coverage requirement.

- Alt. 1: Define one pass ratio for AoA offset range [30, 60, 90] and another pass ratio for AoA offset range [120, 150].

- Alt. 2: Define different pass ratio for each AoA offset within [30, 60, 90, 120, 150], respectively.

Conclusion

In this contribution we discussed on the RF requirements for FR2 UE Multi-Rx, we have the following proposals:

Proposal 1: Adopt the “OR combining” for the 2 AoA simultaneous reception spherical coverage requirement’s metric computation.

Proposal 2: Adopt one of the below definitions for the 2 AoA simultaneous reception spherical coverage requirement.

- Alt. 1: Define one pass ratio for AoA offset range [30, 60, 90] and another pass ratio for AoA offset range [120, 150].

- Alt. 2: Define different pass ratio for each AoA offset within [30, 60, 90, 120, 150], respectively.

- References

- R4-2314668, “WF on FR2_multiRx_UERF”, vivo, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, August 21 – 25, 2023.