3GPP TSG-RAN4 Meeting # 108bis	R4-2316382

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	UE capability for channel raster enhancement

Agenda Item:	5.4.2

Document for:	Approval

- Introduction

At RAN#99, a new WI on channel raster enhancement was approved in RP-230813, with the following objectives:

- Specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible [RAN4].

- Changes to BS channel raster can be considered if required [RAN4].

- Specify the corresponding UE capability, if needed, to enable changes to the channel raster [RAN2, RAN4]:

  - RAN4 is to identify the release of the specifications 38.101-1 and 38.104 and the possibility of early implementation. If corresponding capability signalling is provided for early implementation and such early implementation is possible, the change is to be release independent from the identified release.

NOTE: Changes to channel raster need to be compliant with the definition of global channel raster in RAN4 specification.

In this contribution, we provide our consideration on the third objective, i.e. UE capability.

- Discussion

Per WF agreed in last meeting in [1], on UE capability,

Agreement:

A new UE capability will be specified to support the WI objectives

Open issues:

The new UE capability should be per band.

FFS from which release should the UE capability be applicable.

Whether the capability should (at least for some bands) be mandatory from Rel-18 onwards.

The issue only exists for the bands currently specified with 100 kHz channel raster. One of the questions is whether all these bands are required to support the new channel raster, or it will depend on the operator’s request. If the answer is the former in general, the UE capability is per UE. And if the answer is latter, the capability is per band. Introduction of new channel raster may imply new test is needed, hence our preference is per band

Proposal 1: the new capability is defined per band, and it will depend on the operator’s request.

For an earlier release, the additional UE capability is optional support for UE. Since the issue is only raised in Rel-17, we think it would be acceptable to introduce it from Rel-17

Proposal 2: the new UE capability is applicable from Rel-17.

- Conclusion

In this contribution, we provide our view on the UE capability for channel raster enhancement.

Proposal 1: the new capability is defined per band, and it will depend on the operator’s request.

Proposal 2: the new UE capability is applicable from Rel-17.

- Reference

[1] R4-2314681, WF on channel raster enhancements, Ericsson