3GPP TSG-WG4 Meeting #108bis 	R4-2316396

# Xiamen, China, Oct  09 - Oct 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.133 - Addition of new Test cases for DL interruptions at switching across 3/4 bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of new test cases for UL Tx switching across 3/4 bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test cases in Clause A.6.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test coverage will be available for UL Tx switching across 3/4 bands |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## A.6.5	Signalling characteristics

##### A.6.5.7C.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10C.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

### A.6.5.7D	DL interruptions at switching across 3/4 bands with two transmit antenna connectors

#### A.6.5.7D.1	DL interruptions at switching across 3/4 bands with two transmit antenna for single TAG in TDD-TDD CA

##### A.6.5.7D.1.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic across ¾ bands with two transmit antenna connectors defined in clause 8.2.2.2.10D for single TAG. The test cases are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, where in NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

There are three cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx. The test parameters for the three cells are given in Table A.6.5.7D.1.1-1, Table A.6.5.7D.1.1-2 and Table A.6.5.7D.1.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

For NR TDD SCell (Cell 2) and NR TDD SCell (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod1T1Tto2T] is indicated to UE.

Table A.6.5.7D.1.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 4: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode (Only valid for 4 bands case) |

Table A.6.5.7D.1.1-2: General test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell Cell 4: FR1 SCell (only valid for 4 bands case) | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 FR1 SCell on RF channel number 4 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD Cell 4: CSI-RS.2.5 TDD (only valid for 4 bands case) |  |
| T1 | s | Config 1 | 5 |  |

#### A.6.5.7D.2	DL interruptions at switching across ¾ bands with two transmit antenna in single TAG in FDD-TDD CA

##### A.6.5.7D.2.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching across ¾ bands with two transmit antenna connectors defined in clause 8.2.2.2.10D for single TAG. The test cases are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, where in NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

There are three cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx. The test parameters for the three cells are given in Table A.6.5.7D.2.1-1, Table A.6.5.7D.2.1-2 and Table A.6.5.7D.2.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T]  is 210us or

-	symbol #9 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #10 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

For NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod1T1Tto2T] is indicated to UE.

Table A.6.5.7D.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 4: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode (Only valid for 4 bands case) |

Table A.6.5.7D.2.1-2: General test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell Cell 4: FR1 SCell (only valid for 4 bands case) | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 FR1 SCell on RF channel number 4 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD Cell 4: CSI-RS.2.5 TDD (only valid for 4 bands case) |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7D.2.1-3: Cell specific test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: Cell 4 parameters are same as parameters for Cell 2 and Cell 3 in 4 bands case |  |  |  |  |  |

##### A.6.5.7D.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10D for single TAG case and provided in Table 8.2.2.2.10D-1.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7D.3	DL interruptions at switching across ¾ bands with two transmit antenna across 3/4 bands for two TAGs in TDD-TDD CA

##### A.6.5.7D.3.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching across ¾ bands with two transmit antenna connectors defined in clause 8.2.2.2.10D for two TAGs. The test cases are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, where in NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

There are three cells: FR1 TDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx, and where cell1 and cell2 belongs to TAG1 while cell3 belongs to TAG2. The test parameters for the three cells are given in Table A.6.5.7D.3.1-1, Table A.6.5.7D.3.1-2 and Table A.6.5.7D.3.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

For NR TDD SCell (Cell 2) and NR TDD SCell (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol on the 2nd special slot of every 8 slots:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod1T1Tto2T] is indicated to UE.

Table A.6.5.7D.3.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 4: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode (Only valid for 4 bands case) |

Table A.6.5.7D.3.1-2: General test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in TDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell Cell 4: FR1 SCell (only valid for 4 bands case) | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 FR1 SCell on RF channel number 4 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.2.5 TDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD Cell 4: CSI-RS.2.5 TDD (only valid for 4 bands case) |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7D.3.1-3: Cell specific test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: Cell 4 parameters are same as parameters for Cell 2 and Cell 3 in 4 bands case |  |  |  |  |  |

##### A.6.5.7C.2.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10D for two TAGs case and provided in Table 8.2.2.2.10D-2.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

#### A.6.5.7D.4	DL interruptions at switching across ¾ bands with two transmit antenna in single TAG in FDD-TDD CA

##### A.6.5.7D.4.1	Test Purpose and Environment

The purpose of this test is to verify DL interruption requirements during UE dynamic switching across ¾ bands with two transmit antenna connectors defined in clause 8.2.2.2.10D for two TAGs. The test cases are applicable for an NR inter-band CA configuration and SUL band configuration with inter-band CA when the capability [BandCombination-UplinkTxSwitch-r18] is present, where in NR inter-band CA configuration and SUL band configuration with inter-band CA, the number of NR uplink bands with different carrier frequencies is three or four. NR UL carrier(s) in each of the three or four uplink bands are capable of one or two transmit antenna connector(s), according to the UE capability.

There are three cells: FR1 FDD PCell (Cell 1), FR1 TDD SCell (Cell 2) and FR1 TDD SCell (Cell 3) where cell 1, cell2 and cell 3 are respectively in band A, B and C respectively with 2Tx, and where cell1 and cell2 belongs to TAG1 while cell3 belongs to TAG2. The test parameters for the three cells are given in Table A.6.5.7D.4.1-1, Table A.6.5.7D.4.1-2 and Table A.6.5.7D.4.1-3 below.

For NR TDD PCell (Cell 1), aperiodic CSI-RS for L1-RSRP reporting is triggered with power boosting [6dB] on the following symbol in the special slot:

-	symbol#12 if UE does not report uplinkTxSwitching-DL-Interruption;

-	otherwise,

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T]  is 210us or

-	symbol #9 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #10 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

For NR TDD carrier (Cell 2) and NR TDD carrier (Cell 3), aperiodic CSI-RS for L1-RSRP reporting is configured with power boosting [6dB] on the following symbol in the special slot:

-	symbol#10 if UE does not report [uplinkTxSwitching-DL-Interruption];

-	otherwise,

-	symbol #4 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 210us or

-	symbol #5 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 140us or

-	symbol #8 if UE capability [uplinkTxSwitchingPeriod1T1Tto2T] is 35us.

This test verifies that the UE correctly report the L1-RSRP reporting. The test case is only applicable to UE which supports simultaneousRxTxInterBandCA.

The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, [uplinkTxSwitchingPeriod1T1Tto2T] is indicated to UE.

Table A.6.5.7D.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | NR Cell 1: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR Cell 2: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 3: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR Cell 4: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode (Only valid for 4 bands case) |

Table A.6.5.7D.4.1-2: General test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1 | 1, 2, 3 | Three radio channels are used for this test. |
| Active cell |  | Config 1 | Cell 1: FR1 PCell Cell 2: FR1 SCell Cell 3: FR1 SCell Cell 4: FR1 SCell (only valid for 4 bands case) | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 FR1 SCell on RF channel number 3 FR1 SCell on RF channel number 4 |
| CP length |  | Config 1 | Normal |  |
| DRX |  | Config 1 | OFF |  |
| Measurement gap pattern Id |  | Config 1 | OFF |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| CSI-RS configuration for L1-RSRP reporting |  | Config 1 | Cell 1: CSI-RS.1.5 FDD Cell 2: CSI-RS.2.5 TDD Cell 3: CSI-RS.2.5 TDD Cell 4: CSI-RS.2.5 TDD (only valid for 4 bands case) |  |
| T1 | s | Config 1 | 5 |  |

Table A.6.5.7D.4.1-3: Cell specific test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in FDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | FDD | TDD | TDD |
| TDD configuration | Config 1 |  | N/A | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.1 except that: S=’11DL: 1GP:2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 |
| BWchannel | Config 1 |  | 10 MHz: NRB,c = 52 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.1 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -87 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: Cell 4 parameters are same as parameters for Cell 2 and Cell 3 in 4 bands case |  |  |  |  |  |

##### A.6.5.7D.4.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10D for two TAGs case and provided in Table 8.2.2.2.10D-2.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

Table A.6.5.7D.1.1-3: Cell specific test parameters for DL interruptions at switching across ¾ bands with two transmit antenna connectors in TDD-TDD CA

| Parameter |  | Unit | Cell1 | Cell2 | Cell3 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 | FR1 | FR1 |
| Duplex mode | Config 1 |  | TDD | TDD | TDD |
| TDD configuration | Config 1 |  | TDDConf.2.1 except that S=’1 1DL: :2UL’; nrofDownlinkSymbols: 11 nrofUplinkSymbols: 2 | TDDConf.2.2 | TDDConf.2.2 |
| BWchannel | Config 1 |  | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 | 40 MHz: NRB,c = 106 |
| Initial BWP Configuration | Config 1 |  | DLBWP.0.1 | DLBWP.0.1 | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| SRS configuration | Config 1 |  | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 | SRSConf.1 in Table A.4.4.1.1.1-3 is applied except that: resourceMappingstartPosition: 0 resourceMappingnrofSymbols: n2 |
| PDSCH Reference measurement channel | Config 1 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 |
| SMTC Configuration |  |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB Configuration | Config 1 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low | 2x2 Low | 2x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 | -104 | -104 |
| SS-RSRP Note 3 |  | dBm/SCS | -84 | -87 | -87 |
| Ês/Iot |  | dB | 17 | 14 | 14 |
| Ês/Noc |  | dB | 17 | 14 | 14 |
| NocNote 2 | Config 1 | dBm/SCS | -104 | -101 | -101 |
| IoNote3 | Config 1 | dBm/9.36 MHz | -58.96 | - | - |
|  |  | dBm/ 38.16MHz | - | -55.79 | -55.79 |
| Time offset to Cell1 Note 5 |  | s | - | 0 | 0 |
| Propagation Condition |  |  | AWGN | AWGN | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: Cell 4 parameters are same as parameters for Cell 2 and Cell 3 in 4 bands case |  |  |  |  |  |

##### A.6.5.7D.1.2	Test Requirements

The UE behaviour follows the requirements defined in clause 8.2.2.2.10D for single TAG case and provided in Table 8.2.2.2.10D-1.

UE shall send L1-RSRP report while meeting the accuracy requirements defined in clause 10.1.19.2.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of change 1>