3GPP TSG-RAN WG4 Meeting #108bis	R4-2316450

Xiamen, China, October 09 – October 13, 2023

Source:	ZTE Corporation

Title:	Discussion on scheduling/measurement restrictions for simultaneous DL reception from different directions

Agenda Item:	5.7.2.4

Document for:	Approval

- Introduction

In recent RAN plenary 100 meeting, a revised WID[1] for Rel-18 was approved to further clarify the scope of the  simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. The main update is to remove the NOTEs related with L3 measurement delay. The objective in the revised WID is as follows:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTEs: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

In last RAN4 meeting, RAN4 has spread extensive discussion around the scheduling/measurement restrictions and some agreements were achieved in [2].

- Scheduling restriction #### Issue 3-1-1: Update conditions/cases for scheduling restriction that can be relaxed for CSI-RS based L1 measurements for multi-Rx - Discussed with CR by taking agreements on Issue 2-2-1 into consideration. #### Issue 3-1-2: Cases whether scheduling restriction can be relaxed for L1 measurements for multi-Rx - Introduce scheduling restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS. - FFS whether scheduling restrictions are needed for UEs supporting simultaneous reception with mixed numerologies - No scheduling restriction relaxation (enhancement) for other cases. #### Issue 3-1-3: Conditions/cases that scheduling restriction for L3 measurements can be relaxed for multi-Rx - Scheduling restriction/measurement restriction relaxation (enhancement) for L3 measurements are not further discussed in Rel-18 multi-Rx WI. - Measurement restriction #### Issue 3-2-1: Cases whether measurement restriction for SSB based L1 measurements can be relaxed for multi-Rx <Online agreement> - Introduce measurement restriction relaxation (enhancement) for CSI-RS based RLM, BFD which includes cell specific BFD and TRP specific BFD, and non-group-based and group-based L1-RSRP measurements with repetition OFF. Detailed conditions are FFS. - FFS whether measurement restrictions are needed for UEs supporting simultaneous reception with mixed numerologies - No measurement restriction relaxation (enhancement) for other cases. #### Issue 3-2-2: Conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx - Reuse the same principle of scheduling restriction, if possible, and necessary revisions and additional conditions could be considered.

Still some details are FFS in terms of scheduling/restriction measurement. In this document, we provide our views on these remaining details.

- Discussion

- 2.1 Scheduling restriction

During last meeting, it is agreed that the scheduling restriction relaxation for CSI-RS based RLM, BFD, non GBBR and GBBR with repetition OFF are introduced. While the exact conditions are still FFS. In fact in 107 meeting, the exact conditions for CSI-RS based L1 measurement are summarized as below in [3]:

  - 	The CSI-RS is not in a CSI-RS resource set with repetition ON

  - 	The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time

  - 	CSI-RS has same QCL source as the active TCI state of any one of the PDCCH/PDSCHs

  - 	Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

  - 	UE is activated with multi-Rx operation

To our understanding, regarding the 3rd bullet, it would be more clear with the revision that:

  - 	CSI-RS has same QCL source as the active TCI state of the other one of the PDCCH/PDSCHs

With the two TCI states reported via a GBBR beam pair as the anchors, the CSI-RS and the simultaneous PDSCH/PDCCH should have same QCL source with the two TCI states respectively, so here change the ‘any’ into ‘the other’ would be more clear.

- Proposal 1: The exact conditions for the scheduling restriction for CSI-RS based RLM/BFD/GBBR/non-GBBR is:

- 	The CSI-RS is not in a CSI-RS resource set with repetition ON

- 	The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time

- 	CSI-RS has same QCL source as the active TCI state of the other one of the PDCCH/PDSCHs

- 	Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

- 	UE is activated with multi-Rx operation

Considering the case of mixed numerology between the L1 CSI-RS and the PDCCH/PDSCH, whether scheduling restriction relaxation is allowed for such mixed numerology case, still FFS.

- Here we only consider the CSI-RS based L1 measurement. Firstly the L1 CSI-RS can only be contained within the active BWP according the specification in 38.133. UE is not required to perform CSI-RS based RLM/BFD/CBD/L1-RSRP/L1-SINR measurement outside active BWP. In addition, until right now, the multi-panel reception is only allowed in single cell case. An any time, only one BWP can be activated in a cell. So the numerology of PDCCH/PDSCH and the L1 CSI-RS should be the numerology configured through BWP as below.

BWP ::=                             SEQUENCE {

locationAndBandwidth                INTEGER (0..37949),

subcarrierSpacing                   SubcarrierSpacing,

cyclicPrefix                        ENUMERATED { extended }                                                 OPTIONAL    -- Need R

}

- Since configured through a single IE, so the case of mixed numerology between L1 CSI-RS and PDCCH/PDSCH within a single cell is impossible during simultaneous reception. No no need to consider the mixed numerology for scheduling restriction relaxation.

- Proposal 2: The numerology of simultaneous L1 CSI-RS and PDCCH/PDSCH are always consistent since they are both determined by the ‘subcarrierSpacing’ configured in BWP configuration. No need to consider the mixed numerology case.

- 2.2 Measurement restriction

Regarding measurement restriction, the issue of mixed numerology is also FFS.

- Similar as analysis above, since the numerology of all L1 CSI-RS are determined by the ‘subcarrierSpacing’ configured in BWP configuration, it is impossible to appear the mixed numerology case between different L1 CSI-RS which are simultaneously received.

- Proposal 3: The numerology of simultaneous L1 CSI-RSs are always consistent since they are both determined by the ‘subcarrierSpacing’ configured in BWP configuration. No need to consider the mixed numerology case.

- Conclusion

In this contribution, we have the following proposals around scheduling/measurement restrictions for simultaneous DL reception from different directions:

- Proposal 1: The exact conditions for the scheduling restriction for CSI-RS based RLM/BFD/GBBR/non-GBBR is:

- 	The CSI-RS is not in a CSI-RS resource set with repetition ON

- 	The CSI-RS and one of the PDCCH/PDSCHs are transmitted through different TRPs at the same time

- 	CSI-RS has same QCL source as the active TCI state of the other one of the PDCCH/PDSCHs

- 	Resources of the active TCI states for the two PDCCHs or PDSCHs have been reported via GBBR in a pair

- 	UE is activated with multi-Rx operation

- Proposal 2: The numerology of simultaneous L1 CSI-RS and PDCCH/PDSCH are always consistent since they are both determined by the ‘subcarrierSpacing’ configured in BWP configuration. No need to consider the mixed numerology case.

- Proposal 3: The numerology of simultaneous L1 CSI-RSs are always consistent since they are both determined by the ‘subcarrierSpacing’ configured in BWP configuration. No need to consider the mixed numerology case.

- Reference

- RP-223527 “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

- R4-2314477 “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo.

- R4-2310046 “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)”, vivo.