3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316468

Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.21.2

Source:	MediaTek Inc.

Title:	Discussion on AIML RAN4 requirements for different use cases

Document for:	Discussion

- Introduction

In RAN4#108 meeting, some agreements related Metrics/KPIs for beam prediction requirements/tests were achieved as below [1]. In this contribution, we provide our view about Metrics/KPIs for CSI and BM use cases.

Issue 2-2: Metrics/KPIs for Beam prediction requirements/tests Agreement: - Metrics/KPIs for Beam prediction requirements/tests include - Option 1: RSRP accuracy - Option 2: beam prediction accuracy :Top-1(%), Top-K(%) - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: overhead/latency reduction - Option 5: combinations of above options - The overhead/latency reduction should be considered for the requirements as the side condition

- Discussion

2.1	Metrics/KPIs for CSI requirements/tests

In RAN4#107, RAN4 had some agreements for Metrics/KPIs for CSI requirements/tests  considering throughout or intermediate KPIs used in RAN1. It was considered to use throughput as the metric first. If there is some significant disadvantage observed, then RAN4 can consider using other metrics or intermediate KPIs.

Issue 2-2: Metrics for CSI requirements/tests Agreement: - For metrics for CSI requirements/tests for model inference performance testing - Consider the following possible test metrics - Throughput – absolute throughput or relative throughput - If throughput is not applicable or significant disadvantage is observed by using throughput, intermediate KPIs like cosine similarity, accuracy of predicted CQI, etc, - FFS on whether the KPIs are testable - Companies are encouraged to show how the KPI can be tested in RAN4 - If throughput is not applicable or significant disadvantage is observed by using throughput, other test metrics are not precluded - FFS on whether the KPIs are testable - Companies are encouraged to show how the KPI can be tested in RAN4

Observation 1: In RAN4#107, it was agreed to use throughput as the test metric to define CSI requirements.

Observation 2: In RAN4#107, it was agreed to consider other metrics or intermediate KPIs if throughput is not applicable or significant disadvantage is observed.

Observation 3: Other test metrics and intermediate KPIs are not precluded in the SI stage according to the agreements in RAN4#107.

Issue 2-1: Metrics/KPIs for CSI requirements/tests - Proposals - Option 1: Only use throughput (absolute or relative) - Option 2: Use throughput and other intermediate metrics/KPIs(SGCS, NMSE, etc) - Option 3: use throughput and overhead - Option 4: all of the above metrics

In RAN4#108, RAN4 still had lots of options on the table. We provide our observations for all options in the table below.

Table 1. Observations about test metric for CSI feedback in AI/ML

| Option 1: Only use throughput (absolute or relative) | Option 2: Use throughput and other intermediate metrics/KPIs (SGCS, NMSE, etc) | Option 3: use throughput and overhead |
| --- | --- | --- |
| - Can use ACK/NACK in legacy design to calculate throughput | - Intermediate KPI is only utilized for AI/ML model performance calibration in RAN1 - Need the ground truth which depends on physical layer design depends in RAN1 | - It may be difficult to introduce the requirement by considering both throughput and overhead - The overhead can be used as a side condition |

We think RAN4 can follow the agreements in RAN1#107 to use throughput as the test metric to define CSI requirements and keep other options in the SI stage. In the later WI stage, RAN1 may introduce some specification changes for AI/ML based CSI prediction/compression. At that time, RAN4 can have more discuss about the test metric according to agreements in RAN1.

Proposal 1: In the stage of SI, RAN4 can capture possible test metrics, including throughput, intermediate KPIs or other test metrics in the TR. However, we suggest following the agreements in RAN4#107 to use throughput as the test metric to define CSI requirements in the WI stage. If throughput is not applicable or significant disadvantage is observed, RAN4 can consider other metrics or intermediate KPIs. RAN4 can then determine the test metric after RAN1’s work in Rel-19 WI.

| Option 1: Only use throughput (absolute or relative) | Option 2: Use throughput and other intermediate metrics/KPIs (SGCS, NMSE, etc) | Option 3: use throughput and overhead |
| --- | --- | --- |
| - Can use ACK/NACK in legacy design to calculate throughput | - Intermediate KPI is only utilized for AI/ML model performance calibration in RAN1 - Need the ground truth which depends on physical layer design depends in RAN1 | - It may be difficult to introduce the requirement by considering both throughput and overhead - The overhead can be used as a side condition |

2.2	Metrics/KPIs for Beam prediction requirements/tests

RAN4#108 reached the following agreement on metrics for beam prediction requirements/tests.

Metrics/KPIs for Beam prediction requirements/tests include - Option 1: RSRP accuracy - Option 2: beam prediction accuracy :Top-1(%), Top-K(%) - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: overhead/latency reduction - Option 5: combinations of above options

RAN1#114 also discussed the metrics for model monitoring and reached the following observation.

Observation Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, the following table is identified Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy; Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER; Alt.3: Performance metric based on input/output data distribution of AI/ML; Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP. Applicable to all studied AI models; Applicable to all studied AI models; Applicable to all studied AI models; May not applicable to some implementation of AI model (e.g., not output of predicted L1-RSRP). Reflect the prediction accuracy of AI model; Reflect the system/link performance; Reflect the change of the statics of the input/output data; Reflect accuracy of the predicted 1-RSRP. Not reflect the system/link performance directly; Not reflect the prediction accuracy of AI model directly; Not reflect the prediction performance of AI model directly Not reflect the system/link performance directly; Not reflect the system/link performance directly. Note1: The above analysis shall not give an indication about whether/which metric is supported or specified Note2: Monitoring performance of the above alternatives are not touched in the table

We think we are not supposed to conclude the exact metric to use in SI stage. Similar as RAN1, we can make some analysis on the pros and cons of the options on table. In the next, we will provide our analysis on the three options.

Option 1: RSRP accuracy

RAN1 has defined some L1-RSRP related metrics, i.e.,

- L1-RSRP difference of Top-1 predicted beam: the difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

- the predicted L1-RSRP difference: The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

In the above RAN1 observation, the wording “The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP" is used. It is important to make it clear that which RSRP accuracy Option 1 refers to. In our understanding, Option 1 is the same as Alt.4 in RAN1’s discussion, i.e., the L1-RSRP difference between the measured RSRP and predicted RSRP. As the value of Rx beam forming gain is not available in the field or during the test, it is not possible to get the ideal L1-RSRP in FR2. The one that can be used as a reference during the test can only be the measured RSRP by UE in FR2. That is why in Alt.4 measured RSRP is used as a baseline for comparison. But it is possible to get the ideal L1-RSRP during the test in FR1. Therefore, when RAN4 defines the performance requirements for beam prediction AI/ML model, it is very likely that the requirements are different in FR1 and FR2. As far as we know, all the simulations conducted in RAN1 focus on FR2. In our view, it is important to clarify in RAN4 that only FR2 is considered or both FR1 and FR2 are considered.

Observation 4: It is possible to get the ideal L1-RSRP during the test in FR1.

Proposal 2:  Discuss whether to consider FR1 in AI/ML based beam prediction.

Proposal 3: RSRP accuracy refers to the accuracy of predicted RSRP compared to measured RSRP in FR2 and ideal RSRP in FR1.

As shown in RAN1 observation, Option 1 is not applicable to all the implementations of AI model, i.e., not output of predicted L1-RSRP. Since this is a SI, we are open to discuss different metrics for different model types unless RAN1 makes down-selection on the model types.

Proposal 4:  Different metrics can be used for different model types unless RAN1 makes down-selection on the model types.

Option 2: beam prediction accuracy: Top-1(%), Top-K(%)

Option 2 is applicable to all the implementations of AI model. For option 2, we think it is possible to define test cases, but we wonder how to define the accuracy requirements or the necessity to define the accuracy requirements.

Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

- Related measurement accuracy can be considered to determine x

As the motivation of DL Tx beam prediction is to find the best Tx beam, even the predicted L1-RSRP may be not very accurate, as long as the best Tx beam is found with the aid of AI/ML model. The beam choosing with the aid of AI/ML model would be one of the Top-K predicted beams. In our understanding, Option 3 can reflect this and .Option 3 is applicable to all the implementations of AI model. It is feasible to define both accuracy requirements and test cases.

Based on the above analysis, we propose the following Table for further discussion.

Proposal 5: Regarding the metric(s) for Beam prediction requirements/tests, we can keep all options and have further discussions in the WI based on the table below.

| Option 1: the accuracy of predicted RSRP compared to measured RSRP in FR2 and ideal RSRP in FR1 | Option 2: beam prediction accuracy: Top-1(%), Top-K(%) | Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB |
| --- | --- | --- |
| Not applicable to some implementation of AI model (e.g., not output of predicted L1-RSRP | Applicable to all studied AI models | Applicable to all studied AI models |
| Feasible to define both accuracy requirements and test cases | Feasible to define test cases. No need to define accuracy requirements | Feasible to define both accuracy requirements and test cases |

- Conclusion

In this contribution, we provide our views about test metrics to define requirements for CSI and BM use cases. Our observations and proposals are summarized as below.

Observation 1: In RAN4#107, it was agreed to use throughput as the test metric to define CSI requirements.

Observation 2: In RAN4#107, it was agreed to consider other metrics or intermediate KPIs if throughput is not applicable or significant disadvantage is observed.

Observation 3: Other test metrics and intermediate KPIs are not precluded in the SI stage according to the agreements in RAN4#107.

Proposal 1: In the stage of SI, RAN4 can capture possible test metrics, including throughput, intermediate KPIs or other test metrics in the TR. However, we suggest following the agreements in RAN4#107 to use throughput as the test metric to define CSI requirements in the WI stage. If throughput is not applicable or significant disadvantage is observed, RAN4 can consider other metrics or intermediate KPIs. RAN4 can then determine the test metric after RAN1’s work in Rel-19 WI.

| Option 1: Only use throughput (absolute or relative) | Option 2: Use throughput and other intermediate metrics/KPIs (SGCS, NMSE, etc) | Option 3: use throughput and overhead |
| --- | --- | --- |
| - Can use ACK/NACK in legacy design to calculate throughput | - Intermediate KPI is only utilized for AI/ML model performance calibration in RAN1 - Need the ground truth which depends on physical layer design depends in RAN1 | - It may be difficult to introduce the requirement by considering both throughput and overhead - The overhead can be used as a side condition |

Observation 4: It is possible to get the ideal L1-RSRP during the test in FR1.

Proposal 2:  Discuss whether to consider FR1 in AI/ML based beam prediction.

Proposal 3: RSRP accuracy refers to the accuracy of predicted RSRP compared to measured RSRP in FR2 and ideal RSRP in FR1.

Proposal 4:  Different metrics can be used for different model types unless RAN1 makes down-selection on the model types.

Proposal 5: Regarding the metric(s) for Beam prediction requirements/tests, we can keep all options and have further discussions in the WI based on the table below.

| Option 1: the accuracy of predicted RSRP compared to measured RSRP in FR2 and ideal RSRP in FR1 | Option 2: beam prediction accuracy: Top-1(%), Top-K(%) | Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB |
| --- | --- | --- |
| Not applicable to some implementation of AI model (e.g., not output of predicted L1-RSRP | Applicable to all studied AI models | Applicable to all studied AI models |
| Feasible to define both accuracy requirements and test cases | Feasible to define test cases. No need to define accuracy requirements | Feasible to define both accuracy requirements and test cases |

- Reference

- R4-2314740, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated