3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316469

Xiamen, China, October 09 – October 13, 2023

Agenda item:	5.21.3

Source:	MediaTek Inc.

Title:	Discussion on the interoperability and testability aspects of AIML RAN4 requirements

Document for:	Discussion

- Introduction

In RAN4#108 meeting, a table is provided to summarize issues to be discussed in describing the options for the testing of 2-sided model [1]. In this contribution, we provide our inputs to the table.

- Discussion

There are still 4 options left for test decoders for testing encoders in the 2-sided model.

- Option 1: Test decoder is provided by the vendor of the encoder under test so that the encoder and decoder are jointly designed and trained - Option 2: Test decoder is provided by the vendor of the decoder(infra-vendors) so that the encoder and decoder are jointly designed and trained - Option 3: The Test decoder(s) are fully specified and captured in RAN4 spec to ensure identical implementation across equipment vendors without additional training procedure needed. - Option 4: The Test decoder(s) are partially specified and captured in RAN4 spec.

In Table. 1, we provide our analysis/view on different options discussed in RAN4.

Table 1. Summary of test decoder design options for 2-sided models

|  | Option 1 | Option 2 | Option 3 | Option 4 |
| --- | --- | --- | --- | --- |
| Clarification of options |  |  |  |  |
| Source of the test decoder | vendor of the encoder | vendor of the decoder (infra-vendors) | RAN4 specification (could be TE vender) | Could be vender of encoder, vender of decoder or TE vender |
| Source of decoder training data | vendor of the encoder | vendor of the decoder (infra-vendors) | without additional training procedure needed | Could be vender of encoder, vender of decoder or TE vender |
| DUT vendor knowledge of the test decoder | Full knowledge of the test decoder | Only from agreed common assumptions | Full knowledge of the test decoder | Depends on source of the test decoder |
| Supported training collaboration type (source of training data should be consistent with the collaboration type) | collaboration Type 1 and 3 | collaboration Type 2 and Type 3 | No need | Depends on source of the test decoder |
| Test decoder verification procedure at TE and/or DUT | No need | Needed | No need | Depends on source of the test decoder |
| Feasibility of test decoder verification procedure | NA | Need offline alignment to guarantee the performance | NA | Depends on source of the test decoder |
| Pros/Cons analysis |  |  |  |  |
| Reflection on the real deployment (knowledge of model, training type, etc.) | May not reflect the real deployment when decoder is implemented in gNB side | Could reflect the performance in real deployment | RAN4 should study how to guarantee the test decoder can reflect the decoder used in real field | Depends on source of the test decoder |
| TE requirements to deploy the decoder (e.g. training, complexity, interoperability) | TE must support various test decoders from vendors of the encoders | TE must support various test decoders from vendors of the decoders | TE follows the decoder defined in RAN4 specification | Depends on source of the test decoder |
| Specification Effort (e.g. test decoder) | little effort May need some limit on test decoder to ensure it can be implemented on TE side | little effort May need some limit on test decoder to ensure it can be implemented on TE side | Consensus of a reference model in RAN4 may be a challenging task | Depends on source of the test decoder |
| Confidentiality/IP issues | Decoder structure will be shared to TE vendor | Decoder structure will be shared to TE vendor | No issue | Depends on source of the test decoder No issue if the test decoder is from TE vendor |
| Applicability to different scenarios/conditions/ configurations | Applicable | Applicable | NA | NA |
| Complexity of actual testing procedure for the ecosystem | High TE must support various test decoders from vendor of the encoder The test may involve model delivery | High TE must support various test decoders from vendor of the decoder The test may involve model delivery | Low No model delivery involved if TE implements the test decoder according to the specification | Depends on source of the test decoder |

Observation 1: It is not clear that how the test decoder is “partially specified and captured in RAN4 spec”.

Observation 2: Even if the test decoder is “partially specified” in the RAN4 spec. It is still unclear which entity should provide test decoder, vender of encoder, vendor of decoder or TE vendor.

Proposal 1: Remove Option 4 and only discuss option 1, 2 and 3 for 2-sided model.

- Conclusion

In this contribution, we provide our analysis/view on different options discussed in RAN4 based on the agreed table in [1]. The observations and proposals are summarized as below.

Observation 1: It is not clear that how the test decoder is “partially specified and captured in RAN4 spec”.

Observation 2: Even if the test decoder is “partially specified” in the RAN4 spec. It is still unclear which entity should provide test decoder, vender of encoder, vendor of decoder or TE vendor.

Proposal 1: Remove Option 4 and only discuss option 1, 2 and 3 for 2-sided model.

- Reference

- R4-2314740, “WF on RAN4 requirements for NR AI/ML”, Qualcomm Incorporated

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”