3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316476

Xiamen, China, Oct 9 – Oct 13, 2023

Source: 	Huawei, HiSilicon

Title:	Discussion on UTRA ACLR related PC2 A-MPR requirements

Agenda Item:	4.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

The lack of UTRA ACLR requirement for PC2 has become the blocking factor for completing the PC2 support for several FDD bands. In the sequel, we discuss the potential solution for this problem.

# 2	Discussion

The UTRA ACLR is defined in clause 6.5.2.4.2 of TS38.101-1 as shown below.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

Observation 1: UTRA ACLR requirements are only specified for PC3.

Several network-signaling values indicate UTRA protection as shown below, including NS_100, NS_03U, NS_05U and NS_43U.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.7 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 (NOTE 7) |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_100 | 6.5.2.4.2 | n1, n2, n3, n5, n8, n18, n25, n26, n65, n66, n80, n81, n84, n86, n89 (NOTE 1) |  |  | Table 6.2.3.1-2 |

Observation 2: NS_100 is dedicated for UTRA protection purpose, while NS_03U, NS_05U and NS_43U require UTRA protection in addition to other emission requirements.

If a PC2 capable UE receives one of the above UTRA protection related NS values, the UE behavior would be unspecified. Furthermore, the PC2 A-MPR requirements need to be defined for the completeness of the PC2 FDD basket WI.

Observation 3: It’s necessary to clarify the UTRA ACLR requirements for PC2 for the completeness of FDD PC2 requirements, especially the A-MPR requirements for the relevant bands.

Given that UTRA deployment is phasing out globally, it’s proposed not to develop new UTRA ACLR for PC2.

Proposal 1: In case no explicit request from operators, reuse the PC3 UTRA ACLR for PC2, i.e.,

Proposed UTRA ACLR requirement

|  | Power class 2, 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

If the same UTRA ACLR is applied, the existing PC3 A-MPR for NS_100 would be sufficient for PC2 as well.

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

Regarding NS_03U, NS_43U, the PC2 A-MPR can be defined as max{A-MPR for NS_100, PC2 A-MPR for NS_03U} and max{A-MPR for NS_100, PC2 A-MPR for NS_43U}, respectively.

Proposal 2: The PC2 A-MPR for NS_xxU (i.e. UTRA protection related NS) can be defined as max{A-MPR for NS_100, PC2 A-MPR for NS_xx}.

# 3	Conclusion

In this paper, we have presented the following observations and proposals related to UTRA ACLR.

Observation 1: UTRA ACLR requirements are only specified for PC3.

Observation 2: NS_100 is dedicated for UTRA protection purpose, while NS_03U, NS_05U and NS_43U require UTRA protection in addition to other emission requirements.

Observation 3: It’s necessary to clarify the UTRA ACLR requirements for PC2 for the completeness of FDD PC2 requirements, especially the A-MPR requirements for the relevant bands.

Proposal 1: In case no explicit request from operators, reuse the PC3 UTRA ACLR for PC2, i.e.,

Proposed UTRA ACLR requirement

|  | Power class 2, 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

Proposal 2: The PC2 A-MPR for NS_xxU (i.e. UTRA protection related NS) can be defined as max{A-MPR for NS_100, PC2 A-MPR for NS_xx}.

# References

- R4-2314646 WF on HPUE for FDD single bands, China Unicom, RAN4#108