3GPP TSG-RAN WG4 Meeting #108bis	R4- 2316483

# Xiamen, China, 9th – 13th October, 2023

Source:	Huawei, HiSilicon, BT plc

Title:	TP to TR 38.718-02-01 Addition of UL_n28A-n46C to CA/DC configurations

Agenda item:	4.10.2

WI code:	NR_CADC_R18_2BDL_xBUL-Core

Document for:	Approval

# Background

This contribution provides text proposal for adding UL_n28A-n46C to the UL configurations of the NR CA/DC band combinations as defined in the WID [1]. It is reliant on uplink CA_n46C being specified during RAN4#108bis.

# Text Proposal

##### ---Start of changes---

## 5.x	CA_n28-n46

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n28-n46

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n46 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n28-n46

| NR CA Configuration | UL CA Configuration | NR Band | Channel bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n46A | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n28A-n46C | CA_n28A-n46A CA_n28A-n46C | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n28A-n46D | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n28A-n46(2A) | CA_n28A-n46A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |

#### 5.x.1.3	UE Co-existence studies

The co-existence study has been completed in Rel-17.

#### 5.x.1.4	∆TIB and ∆RIB values

Reuse the existing requirements.

#### 5.x.1.5	REFSENs requirements

Reuse the existing requirements.

### 5.x.2		Specific for 2 bands UL CA

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28-n46 | 23 | +2/-3 |

#### 5.x.2.2		UE co-existence studies

#### 5.x.2.2.1	UE co-existence studies for Uplink Intra-Band Contiguous CA

When CA_n28A-n46C is configured as UL, the potential IMD interference is calculated as below.

Table 5.x.2.2.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 20 | 0 | 200 | 5150 | 5925 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 5150 | 5150 | 5350 | 5925 | 5925 | 5725 |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H | - | - |
| Interference ranges | 0 | 200 | 10300 | 11850 | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 4950 | 6125 | 15450 | 17775 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 0 | 400 | 10100 | 12050 | 20600 | 23700 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 4750 | 6325 | 15250 | 17975 | 25750 | 29625 |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 0 | 600 | 9900 | 12250 | 20400 | 23900 |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 4550 | 6525 | 15050 | 18175 | 25550 | 29825 |

It can be seen that there is no IMD interference (up to 7th order) falling into band n28 DL.

Furthermore, there is no triple-beat collision issue as per WF R4-2220556, since band n28 belongs to LB band group and band n46 belongs to UHB band group.

#### 5.x.2.3		REFSENS requirements

No update to the existing requirements is needed.

##### ---Next change---

## 7.x	DC_n28-n46

### 7.x.1	Operating bands for DC_n28-n46

Table 7.x.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n28-n46 | n28, n46 |

### 7.x.2   Configurations for DC_n28-n46

Table 7.x.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n28A-n46A DC_n28A-n46C DC_n28A-n46D | DC_n28A-n46A |
| DC_n28A-n46C | DC_n28A-n46C |
| DC_n28A-n46(2A) | DC_n28A-n46A |

### 7.x.3   Maximum output power for NR-DC

Table 7.x.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28-n46 | 23 | +2/-31 |
| NOTE 1: refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

##### ---End of changes---

# Reference

[1]	RP-222904, “Revised WID Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation