# 3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316509

# Xiamen, China, October 09 – October 13, 2023

Agenda Item:			5.2.4

Source:					Qualcomm Incorporated

Title:                        Discussion on demodulation test method for FR2 multi-Rx UE

Document for:		Approval

# Introduction

In RAN4#107, the test methods for FR2 multi-Rx UE were discussed and the WF was approved in [1]. In this meeting, we provide our views on the test method for UE Demodulation.

# Discussion

## Noc level configuration

In RAN4#107, the principle of Noc level configuration was agreed [1] and the X value which is the allowable degradation in sensitivity from legacy REFSENS is FFS.

Issue 3-1-3: Noc level configuration - Proposals - Option 1 (R4-2309244): RAN4 to agree the wanted noise, i.e., Noc level is set by the below equation for multi-Rx demodulation test on band Y: - Noc = REFSENSPC3, band Y, 50MHz -10log10(SCSREFSENS x PRBREFSENS x 12) - SNRREFSENS + ∆thermal + X, where - REFSENSPC3, band Y, 50MHz is the REFSENS value in dBm specified for Power Class 3 UE in band Y for 50MHz channel bandwidth in TS 38.101-2 Table 7.3.2.3-1, [dBm/Hz] - SCSREFSENS is a subcarrier spacing associated with NRB for 50MHz in TS 38.101-2 Table 5.3.2-1, chosen as 120kHz. - PRBREFSENS is NRB associated with subcarrier spacing 120kHz for 50MHz in TS 38.101-2 Table 5.3.2-1 and is 32. - 12 is the number of subcarriers in a PRB - SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS - ∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a rise in total noise of ∆BB. ∆thermal = 6dB, giving a rise in total noise of 1dB. - X is the allowable degradation in sensitivity from legacy REFSENS requirements in dB - Option 2: TBA - Agreement: - Option 1 agreed

In RAN4#108, the range of X value [2~5]dB was agreed in [2].

Issue 3-1-1: X value in Noc level configuration - Proposals - Option 1 (Qualcomm): As a baseline, RAN4 to use X = 2 for multi-Rx demodulation test directions selection - Option 2: (Huawei): Clarify the simulation assumptions before specifying X - Agreement: - Further align the simulation assumption and make decision in RAM4#108bis for X value within the range [2~5].

There are two factors to determine the value X, which indicates allowable degradation in sensitivity from legacy REFSENS requirements in dB

- Testable SNR

- Feasible number of AoA pairs satisfying criteria: spherical coverage requirement with X degradation, and minimum isolation.

The following Table 1 provides the testable SNR with different X values. Note that the testable SNR is calculated based on the parameters defined in TR 38810, i.e., IFF with 100MHz CBW on n260.

Table 1: Testable SNR with different X values

| X values (dB) | Maximum testable SNR (dB) |
| --- | --- |
| 0 | 17.7 |
| 1 | 16.7 |
| 2 | 15.7 |
| 4 | 13.6 |
| 6 | 11.5 |
| 8 | 9.4 |
| … | … |
| 12.6 | 5.1 |

For the typical MCS, e.g., 16QAM, 0.48, the requirement for PDSCH is around 14dB as shown in Figure 1. If the testable SNR for multi-Rx demodulation is lower than 14dB, then only the modulation of QPSK can be verified for multi-Rx demodulation testing.

Figure 1: Radiated PDSCH requirements specified in TS 38.101-4 [3]

In figures below, we show legacy antenna gain CDF of two UE implementations that have standards compliant gain drop for PC3 devices. The UE is assumed to be a box with 6 faces. Table 2 captures the fraction of directions on the sphere that meet the sensitivity condition for various X values.

| Identical 4x1 dual pol. modules on opposite faces | Identical 4x1 dual pol. modules on adjacent faces |
| --- | --- |

Figure 2: Legacy antenna gain CDF of two standard UE implementations

Table 2: Availability of qualifying test directions with different X values

| X (dB) | % of sphere with sensitivity better than REFSENS + X |  |
| --- | --- | --- |
|  | Modules on opposite faces | Modules on adjacent faces |
| 1 | 5 | 5 |
| 2 | 10 | 10 |
| 4 | 19 | 22 |

Observation 1: X = 2 dB implies 1/5th of all legacy spherical coverage directions (or 10% of all directions) will qualify for ‘standard’ UEs used for UE RF requirement simulation (i.e., those that just meet the legacy spherical coverage requirements).

In addition, the test directions need to satisfy the minimum isolation requirements of [12dB]. We provide the simulation to evaluate the number of AoA pair satisfying minimum isolation of [12dB]. The simulation assumptions are following the assumptions agreed in RF session in [3]. Module locations were grouped into two categories.

- In the first category, one module pattern covered the polar region of the reference coordinate system. Examples: Front+Bottom, Front+Top, Front+Right

- In the second category, both modules cover equatorial regions (away from polar regions). Examples: Top+Left, Top+Right

Note that the UE assumptions used in the simulations fully align with the assumption agreed in [4] in UE RF session. And it is reasonable to reuse the same UE assumptions in the discussion for multi-Rx demodulation.

Observation 2: The UE assumptions used in the simulations fully align with the assumption agreed in [4] in UE RF session. And it is reasonable to reuse the same UE assumptions in the discussion for multi-Rx demodulation.

The isolation is quantified as SIR at (PC3) UE receiver. In this case, the min. isolation of 12dB means the difference of signal level received from AoA1 and AoA2 are larger than 12dB. In addition, for each AoA pair, the test directions have sensitivity better than the legacy REFSENS requirements + X (X=1, 2, 4dB).

Figure 3: simulation results for number of AoA pairs for a 15⁰ step size grid, on adjacent faces, and with at least 12dB isolation with X = 1, 2, 4dB)

Figure 4: Simulation results for number of AoA pairs for a 15⁰ step size grid, on opposite faces, and with at least 12dB isolation with X = 1, 2, 4dB)

Figure 5: simulation results for number of AoA pairs for a 10⁰ step size grid, on adjacent faces, and with at least 12dB isolation with X = 1, 2, 4dB)

Figure 6: Simulation results for number of AoA pairs for a 10⁰ step size grid, on opposite faces, and with at least 12dB isolation with X = 1, 2, 4dB)

With the step size of 15⁰, from Figure 3, with X=2dB, the number of AoA pairs with at least 12dB isolation is 9. From Figure 4, with X=2dB, the number of AoA pairs with at least 12dB isolation is 6.

Observation 3: With 2dB, the number of AoA paris with at least 12dB isolation is at least 6 with 15⁰ step size grid.

The analysis of number of AoA pairs for 10⁰ step size is also provided. From Figure 5, with X=2dB, the number of AoA pairs with at least 12dB isolation is 34. From Figure 6, with X=2dB, the number of AoA pairs with at least 12dB isolation is 23.

Observation 4: With 2dB, the number of AoA paris with at least 12dB isolation is at least 23 with 10⁰ step size grid.

We should note that the for the multi-Rx demodulation testing, only one AoA pair is needed, which can be based on UE declaration. And in practice, UEs far exceed the spherical coverage requirement, which has the effect of increasing the number of qualifying directions for any relevant value of X.

Observation 5: Only one pair is needed for multi-Rx demodulation testing, which can be based on UE declaration.

Observation 6: In practice, UEs far exceed the spherical coverage requirement, which has the effect of increasing the number of qualifying directions for any relevant value of X.

Note that the SIR is reflecting the cross-interference between two TRPs. The minimum isolation of [12dB] should be guaranteed between polarizations. With a small value of X, it is easier to achieve the minimum isolation between polarizations.

Observation 7: With a small value of X, it is easier to achieve the minimum isolation between polarizations.

To retain a balance between ease of identifying directions and retaining testable SNR, it is proposed to adopt X = 2 as baseline.

Proposal 1: RAN4 to adopt X = 2 as the allowable degradation from legacy REFSENS requirements for Multi-Rx demodulation test directions selection.

## Minimum isolation requirements

In RAN4#108, the agreements for assumptions of minimum isolation requirements were captured in [1].

Issue 3-2-1: Assumptions for minimum isolation simulation and corresponding MU - Proposals - Option 1 (Qualcomm): The following assumptions are adopted for simulation of minimum isolation requirements. - For the reference SNR, the following assumptions could be considered: - Assume α = 0, β = 0, and γ = good enough isolation, e.g., 100dB - Channel model parameters - TDLA30-75 is assumed for 100 MHz/120 kHz - Time offset values: {0.25us, -0.0625us}/0; Frequency offset: 600Hz/0 - MCS: MCS13 with 2+2 - Receiver assumptions: Separate processing per Rx chain. - For comparison, to run the simulation with the isolation range of [8dB, 20dB] including both cross-polarizations and cross-talk and then compare the offset between reference SNR and required SNR with different isolation values. - Option 2: TBA - Agreement: - The following assumptions are adopted for simulation of minimum isolation requirements. - For the reference SNR, the following assumptions could be considered: - Assume α = 0, β = 0, and ρ=γ = good enough isolation, e.g., 100dB - Channel model parameters - TDLA30-75 is assumed for 100 MHz/120 kHz - Time offset values: 0; Frequency offset: 0 - MCS: MCS17 with 1+1 - Receiver assumptions: Separate processing per Rx chain. - For comparison, to run the simulation with the isolation range of [8dB, 20dB] including both cross-polarizations and cross-talk and then compare the offset between reference SNR and required SNR with different isolation values.

Based on the agreed assumptions, we have the following preliminary simulation results as the reference to decide the min. isolation for multi-Rx demodulation testing [3].

Table 3: Simulation assumptions

| Transmission Scheme | Multi-DCI fully overlapping |
| --- | --- |
| Correlation model | FR2 spatial correlation model [6] and α = 0, β = 0 |
| SCS/BW | 120KHz, 100MHz |
| Layer combination | 1L+1L |
| MCS | 17 (1L+1L) |
| Channel model | TDLA-30ns-75Hz |
| Precoding | Random |
| DMRS port index | 7 and 8 for TRP1 and TRP2, respectively |
| No. of PDSCH DMRS symbols | 2 (1+1) |
| No. of PDSCH symbols | 10 |
| Cross-polarization coefficient | 0.0625 |
| Carrier frequency | 28GHz |

Table 4: simulation results with different cross-talk isolation

| Independent Per TRP Processing (1L+1L) |  |
| --- | --- |
| -12dB | -15dB |
| MCS17 | MCS17 |
| 10 | 11.77 |

# Conclusions

In this paper, we discuss the test methods for UE demodulation for multi-Rx UE. The following observations and proposals are made:

Observation 1: X = 2 dB implies 1/5th of all legacy spherical coverage directions (or 10% of all directions) will qualify for ‘standard’ UEs used for UE RF requirement simulation (i.e., those that just meet the legacy spherical coverage requirements).

Observation 2: The UE assumptions used in the simulations fully align with the assumption agreed in [4] in UE RF session. And it is reasonable to reuse the same UE assumptions in the discussion for multi-Rx demodulation.

Observation 3: With 2dB, the number of AoA paris with at least 12dB isolation is at least 6 with 15⁰ step size grid.

Observation 4: With 2dB, the number of AoA paris with at least 12dB isolation is at least 23 with 10⁰ step size grid.

Observation 5: Only one pair is needed for multi-Rx demodulation testing, which can be based on UE declaration.

Observation 6: In practice, UEs far exceed the spherical coverage requirement, which has the effect of increasing the number of qualifying directions for any relevant value of X.

Observation 7: With a small value of X, it is easier to achieve the minimum isolation between polarizations.

Proposal 1: RAN4 to adopt X = 2 as the allowable degradation from legacy REFSENS requirements for Multi-Rx demodulation test directions selection.

# References

- R4-2309811, WF for FR2 OTA test SI, Qualcomm, RAN4#107, May 2023.

- R4-2313888, WF for FR2 OTA test SI, Qualcomm, RAN4#108, August 2023.

- TS 38.101-4

- R4-2303708, WF on NR FR2 multi-Rx chain DL reception, Samsung et al, RAN4#106, Athens, GR, Feb-Mar. 2023

- R4-2315352, Simulation Results on PDSCH Performance Requirements for FR2 Multi-Rx, October, 2023

- R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception,” 3GPP TSG RAN WG4 Meeting #107, Incheon, KR, May 22 – 26, 2023.