3GPP TSG-RAN WG4 Meeting #108bis									            R4-2316562

Xiamen, China, October 9 – 13, 2023

Agenda item:	5.24.2.4

Source:	Apple

Title:	Discussion on Enhanced CHO configurations

Document for:	Discussion

# Introduction

Enhanced CHO configurations have been discussed widely for several meetings. The latest agreement can be found in the approved WF [1]. Besides, a big CR [2] was endorsed to capture all the agreed core requirements. However, not all scenarios have been captured in the endorsed CRs in RAN4#108. In this contribution we provide our view on how to capture the related scenario.

# Discussion

### 2.1 CHO including target MCG and target SCG in NR-DC (obj. 3)

In the big CR [2] it can be seen that the following scenarios are captured:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC (clause 6.1.6.1)

- FR1-FR2 NR-DC to FR1-FR2 NR-DC (clause 6.1.6.2)

However, in the big CR [3] from R18 RRM enhancement work item the following scenarios are specified in requirements for handover with PSCell.

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

The yellow highlighted scenarios are missing in obj. 3.

Proposal 1: to align scenarios in requirements for handover with PSCell, the following scenarios shall be added in scope of obj.3.

- FR1-FR1 NR-DC to FR1-FR2 NR-DC

- FR1-FR2 NR-DC to FR1-FR1 NR-DC

### 2.2 CHO including target MCG and candidate SCG for CPC/CPA in NR-DC (obj. 4)

In the big CR [2] it can be seen that the following scenarios are captured:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC (clause 6.1.7.1)

- FR1-FR2 NR-DC to FR1-FR1 NR-DC (clause 6.1.7.1)

- FR1-FR2 NR-DC to FR1-FR1 NR-DC (clause 6.1.7.1.2)

- FR1-FR2 NR-DC to FR1-FR2 NR-DC (clause 6.1.7.1.2)

As it can be seen, the turquoise highlighted is redundant. Instead, it shall be FR1-FR1 to FR1-FR2, which is missing currently.

Proposal 2: fix the big CR to capture the following scenarios correctly for obj.4:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

# Conclusion

In this contribution, we provide further discussion on enhanced CHO configurations. After discussion, the following conclusions are provided:

Proposal 1: to align scenarios in requirements for handover with PSCell, the following scenarios shall be added in scope of obj.3.

- FR1-FR1 NR-DC to FR1-FR2 NR-DC

- FR1-FR2 NR-DC to FR1-FR1 NR-DC

Proposal 2: fix the big CR to capture the following scenarios correctly for obj.4:

- FR1-FR1 NR-DC to FR1-FR1 NR-DC,

- FR1-FR1 NR-DC to FR1-FR2 NR-DC,

- FR1-FR2 NR-DC to FR1-FR1 NR-DC,

- FR1-FR2 NR-DC to FR1-FR2 NR-DC.

# References

- R4-2314332, WF on NR Mobility Enhancements (Part 2), Apple

- R4-2314488, Draft Big CR to TS 38.133 on RRM core requirements for Further NR mobility enhancements, Apple

- R4-2310080, Big CR for R18 RRM enhancement – FR1+FR1 NR-DC, OPPO

- R4-2310064, WF on NR Mobility Enhancements RRM requirements (part 2), Apple.

- R4-2306396, WF on NR Mobility Enhancements RRM requirements (part 2), Apple.