3GPP TSG-RAN WG4 Meeting #108-bis                                                          R4-2316600

Xiamen, China, October 09th – 13th, 2023

Source:	Samsung

Title:	Remaining RRM core requirement issues for intra-band non-collocated NR-CA

Agenda item:			5.11.2

Document for:	Discussion

# 1. Introduction

In RAN4#108, the big CR on RRM requirements for supporting intra-band non-collocated NR-CA deployment has been agreed as [R4-2312870], while the only issue remaining for the RRM core requirement is how/whether the RRM core requirement impact for UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2 under discussion as captured in status report [RP-232113] as follows:

<RRM> - If needed, how UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2. This depends on the conclusion of UE RF.

After RAN discussion, RAN LS (RP-232692: LS on signaling support for intra-band non-collocated NR-CA, EN-DC) is approved to RAN2 and CC RAN4, in which the new UE capability is introduced to support non-collocated deployment for TDD-TDD intra-band NR-CA and the network signalling is introduced to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE.

In this contribution, we consider how to support this feature and reflect the content of the RAN LS in Rel-18 RAN4 RRM specification.

# 2. Discussion

### 2.1 UE capability on TDD-TDD intra-band NR CA

As mentioned in LS [1], the UE capability “interBandMRDC-WithOverlapDL-Bands-r16” was introduced since Rel-16 for UE to indicate the support of non-collocated deployment for FDD-FDD or TDD-TDD inter-band EN-DC operation with overlapping or partially overlapping bands, while the UE capability to support non-collocated deployment for TDD-TDD intra-band NR-CA is being discussed in Rel-18.

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

In short, UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 requirement, while the UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 1 requirement. In addition, at least one UE capability to indicate supporting TDD-TDD intra-band Non-Collocated NR-CA is required according to the RAN LS [1] and this UE capability description could be similar with interBandMRDC-WithOverlapDL-Bands-r16. In short, UE supporting the new UE capability (e.g. [intraBandNR-CA-non-collocated-r18]) applies Type 2 requirement, while the UE not supporting the new UE capability applies Type 1 requirement.

In addition, RAN2/RAN4 is working on changing the field description interBandMRDC-WithOverlapDL-Bands-r16 i.e. the terminology Type 1/2 are not used in the specifications (i.e. describe the feature referring the RAN4 specification).

### 2.2 New RRC signalling for NR CA

According to LS [1], RAN agreed to introduce new RRC signalling to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured with the value of IE maxMIMO-Layers no more than 2. See the below description on this RRC singaling.

2) New BS signalling During RAN#101 meeting, RAN agreed to introduce new BS signalling by RRC to indicate whether Type 1 or Type 2 capability requirements will be applied by a UE configured by the network with the value of IE maxMIMO-Layers no more than 2 and task RAN2 to develop relevant signalling. This new BS signalling is applied to the below cases. For TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands This new BS signalling is expected to be applicable for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands provided the UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”. For TDD-TDD intra-band NR-CA This new BS signalling is expected to be applicable for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”.

Based on the above LS on TDD-TDD intra-band NR-CA, we expect the new BS signalling will be introduced for TDD-TDD intra-band NR-CA provided the UE has indicated “New UE capability”. Accordingly, we expect the changes on the relevant specification as:

Proposal 1: RAN4 shall take the newly introduced BS signaling into RRM specification implementation for UE supporting [intraBandNRCA-NonCollocated-r18]. Below can be treated as an example:

- For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in Table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18] and BS signaling [xxx] is indicated.

### 2.3 New RRC signalling for EN-DC

Note that “interBandMRDC-WithOverlapDL-Bands-r16” for EN-DC and “the new UE capability” for NR-CA are two independent UE capabilities. And a SA NW has no idea of whether UE indicating “interBandMRDC-WithOverlapDL-Bands-r16”, similarly an NSA NW has no idea of whether UE indicating “New UE Capability” for NR-CA.

Below is our understanding on the applicability of the new RRC signaling for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands:

- Rel-16/17 NW: No new RRC signaling

  - UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 requirement

  - UE not supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 1 requirement

- Rel-18 NW: With the new RRC signaling introduction (And without early implementation in RAN2)

  - If new RRC signaling is present,

    - The Rel-18 UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 requirement.

    - The Rel-16/17 UE cannot understand this new RRC signaling.

  - If new RRC signaling is not present,

    - The 18 UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 1 capability requirement.

    - The Rel-16/17 UE supporting interBandMRDC-WithOverlapDL-Bands-r16 applies Type 2 capability requirement.

With above understanding, in terms of TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands, we think it should be further clarified that this new RRC BS signaling is supposed to instruct Rel-18 and forward UE (provided this Rel-18 UE has indicated “interBandMRDC-WithOverlapDL-Bands-r16”), rather than Rel-16/17 UE. To serve this purpose, a general solution in RAN2 is proposed to introduce a new UE capability in clause 4.2.2 (General parameters) of 38.306 to indicate whether the UE supports the network controlled indication, something like below:

Please note that NW may cannot understand directly the UE is a Rel-16 UE or a Rel-18 UE, and there is also no intention to bring RAN2 things into RAN4. Above RAN2 signalling is just information sharing, for RAN4’s better understanding. It should be noted that the same discussion is also discussed in RF session and we expect RRM session can further follow the conclusion made by RF session.

Proposal 2: The discussion on RRM requirement impact for TDD-TDD inter-band EN-DC with overlapping bands could be delayed until clear interpretation for the new BS signaling is concluded in RAN4 RF session.

# 3. Conclusion

In this contribution, we consider the remaining RRM core requirement impact for intra-band non-collocated NR-CA, EN-DC, to reflect the content of the RAN LS. The following two proposals are obtained:

Proposal 1: RAN4 shall take the newly introduced BS signaling into RRM specification implementation for UE supporting [intraBandNRCA-NonCollocated-r18]. Below can be treated as an example:

- For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative transmission timing difference as shown in Table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r18] and BS signaling [xxx] is indicated.

Proposal 2: The discussion on RRM requirement impact for TDD-TDD inter-band EN-DC with overlapping bands could be delayed until clear interpretation for the new BS signaling is concluded in RAN4 RF session.

# 4. Reference

RP‑232692, “LS on signaling support for intra-band non-collocated NR-CA, EN-DC”, KDDI, RAN-P#101.

R4-2312870, “BigCR on RRM requirements for supporting intra-band non-collocated NR-CA”, Huawei, RAN4#108.