3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316631

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	NCR Demodulation Performance Requirements

Agenda item:	5.28.6

Document for:	Discussion

Introduction

In RAN4#108, the discussion on Network-Controlled Repeater (NCR)-Mobile Termination (MT) demodulation performance requirements has taken place, and the way forward (WF) is documented in [1]. In the following, we summarize the related WF that will be further discussed in this contribution:

Issue 1-1-2: MCS for PDSCH FR2 requirements - Agreements: - MCS 4 is for PDSCH FR2. - QPSK and 16QAM for PDSCH FR1. Issue 1-1-3: Test scope for PDSCH FR1 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); MCS; Propagation condition; Antenna configuration; Test metric. 1; 10; 15; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 2; 10; 15; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 3; 10; 15; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 4; 10; 15; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. 5; 40; 30; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 6; 40; 30; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 7; 40; 30; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 8; 40; 30; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. Issue 1-1-4: Test scope for PDSCH FR2 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); Propagation condition; Antenna configuration; Test metric. 1; 100; 120; TDLA30-75; 2x2; 70% max throughput 1% BLER. Issue 1-2-1: Whether to define new requirements on PDCCH for signaling of Access link beam change indication - Agreements: - FFS whether new PDCCH requirements needed or not for DCI format 5_0/2_8 with large payload size. Issue 1-2-2: Test scope for PDCCH FR1 - Agreements: - Following table used for simulation purpose on the requirements with existing DCI if introduced. Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix. 10; 48; 1; 8; TDLA30-10; 2x4 Low. Issue 1-2-3: Test scope for PDCCH FR2 - Agreements: - No new simulation for existing DCI, FFS for new DCI if introduced. Issue 1-3-1: Whether to define PMI requirements - Agreements: - FFS for whether to define PMI requirements.

Discussion

As summarized above, there are three open subtopics in NCR Demodulation that need to be discussed in RAN4#108bis. We provide our views on each of those subtopics in this section. We start with PDSCH requirements followed by PDCCH requirements and PMI requirements.

PDSCH Requirements

The following three issues from RAN4#108 are related to PDSCH requirements:

Issue 1-1-2: MCS for PDSCH FR2 requirements - Agreements: - MCS 4 is for PDSCH FR2. - QPSK and 16QAM for PDSCH FR1. Issue 1-1-3: Test scope for PDSCH FR1 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); MCS; Propagation condition; Antenna configuration; Test metric. 1; 10; 15; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 2; 10; 15; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 3; 10; 15; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 4; 10; 15; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. 5; 40; 30; 4; TDLA30-10; 2x2; 70% max throughput 1% BLER. 6; 40; 30; 4; TDLA30-10; 2x4; 70% max throughput 1% BLER. 7; 40; 30; 13; TDLA30-10; 2x2; 70% max throughput 1% BLER. 8; 40; 30; 13; TDLA30-10; 2x4; 70% max throughput 1% BLER. Issue 1-1-4: Test scope for PDSCH FR2 - Agreements: Case number; Bandwidth (MHz); SCS (kHz); Propagation condition; Antenna configuration; Test metric. 1; 100; 120; TDLA30-75; 2x2; 70% max throughput 1% BLER.

Concerning the simulation activities, we have provided our simulation results on NCR-MT PDSCH in RAN4#108 as documented in [2]. The agreements in RAN4#108’s WF in [1] have included the test cases for both FR1 and FR2. Hence, for the discussions in RAN4#108bis, we provide our simulation results on NCR-MT PDSCH in our contributions in [7], as a reference, based on the agreed test cases and parameters in [1].

Test metrics and test cases for NCR-MT PDSCH have been agreed by interested companies in RAN4#108.

Simulation results for NCR-MT PDSCH from interested companies are available in RAN4#108 and RAN4#108bis meetings.

RAN4 to discuss the PDSCH simulation results, do the alignment and decide on the final requirements on PDSCH for NCR-MT.

PDCCH requirements

The next three open issues are about the PDCCH requirements as given below:

Issue 1-2-1: Whether to define new requirements on PDCCH for signaling of Access link beam change indication - Agreements: - FFS whether new PDCCH requirements needed or not for DCI format 5_0/2_8 with large payload size. Issue 1-2-2: Test scope for PDCCH FR1 - Agreements: - Following table used for simulation purpose on the requirements with existing DCI if introduced. Bandwidth (MHz); CORESET RB; CORESET duration; Aggregation level; Propagation Condition; Antenna configuration and correlation Matrix. 10; 48; 1; 8; TDLA30-10; 2x4 Low. Issue 1-2-3: Test scope for PDCCH FR2 - Agreements: - No new simulation for existing DCI , FFS for new DCI if introduced.

Concerning Issue 1-2-1, we have provided our views in RAN4#107 and RAN4#108 meetings. They are again provided in the following.

In the current requirements, the receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg). Corrodingly, for IAB-MT, DCI formats 1_0 and 1_1 are defined in the fixed reference channels (FRC) for PDCCH in Appendix A of [3].

However, for NCR-MTs, the scheduling grants may be used rarely, because the support of user-plane traffic is optional.

On the other hand, it is agreed in RAN1 [4][5] that the access link beam change indication will be carried by DCI type 5_0 (or DCI format 2_8, the naming is not yet final). While the minimum requirements criteria of 1% of probability of misdetection can be used, necessary adaptation in PDCCH test parameters should be done for DCI type 5_0. Some of the corresponding L1 agreements are listed below:

RAN1#112 [4] Agreement The DCI Format 5_0 carrying the side control information is monitored by the NCR-MT at least in the UE specific search space. - Note: The existing configuration of UE specific search space will be reused. Agreement For periodic and semi-persistent beam indication, - The maximum number of periodic beam indication is 32. - The maximum supported number of beam is 64 (to determine the value range of beam index). - The maximum number of duration of time resource is 112 symbols. For periodic beam indication, The maximum number of forwarding resources in one periodic beam indication is 1024. RAN1#112-bis-e [5] Agreement (NR network-controlled repeaters) For the aperiodic beam indication via DCI 5-0, the following option is supported: - Option-4: - The DCI size of DCI Format 5-0 is implicitly determined by the RRC configuration with the maximum value as 128. - The [maximum] number of fields for time resource indication (Tmax) is explicitly configured by dedicated RRC parameter with the maximum value as [16] or [32]. - FFS: How to support and address an actual number of fields for time resource indication is smaller than the configured maximum number.

In some configurations, DCI 5_0 size can be pretty large (max size 128 bits), because it can carry access beam configuration for multiple time resources and beams. The demodulation performance of DCI payloads of such large sized have not yet been covered in RAN4 before.

For NCR-MT, scheduling grant (i.e., DCI format 1_0/1_1) is not typical, whereas DCI type 5_0/2_8 can be used for access link beam change indication.

The DCI size can have the maximum value of 128 and the demodulation performance of DCI payloads of such large sized have not yet been covered in RAN4 before.

RAN4 shall define new demodulation performance requirements for NCR PDCCH.

RAN4 shall adapt test parameters for NCR PDCCH requirements following DCI format 5_0/2_8 at least to adapt the max payload size accordingly (i.e., 128 bits).

We note here that, as discussed by companies both online and offline in RAN4#107 meeting, interested companies can provide initial simulation for PDCCH based on the parameters for IAB/UE specifications, and to adapt only the size of the payload. To support our proposal, we provide simulation results on PDCCH with DCI format 5_0/2_8 in [2], in which we adapt the parameter for the payload to the maximum value (128 bits). From the simulation results in [2], compared to the original DCI size in [3], DCI Type 5_0/2_8 requires considerably higher SNR. As one example case, we provide the comparison below in Table 1:

Table 1. Simulation Results on PDCCH with 30 kHz SCS: DCI 1_0 versus DCI 5_0/2_8

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value (DCI 1_0) |  | Reference value (DCI 5_0/2_8) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) | Pm-dsg (%) | SNR (dB) |
| 40 | 102 | 1 | 2 | TDLA30-10 | 1x2 Low | 1 | 3.4 | 1 | 10.6 |

For discussions in RAN4#108bis, we provide 3 tables in [7] to compare the required SNRs of legacy DCI (1_0) with DCI 5_0/2_8 for test cases of IAB-MT and UE as in the current IAB-MT and UE specifications, based on the agreed parameters for NCR-MT. They are tabulated in Table 3, Table 4, and Table 5 in [7] for FR1 TDD, FR1 FDD and FR2 TDD, respectively. From those tables, it can be seen that DCI 5_0/2_8 requires much higher SNR values than DCI 1_0. Hence, as in our proposal above, RAN4 shall define new demodulation performance requirements for NCR PDCCH with DCI 5_0/2_8.

Simulation results for NCR-MT PDCCH with payloads for DCI 5_0 are available in both RAN4#108 and RAN4#108bis.

RAN4 shall consider the simulation results on PDCCH with new payload for DCI 5_0/2_8 in the discussion for new PDCCH requirements and discuss the final requirements on PDCCH for NCR-MT.

Concerning Issue 1-2-2 and 1-2-3, they depend on the decision made in RAN4 on Issue 1-2-1.

For PDCCH requirements, for legacy DCI (1_0), IAB-MT or UE requirements can be reused for NCR-MT.

Based on the agreements made for PDCCH for NCR-MT, one additional test case is to be added to legacy DCI (1_0) requirement for FR1, and there will be no new simulation needed for FR2.

If there are no new requirements to be introduced on PDCCH for DCI 5_0/2_8, legacy requirements can be reused with one additional test case for FR1.

PMI requirements

The next open issue is about PMI requirements as below:

Issue 1-3-1: Whether to define PMI requirements - Agreements: - FFS for whether to define PMI requirements.

In RAN4#106bis-e, RAN4#107 and RAN4#108 discussions, we have provided our views on this issue. Our views are summarized in the following:

As NCR is part of the network device, in the network planning process, NCR will be deployed in a location with a relatively good channel condition to the gNB. Furthermore, as both NCR and gNB are in a fixed location and not mobile, the channel between them will be considerably stable and not change frequently. In general, such a good condition on the first hop is needed to ensure that the use of the intermediate node (in a form like relay, IAB, RF repeater or NCR) would bring benefits to the network. As an example, in a decode and forward relaying, the relay node should not be used if the minimum SINR between the first hop and the second hop is less than the SINR of the direct link, cf. [6]. We note here that in NCR, the good channels for the first hop are intended for NCR-FWD backhaul link. It is expected that NCR-FWD will carry much higher data rate compared to NCR-MT which uses the same channel. Hence, NCR-MT will benefit from the good channels of NCR-FWD coming from the network planning.

In a usual direct communication between gNB and UE, PMI is intended to maximize the throughput based on the channel condition.  Nonetheless, it is expected that the C-link in NCR-MT will only carry low data rate for carrying only side control information.

The arguments to have PMI reporting is the change of the channel condition. Nonetheless, the changing of the channel condition can already be adapted by adapting the CQI, hence, the MCS. In RAN4#106bis-e it is already agreed to define CQI requirements.

NCRs are part of the network and can profit from network planning. Good channel conditions and known spatial transmission/receptions environments can be assumed. Overall low data rates are expected for NCR control information, hence fixed PMI or even no-PMI will be feasible in deployment. Link adaptation is covered in CQI requirements.

RAN4 shall not consider defining requirements for PMI reporting.

Conclusion

In this paper, we provide our views on the remaining open issues on NCR demodulation performance requirements. They are summarized in the following Observations and Proposals:

Observation 1: Test metrics and test cases for NCR-MT PDSCH have been agreed by interested companies in RAN4#108.

Observation 2: Simulation results for NCR-MT PDSCH from interested companies are available in RAN4#108 and RAN4#108bis meetings.

Proposal 1: RAN4 to discuss the PDSCH simulation results, do the alignment and decide on the final requirements on PDSCH for NCR-MT.

Observation 3: For NCR-MT, scheduling grant (i.e., DCI format 1_0/1_1) is not typical, whereas DCI type 5_0/2_8 can be used for access link beam change indication.

Observation 4: The DCI size can have the maximum value of 128 and the demodulation performance of DCI payloads of such large sized have not yet been covered in RAN4 before.

Proposal 2: RAN4 shall define new demodulation performance requirements for NCR PDCCH.

Proposal 3: RAN4 shall adapt test parameters for NCR PDCCH requirements following DCI format 5_0/2_8 at least to adapt the max payload size accordingly (i.e., 128 bits).

Observation 5: Simulation results for NCR-MT PDCCH with payloads for DCI 5_0 are available in both RAN4#108 and RAN4#108bis.

Proposal 4: RAN4 shall consider the simulation results on PDCCH with new payload for DCI 5_0/2_8 in the discussion for new PDCCH requirements and discuss the final requirements on PDCCH for NCR-MT.

Observation 6: For PDCCH requirements, for legacy DCI (1_0), IAB-MT or UE requirements can be reused for NCR-MT.

Observation 7: Based on the agreements made for PDCCH for NCR-MT, one additional test case is to be added to legacy DCI (1_0) requirement for FR1, and there will be no new simulation needed for FR2.

Proposal 5: If there are no new requirements to be introduced on PDCCH for DCI 5_0/2_8, legacy requirements can be reused with one additional test case for FR1.

Observation 8: NCRs are part of the network and can profit from network planning. Good channel conditions and known spatial transmission/receptions environments can be assumed. Overall low data rates are expected for NCR control information, hence fixed PMI or even no-PMI will be feasible in deployment. Link adaptation is covered in CQI requirements.

Proposal 6: RAN4 shall not consider defining requirements for PMI reporting.

- References

- R4-2313923, WF on NCR_MT demodulation requirements, ZTE Corporation, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023

- R4-2312803, Simulation Results on NCR-MT PDSCH and PDCCH Demodulation Requirements, Nokia, Nokia Shanghai-Bell, 3GPP RAN4#108, Toulouse, France, August 21 – 25, 2023

- 3GPP TS 38.174, Integrated access and backhaul radio transmission and reception, V17.2.0, 12-2022

- R1-2302256, Final Report of 3GPP TSG RAN WG1 #112 v1.0.0, MCC Support, Athens, Greece, 27th February – 3rd March 2023

- R1-2304301, Final Report of 3GPP TSG RAN WG1 #112bis-e v1.0.0, MCC Support, Online, April 17th - 26th, 2023

- A. Amah and A. Wittneben, “Opportunistic relaying for wireless multicasting,” IEEE International Symposium on Personal, Indoor, and Mobile Radio Communications (PIMRC), London, UK, 08-11 September 2013.

- R4-2316632, Simulation Results on NCR-MT PDSCH and PDCCH Demodulation Requirements, 3GPP RAN4#108bis, Xiamen, China, October 09 – 13, 2023