3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316634

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on HST FR2 Enhanced with Carrier Aggregation

Agenda item:	5.12.4.2

Document for:	Information

Introduction

In RAN4#107, some companies including Nokia have provided preliminary simulation results on HST FR2 UE Demodulation PDSCH with Carrier Aggregation (CA), which are documented in [1]. It is worth to be mentioned here, that those early simulation results were without considering any impairments and/or additional margin. In RAN4#108, companies provided simulation results for the two cases, namely, with and without impairments. The simulation results from RAN4#108 is documented in [2]. Furthermore, the WF from RAN4#108 on HST FR2 PDSCH with CA is documented in [3].

Another related outcome from RAN4#108 is the simulation assumptions for HST FR2 PDSCH with CA. Those simulation assumptions in [5] will be used by companies to conduct simulation for HST FR2 PDSCH with CA, which results will be then discussed in RAN4#108bis to define the requirements. The agreed simulation assumptions from [5] for PDSCH with CA is provided in the Appendix for a direct reference.

Discussion

In this paper, we provide our simulation results on HST FR2 Enhancement UE Demodulation PDSCH with CA. Our complete views and responses to the open issues in WF [3] can be found in our discussion paper [4].

The simulation results on HST FR2 UE Demodulation PDSCH with CA without impairments are tabulated in Table 1 below:

Table 1. Simulation Results on HST FR2 PDSCH with CA without Impairments

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,4 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,5 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 11,4 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 10,5 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,4 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,5 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 11,4 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 10,5 |

The simulation results on HST FR2 PDSCH with CA with impairments are tabulated in Table 2 below:

Table 2. Simulation Results for Different Test Cases for HST FR2 with CA with impairments

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 13.1 |
| 1-4 | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 14.1 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 13.1 |
| 2-4 | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 14.1 |

As expected, the simulation results with impairments require higher SNR to achieve the same target performance of 70% throughput compared to without impairments. We provide our observations and proposals related to the simulation results above in [4].

Conclusion

In this paper, we provide our simulation results on HST FR2 UE Demodulation PDSCH with CA, in which the simulation results are now considering both with and without impairments. As expected, the simulation results with impairments require higher SNR than without impairments to achieve the same target performance of 70% throughput.

- References

- R4-2308834, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4#107, Incheon, South Korea, May 22 - 26, 2023.

- R4-2312211, Simulation results summary for Rel-18 FR2 HST demodulation requirement, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- R4-2313919, WF for FR2 HST demodulation requirements, Samsung, RAN4#108, Toulouse, France, August 21 - 25, 2023.

- R4-2316633, HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Carrier Aggregation, Nokia, Nokia Shanghai Bell, RAN4 #108bis, Xiamen, China, October 09 – 13, 2023.

- R4-2313920, Simulation assumption for FR2 HST demodulation, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- Appendix

Table A-1 Test Cases

| Test number | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | FRC 1-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-2 | FRC 1-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-3 | FRC 1-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | TBD |
| 1-4 (For information) | R.PDSCH.5-12.2 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-BI-B | 1 | 2x2 | 70 | 13.9 |
| 2-1 | FRC 2-1 | 50 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-2 | FRC 2-2 | 100 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-3 | FRC 2-3 | 400 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | TBD |
| 2-4 (For information) | R.PDSCH.5-12.1 TDD | 200 / 120 | 64QAM, 0.43 | FR2.120-1 | HST-DPS-FR2-UNI-A | 2 | 2x2 | 70 | 13.7 |

Table A-2. FRC for test 1 (HST-DPS-FR2-BI-B, single TCI states)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.2 TDD (For information) | FRC 1-1 | FRC 1-2 | FRC 1-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 | 13 |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Allocated slots per 2 frames |  | 124 | 124 | 124 | 124 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | [13832] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slots i = 5 and 85 (Note 3) | Bits | 181968 | [33408] | [76632] | [392640] |
| For Slots i = 6 and 86 (Note 3) | Bits | 181968 | [33408] | [76632] | [f392640] |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| Max. Throughput averaged over 2 frames | Mbps | 508.251 | [121.433] | [254.15] | [1016.651] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |

Table A-3. FRC for test 2 (HST-DPS-FR2-UNI-A, two TCI state)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.5-12.1 TDD (For information) | FRC 2-1 | FRC 2-2 | FRC 2-3 |
| Channel bandwidth | MHz | 200 | 50 | 100 | 400 |
| Subcarrier spacing | kHz | 120 | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 132 | 32 | 66 | 264 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 | 9 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 | 13 |
| For slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For slot i = 2 |  | 13 | 13 | 13 | 13 |
| For slot i = 3 |  | 9 | 9 | 9 | 9 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  | 17 | 17 | 17 | 17 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 | 0.43 |
| Number of MIMO layers |  | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 | 18 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 | 18 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 18 | 18 | 18 | 18 |
| For Slot i = 3 |  | 18 | 18 | 18 | 18 |
| Overhead for TBS determination |  | 6 | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 57376 | [13832-] | [28680] | [114776] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 90176 | [21504] | [45096] | [180376] |
| For Slot i = 3 |  | 57376 | [13832] | [28680] | [114776] |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 | 24 |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 24 | 24 | 24 | 24 |
| For Slot i = 3 |  | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | [2] | [4] | [15] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 11 | [3] | [6] | [22] |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 11 | [2] | [4] | [15] |
| For Slot i = 3 |  | 7 | [3] | [6] | [22] |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slots i = 5 and 85 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slots i = 6 and 86 (Note 3) | Bits | 195696 | [41856] | [90360] | [406368] |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 84, 87, …, 159} | Bits | 137808 | [33408] | [68904] | [275616] |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |
| For Slot i = 2 |  | 210672 | [51072] | [105336] | [421344] |
| For Slot i = 3 |  | 137808 | [33408] | [68904] | [275616] |
| Max. Throughput averaged over 2 frames | Mbps | 515.629 | [123.2] | [257.839] | [1031.409] |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

Table A-4. Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 1 and 2 |
|  |  |  |  | 6 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 6 |
|  |  |  |  | l0 = 8 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 5 and 6 |
|  |  |  |  | 6 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 9 and 10 |
|  |  |  |  | 6 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 14 |
|  |  |  |  | l0 = 8 for CSI-RS resource 15 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 13 and 14 |
|  |  |  |  | 6 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 17 and 18 |
|  |  |  |  | 6 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 22 |
|  |  |  |  | l0 = 8 for CSI-RS resource 23 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 21 and 22 |
|  |  |  |  | 6 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 25 and 26 |
|  |  |  |  | 6 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 30 |
|  |  |  |  | l0 = 8 for CSI-RS resource 31 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 29 and 30 |
|  |  |  |  | 6 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 (Note2) | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 (Note2) | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 (Note2) | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 (Note2) | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 (Note2) | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 (Note2) | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 (Note2) | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in TS38.101-4 Annex A.1.3 |
| Note 1: For Test 1-x, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0 (i≠0). PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=12k-2n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=2,3,4… to slot# 2k-1n+THARQ+TMAC proc,k=1,2,3…, PDCCH and PDSCH associated with TCI # ((k mod 4)+8) is transmitted by kth RRH from slot# 2k+1n+1+THARQ+TMAC proc+TfirstSSB+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=0,1,2… to slot# [2k+1n+THARQ+TMAC proc],k=0,1,2…, where k is the RRH number, n = 28800 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing, TfirstSSB = 132 is the number of slots to first SSB transmission occasion after MAC CE command is decoded by the UE, TSSB proc= 16 is the number of slots for SSB processing, TfirstTRSafterSSB = 66 is the number of slots to first TRS transmission occasion after first SSB is processed by the UE, TTRSproc= 16 is the number of slots for TRS processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, SSB # (k mod 4) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0. PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=1k-1n+2+THARQ+TMAC proc,k=2,3,4… to slot# [k * n+THARQ+TMAC proc], k=1,2,3…, where k is the RRH number, n = 57600 is half of the number of slots between two RRH, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. Note 2: Only configured for Test 1-x. |  |  |  |  |