3GPP TSG-RAN WG4 Meeting #108bis 	R4-2316636

Xiamen, China, October 09 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception

Agenda item:	5.12.4.3

Document for:	Information

Introduction

The way forward (WF) from the discussions on HST FR2 Enhanced PDSCH with multi-RX is documented in [1]. Several issues related to simulation on PDSCH with multi-RX are summarized below:

Issue 3-2-1: Transmission schemes Agreement: - Do not consider single-DCI based multi-TRP scheduling for PDSCH requirement Issue 3-2-3: PDSCH resource scheduling for requirements Agreement: - Only cover full overlapping Issue 3-2-4: Layer combination for full overlapping Agreement: - 2+2 Issue 3-2-6: PDSCH rate matching in mTRP transmission Agreement: - PT-RS allocation does not overlap with PDSCH allocation per TRP Issue 3-2-7: PDSCH scheduling and Number of DMRS in TDD DL special slot Agreement: - PDSCH scheduling in the special DL slot is assumed with 3 DMRS configuration Issue 3-2-8: Number of SSB and TCI state configuration for each cell Agreement: - Maximum 8 SSB and TCI states configuration for each cell is configured Issue 3-2-9: Number of active TCIs tracking Agreement: - RAN4 to assume the baseline behaviour for FR2 HST UE under test is to track 2 Active TCI states, one per panel Issue 3-2-10: TRS periodicity Agreement: - RAN4 to choose 10ms TRS periodicity to align with existing FR2 HST requirements Issue 3-2-11: PDSCH allocation timeline in the UE Demod Test Agreement: - PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. - The detail of scheduling pattern can be further discussed in the next meeting - Option 1: RAN4 to follow the same approach used in FR1 HST DPS requirements and FR2 HST DPS requirements (Bidirectional) and define a PDCCH/PDSCH allocation timeline independent per each Panel, including a number of slots in which PDCCH and PDSCH are DTX and throughput statistics are not considered, starting from the TCI state switch and until the UE has received one TRS from the new RRH (TfirstTRS) and has had time to process it (TTRSproc), including THARQ and TMAC proc; - Other options are not precluded. Issue 3-2-12: Number of MCS for PDSCH requirement for multi-Rx reception Agreement: - It is feasible to define PDSCH demodulation requirement per Rx panel with different MCS per Rx panel - Companies are encouraged to check the feasibility of MCS pair (MCS 17, MCS [13]) or (MCS 17, MCS [19]) in next meeting - Other feasible MCS pair are not precluded Issue 3-2-13: Power scaling for two served RRH Agreement: - Apply a constant scaling factor 1/2 to the transmitted PDSCH signal from each TRxP, as same as Rel-16 multi-TRP SDM transmission scheme Issue 3-2-15: Simulation Assumption for initial simulation purpose - Companies are encouraged to provide simulation results for PDSCH requirement with multi-Rx reception in the RAN4#108bis meeting based on simulation assumption in R4-2313920

One outcome in RAN4#108 is the updated simulation assumptions and parameters, which is documented in [3]. With such updates of simulation assumptions and parameters, companies need to provide new simulation results according to [3]. We provide the simulation parameters for HST FR2 PDSCH with multi-RX as in [3] in the Appendix for a direct reference.

Discussion

In this paper, we provide our simulation results based on the above-mentioned WF. We note here, there are still some related discussions to be agreed in RAN4 before defining the final requirements. For example, the discussions on the power profile and the possibility of changing the MCS during the train movement according to the received power. Nonetheless, the current simulation results from companies can already be used to have constructive discussion on defining the requirements for HST FR2 PDSCH with multi-RX. More of our observations and proposals related to HST FR2 PDSCH with multi-RX can be found in [2].

Below are our simulation results for the agreed case, i.e., mDCI with 2+2 layer combination and independent processing, presented in Table 1:

Table 1. Simulation Results on HST FR2 PDSCH with multi-RX with mDCI and independent processing

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (17,13) | 2 | 2x2 Low | 70 | (11.4, 7.5) |
| 200 | 120 | 30 | (17,19) | 2 | 2x2 Low | 70 | (11.4, 12.6) |

Conclusion

In this paper we provide initial simulation results with the agreed simulation assumptions and parameters as in WF [1]. While we provide our simulation with the given assumptions in this contribution, we provide our observations and proposals in [2].

- References

- R4-2313919, WF for FR2 HST demodulation requirements, Samsung, RAN4#108, Toulouse, France, August 21 - 25, 2023.

- R4-2316635, HST FR2 Enhanced: UE Demodulation PDSCH Requirements with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4 #108bis, Xiamen, China, October 09 – 13, 2023.

- R4-2313920, Simulation assumption for FR2 HST demodulation, Samsung, RAN4 #108, Toulouse, France, August 21 – 25, 2023.

- Appendix

Table A. Simulation assumption for PDSCH demodulation requirements with multi-Rx simultaneous reception [3]

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| UL-DL configuration |  |  |  | FR2.120-1 |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | 13 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 2 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | full-overlapping |  |
| Number of HARQ Processes |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | As defined in Annex A.1.3 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| - Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | TRxP #1 | TRxP #2-1 | TRxP #2-2 |  |
| Channel bandwidth | MHz | 200 | 200 | 200 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 132 | 132 | 132 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 13 | 19 |  |
| Modulation |  | 64QAM | 16QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.48 | 0.50 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 36 | 36 | 36 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 36 | 36 | 36 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 51216 | 33816 | 53288 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 77896 | 57376 | 92200 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 7 | 5 | 7 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 10 | 7 | 11 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 153456 | 102304 | 153456 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 125136 | 72864 | 109296 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 182160 | 121440 | 182160 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 441.601 | 319.213 | 511.326 |  |
| - Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms - Note 2: Slot i is slot index per 2 frames - Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. - Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |