3GPP TSG-RAN WG4 Meeting #108bis	R4-2316663

Xiamen, China, Oct 9th – 13th, 2023

Agenda Item:	5.4.2

Source:	MediaTek Inc.

Title: 	On UE capability on channel raster enhancement

Document for:	Discussion

# Introduction

RAN4#108 continued to discuss the channel raster enhancements, and a WF was agreed with the following open questions on UE capability where a new UE capability is agreed for this WID [1]:

Open issues: The new UE capability should be per band. FFS from which release should the UE capability be applicable. Whether the capability should (at least for some bands) be mandatory from Rel-18 onwards.

In this contribution, we continue discussing these open issues related to the new UE capability for channel raster enhancement.

# Discussion

For the reporting granularity of the new UE capability, it is practically not sensible that a UE can support the enhanced channel raster for some bands, but at the same time it does not support the feature for other bands, thus a per-UE capability should be sufficient. However, if considering other aspects such as IoDT, per-band capability might be helpful.

Proposal 1: The new UE capability supporting enhanced channel raster is set as per-UE capability if not considering other aspects e.g., IoDT.

Regarding its early implementation, this channel raster enhancement is an independent feature, and it is only applicable to RRC_CONNECTED state, thus it can be implemented early from Rel-15, however, if some potential NBC issue is identified, at least the early implementation can be from Rel-17.

Proposal 2: The new UE capability can be implemented early from Rel-15 if no potential NBC issue is identified, otherwise, early implementation can be from Rel-17.

# Conclusion

In this contribution we have the following proposals for the new UE capability for channel raster enhancement:

Proposal 1: The new UE capability supporting enhanced channel raster is set as per-UE capability if not considering other aspects e.g., IoDT.

Proposal 2: The new UE capability can be implemented early from Rel-15 if no potential NBC issue is identified, otherwise, early implementation can be from Rel-17.

# References

- R4-2314681	WF on channel raster enhancement, Ericsson