3GPP TSG-RAN WG4 Meeting #108bis                                                         R4-2316689

Xiamen, China, October 09 - 13, 2023

Source:	ZTE Corporation, CHTTL

Title:	TP for TR 38.846: Guidelines on delta TIB and RIB special values for band combinations

Agenda Item:	5.1.3

Document for:	Approval

- Introduction

There are some special delta TIB and RIB values for the band combinations having SUL, SDL, immediately close component band, band combination with overlapping component band, and EN-DC combination with LTE LAA component band in the current specifications. However, the denotations for the delta TIB and RIB values for these special constituent band are inconsistent among different band combinations, some of which having the value of “-”, some of which having the value of “N/A”, while some of others having the value of non-zero number. For the values of delta TIB,c and RIB,c, it is noted that the ΔTIB,c / ΔRIB,c is set to zero which is denoted as “-” in the specification. For the constituent band of the DC/CA band combination which does not support uplink/downlink, the deriving delta T/R requirements should not be applicable to the corresponding band. This contribution provides a text proposal on the guidelines for specifying the delta TIB and RIB value for band combination with special component band such as SDL, SUL, immediately close band, band combination with overlapping band, and EN-DC combination with LTE LAA band, etc.

- Reference

- TR 38.846, Study on simplification of band combination specification for NR and LTE v1.1.0

- R4-2316688, Further considerations on delta TIB and RIB special values for band combinations, ZTE Corporation, CHTTL, RAN4#108bis.

- Text Proposal

----- Start of TP -----

6.3	Guidelines on delta TIB and RIB due to band combinations

To optimize the tables of ΔTIB,c and ΔRIB,c due to band combinations, a new template for Rel-18 is proposed in clause 8.3.2 in TR 38.862.

Regarding to the optimized template for ΔTIB,c and ΔRIB,c tables, only the configurations having the same component E-UTRA / NR bands can be grouped into one cell (row). For example, in Table 6.3-1 for the ΔTIB,c of the following inter-band EN-DC configurations, since the component bands are not the same, two rows should be filled separately in the new template. However, for the configurations “DC_3-7-8_n1-n78”, “DC_3-3-7-8_n1-n78”, “DC_3-7-7-8_n1-n78” and “DC_3-3-7-7-8_n1-n78” having the same component bands, they should be merged into one cell.

Table 6.3-1: Example for ΔTIB,c for Inter-band EN-DC configurations

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 or n8 | 0.6 |
|  | n1 | 0.6 |
|  | n78 | 0.8 |

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-8_n1-n78 DC_3-3-7-8_n1-n78 DC_3-7-7-8_n1-n78 DC_3-3-7-7-8_n1-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |
| DC_3-7_n1-n8-n78 DC_3-3-7_n1-n8-n78 DC_3-7-7_n1-n8-n78 DC_3-3-7-7_n1-n8-n78 | 0.6 | 0.6 | 0.6 | 0.6 | 0.8 |

Guideline 1: It is supposed that only the configurations having the same component E-UTRA / NR bands can be grouped into one cell (row) for the new ΔTIB,c and ΔRIB,c templates.

With regard to the values for a band combination in the ΔTIB,c / ΔRIB,c table, considering that a statement of ‘Unless otherwise stated, ΔTIB,c / ΔRIB,c is set to zero’ having been specified in the general part of specification, it is reasonable to remove the combination in the ΔTIB,c / ΔRIB,c table with all component band having the value of ‘-’ (zero). For example in the following ΔRIB,c Table 6.3-2, the CA combinations CA_n1-n3-n5, CA_n1-n3-n18 and CA_n1-n3-n20 do not need to be listed in the table.

Table 6.3-2: Example for ΔRIB,c for Inter-band CA configurations

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n3-n5 | - | - | - |
| CA_n1-n3-n8 | 0.2 | 0.2 | 0.5 |
| CA_n1-n3-n18 | - | - | - |
| CA_n1-n3-n20 | - | - | - |

Guideline 2: For the band combination with all the component bands having the ΔTIB,c / ΔRIB,c values as ‘-’ (zero), there is no need to be listed in the ΔTIB,c / ΔRIB,c table.

There are some special delta TIB and RIB values for the band combinations having SUL, SDL, immediately close component band, band combination with overlapping component band, and EN-DC combination with LTE LAA component band in the current specifications. However, the denotations for the delta TIB and RIB values for these special constituent band are inconsistent among different band combinations, some of which having the value of “-”, some of which having the value of “N/A”, while some of others having the value of non-zero number. To avoid inconsistencies, the following guideline is applied to the band combinations having special component band.

Guideline 3: For the component bands which are special bands such as SDL band, SUL band, immediately close band, band combination with overlapping band, and EN-DC combination with LTE LAA band, etc.

·  Non-zero value is not allowed for the special bands in the delta TIB/RIB tables.

·  If uplink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta T requirements for that constituted band of the band combination.

·  If downlink is not supported on a constituted band of the DC/CA band combination, "N/A" is used when deriving the delta R requirements for that constituted band of the band combination.

----- End of TP -----