3GPP TSG-RAN4 Meeting #108bis	R4-2316710

# Xiamen, China, October 9 – October 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on inter-frequency measurement without gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-09-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The draftCR provide inter-frequency measurement delay requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add scenario for UE report “nogap-nointerruption” and “nogap-interruption” at introduction at clause 9.3.1 Measurement requirements for “nogap-nointerruption” and “nogap-interruption” are defined at clause 9.3.9.1 and clause 9.3.9.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements on are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1, 9.3.9.1, 9.3.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of 1st change >>

## 9.3	NR inter-frequency measurements

### 9.3.1	Introduction

A measurement is defined as an SSB based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.2.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-frequency cells if carrier frequency information is provided by PCell or PSCell, even if no explicit neighbour list with physical layer cell identities is provided.

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP and its delay requirements are specified in clause 9.3.9, for UE capable of interFrequencyMeas-NoGap provided that

-	the UE supports interFrequencyMeas-Nogap-r16 [15], and

-	the SSB is completely contained in the active BWP of the UE.

Besides the conditions listed above, for UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement,

-	An inter-frequency SSB measurement is defined as measurement without gap if

-	the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and

-	the SSB is not completely contained in the active BWP of the UE

The delay requirements are specified in clause 9.3.9.

-	An inter-frequency SSB measurement is defined as measurement with NCSG if

-	the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and

-	the SSB is not completely contained in the active BWP of the UE

When network configures NCSG, the delay requirements are specified in clause 9.3.10.

When network configures measurement gap, the delay requirements are specified in clauses 9.3.4 and 9.3.5.

-	An inter-frequency SSB measurement is defined as measurement with gap if

-	the UE indicates ‘gap’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and

-	the SSB is not completely contained in the active BWP of the UE

When network configures measurement gap, the delay requirements are specified in clauses 9.3.4 and 9.3.5.

-	For inter-frequency SSB based measurements with NCSG, UE may cause scheduling restriction as specified in clause 9.3.10.3.

For UE supporting [NeedForGap-InfoNR-R18] for inter-frequency measurement,

-	An inter-frequency SSB measurement is defined as measurement without gap if

-	the UE indicates ‘[nogap-nointerruption]’ or [nogap-intrruption] via [NeedForGap-InfoNR-R18] for the inter-frequency measurement

-	UE is allowed to cause interruption during inter-frequency measurement without gap when UE indicate [nogap-interruption], the interruption requirement is defined in [clause 8.2.2.2.X]

The delay requirements are specified in clause 9.3.9.

For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.5.3.

Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

SSB based measurements are configured along with a measurement timing configuration (SMTC) per carrier, which provides periodicity, duration and offset information on a window of up to 5ms where the measurements on the configured inter-frequency carrier are to be performed. For inter-frequency connected mode measurements, one measurement window periodicity may be configured per inter-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB and measure RSSI of RSRQ on an inter-frequency measurement object which start earlier than the gap starting time + switching time, nor detect SSB and measure RSSI of RSRQ which ends later than the gap end – switching time. When the inter-frequency cells are in FR2 and the per-FR gap is configured to the UE in EN-DC, SA NR, NE-DC and NR-DC, or the serving cells are in FR2, the inter-frequency cells are in FR2 and the per-UE gap is configured to the UE in SA NR and NR-DC, the switching time is 0.25ms. Otherwise the switching time is 0.5ms.

The requirements in this clause shall also apply, when the UE is configured to perform SRS carrier based switching and using measurement gaps.

Longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

## << End of 1st change >>

## << Start of 2nd change >>

### 9.3.9	Inter frequency measurements without measurement gaps

#### 9.3.9.1	Inter frequency Cell identification

UE satisfying the applicability conditions specified in 9.3.1 on the requirement in this clause shall be able to identify a new detectable inter frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter frequency SS block of an already detected cell within Tidentify_inter_without_index.

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, it is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 9.3.9.1-1, table 9.3.9.1-1a, table 9.3.9.1-2, and table 9.3.9.1-2a.

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 9.3.9.1-3 and table 9.3.9.1-3a.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.9.2-1, table 9.3.9.2-2 and table 9.3.9.2-3 when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 9.3.9.2-1, table 9.3.9.2-1a, table 9.3.9.2-2, table 9.3.9.2-2a, and table 9.3.9.2-3, table 9.3.9.2-3a, and table 9.3.9.2-3b when highSpeedMeasInterFreq-r17 is configured and UE supports measurementEnhancementInterFreq-r17.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps or NCSG, i.e. when interfrequency SMTC is fully non overlapping or partially overlapping with measurement gaps or UE indicates [nogap-nointerruption] or when inter-frequency SMTC is fully non overlapping with measurement gaps for UE indicating [nogap-interruption], 
or it is determined according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when interfrequency SMTC is fully overlapping with measurement gaps, or according to CSSFwithin_ncsg,i in clause 9.1.5.x for measurement conducted within NCSG, i.e. when inter-frequency SMTC is fully overlapping with NCSG or when inter-frequency SMTC is fully or partially overlapping with measurement gaps for UE indicating [nogap-interruption].

For inter-frequency SSB based measurements without measurement gaps in active BWP [or the UE indicates [nogap-nointrruption] via [NeedForGap-InfoNR-R18]],

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync = 24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 36. For FR1, Mpss/sss_sync_inter = 5.

MSSB_index_inter: For a UE supporting FR2-2 power class 1, MSSB_index_inter = 72. For a UE supporting FR2-2 power class 2, MSSB_index_inter = 48. For a UE supporting FR2-2 power class 3, MSSB_index_inter = 48. For FR1, MSSB_index_inter = 3.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter = 40. For a vehicle mounted UE supporting FR2-1 power class 2, Mpss/sss_sync_inter=24. For a UE supporting FR2-1 power class 3, Mmeas_period_inter = 24. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 24. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 36. For FR1, Mmeas_period_inter = 5.

If the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement [or the UE indicates [nogap-intrruption] via [NeedForGap-InfoNR-R18]],

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60. For FR1, Mpss/sss_sync_inter = 8.

MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples. For FR1, MSSB_index_inter = 3.

Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60. For FR1, Mmeas_period_inter = 8.When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is a scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule provided that concurrent measurement gaps are configured.

Kp = 1 when Navailable = 0.

Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When interfrequency SMTC is fully non overlapping with measurement gaps or NCSG, or interfrequency SMTC is fully overlapping with MGs or NCSG, Kp =1.

When interfrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by inter-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by inter-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Table 9.3.9.1-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil(Mpss/sss_sync_inter x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2x Mpss/sss_sync_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-1a: Time period for PSS/SSS detection, when UE indicate [nogap-interruption] (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil(Mpss/sss_sync_inter x Kp) x max(80ms, SMTC period) )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 x Mpss/sss_sync_inter x Kp) x max(80ms, SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-2: Time period for PSS/SSS detection, (FR2)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void |  |

Table 9.3.9.1-2a: Time period for PSS/SSS detection, when UE indicate [nogap-interruption] (FR2)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_inter x Kp x Klayer1_measurement) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void |  |

Table 9.3.9.1-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil(MSSB_index_inter x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(MSSB_index_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-3a: Time period for time index detection, when UE indicate [nogap-interruption] (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil(MSSB_index_inter x Kp ) x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter x Kp) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(MSSB_index_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 9.3.9.1-4: Time period for time index detection (FR2-2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(MSSB_index_inter x Kp) SMTC period) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(1.5 MSSB_index_inter x Kp) Max(SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(MSSB_index_inter x Kp) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp is applicable for UE supporting [concurrent gaps] |  |

Table 9.3.9.1-4a: Time period for time index detection, when UE indicate [nogap-interruption] (FR2-2)

| Condition NOTE1,2 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(200ms, Ceil(MSSB_index_inter x Kp) max(80ms, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(1.5 MSSB_index_inter x Kp) Max(80ms, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(MSSB_index_inter x Kp) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: Kp is applicable for UE supporting [concurrent gaps] |  |

#### 9.3.9.2	Measurement period

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 9.3.9.2-1 and 9.3.9.2-2, if UE supports inter-frequency measurement without measurement gaps. When highSpeedMeasInterFreq-r17 is configured and UE supports [measurementEnhancementInterFreq-r17], T SSB_measurement_period_inter is specified in table 9.3.9.2-3.

Table 9.3.9.2-1: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil(Mmeas_period_inter x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x Mmeas_period_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( Mmeas_period_inter x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.9.2-1a: Measurement period for inter-frequency measurements without gaps when UE indicate [nogap-interruption] (FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil(Mmeas_period_inter x Kp) x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x Mmeas_period_inter x Kp) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( Mmeas_period_inter x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.9.2-2: Measurement period for inter-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.9.2-2a: Measurement period for inter-frequency measurements without gaps when UE indicate [nogap-interruption (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.3.9.2-3: Measurement period for inter-frequency measurements without gaps in the active BWP when highSpeedMeasInterFreq-r17 is configured (FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period, DRX cycle)) x CSSFinter |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle) x CSSFinter |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC period > 40 ms, otherwise M2 = 1 NOTE 3: Y=3 when SMTC period <= 40ms, Y=5 when SMTC period > 40ms |  |

Table 9.3.9.2-3a: Measurement period for inter-frequency measurements without gaps when highSpeedMeasInterFreq-r17 is configured (FR1), UE supporting ‘nogap-noncsg’

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, 7 Max(MGRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 160ms | max(200ms, ceil(7 x M2 NOTE3) x max(MGRP, SMTC period, DRX cycle)) x CSSFinter |
| 160ms < DRX cycle ≤ 320ms | ceil(7 x M2 NOTE3) x DRX cycle x CSSFinter |
| DRX cycle>320ms | 4 x M2 NOTE3 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

Table 9.3.9.2-3b: Measurement period for inter-frequency measurements without gaps when highSpeedMeasInterFreq-r17 is configured (FR1), when UE indicate [nogap-interruption]

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, 7 Max(80ms, SMTC period)) CSSFinter |
| DRX cycle ≤ 160ms | max(200ms, ceil(7 x M2 NOTE3) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| 160ms < DRX cycle ≤ 320ms | ceil(7 x M2 NOTE3) x DRX cycle x CSSFinter |
| DRX cycle>320ms | 4 x M2 NOTE3 x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 |  |

## << End of 2nd change >>