3GPP TSG-RAN WG4 Meeting # 108bis	R4-2316735

Xiamen, China, October 09 – October 13, 2023

Source: 	Qualcomm Incorporated

Title: 	UE Demod for ATG UE Demod

Agenda item:	5.13.7.2

Document for:	Approval

- Introduction

In this paper, we share our considerations regarding the open items for UE Demodulation Requirements for ATG UE, based on the WF approved in the last RAN4 meeting [1].

Alignment simulation results based on the agreed simulation assumptions are included in our companion paper R4-2316731.

# Legacy PDSCH Requirements Scope

In WF [1] there is no agreement related to the scope of legacy PDSCH requirements for ATG UE Demod. To that end, to reduce the burden of the testing load, we support selecting a limited set of legacy tests, and we support the tentative WF from the original topic summary.

Proposal 1: RAN4 to choose the following legacy PDSCH requirements as applicable to ATG UEs. No additional PDSCH tests should be considered;

- Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

- Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

- Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

- Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

# Legacy PDCCH Requirements Scope

In WF[1] there are still a number of options open for the scope of legacy PDCCH requirements and the rationale behind the proposal is unclear. Similar to our previous proposal regarding the applicability of legacy PDSCH requirement, we propose to consider for FDD and TDD only 1 test per each 1TX and 2TX Antenna table, providing a mix of coverage for various AL.

Proposal 2: RAN4 to choose 1 test per each 1TX and 2TX configuration, with different AL to provide sufficient coverage, for 2 and 4 RX separately. The tables for are:

- 2RX: FDD (5.3.2.1.[1/2]), TDD (5.3.2.2.[1/2]);

- 4RX: FDD (5.3.3.1.[1/2]), TDD (5.3.3.2.[1/2]);

Proposal 3: RAN4 to reuse legacy applicability rule for PDCCH requirements based on the number of RX, according to which only 4RX requirements are applicable if UE supports both 2 and 4 RX;

# PMI Reporting requirements

Proposal 4: RAN4 not to further consider PMI reporting requirements in the scope of ATG UE Demod;

- Conclusions

Proposal 1: RAN4 to choose the following legacy PDSCH requirements as applicable to ATG UEs. No additional PDSCH tests should be considered;

- Test num 1-3 and 1-4 in Table 5.2.2.1.1-3 (2R FDD)

- Test num 1-3 and 1-4 in Table 5.2.2.2.1-3 (2R TDD)

- Test num 1-3 and 1-4 in Table 5.2.3.1.1-3 (4R FDD)

- Test num 1-3 and 1-4 in Table 5.2.3.2.1-3 (4R TDD)

Proposal 2: RAN4 to choose 1 test per each 1TX and 2TX configuration, with different AL to provide sufficient coverage, for 2 and 4 RX separately. The tables for are:

- 2RX: FDD (5.3.2.1.[1/2]), TDD (5.3.2.2.[1/2]);

- 4RX: FDD (5.3.3.1.[1/2]), TDD (5.3.3.2.[1/2]);

Proposal 3: RAN4 to reuse legacy applicability rule for PDCCH requirements based on the number of RX, according to which only 4RX requirements are applicable if UE supports both 2 and 4 RX;

Proposal 4: RAN4 not to further consider PMI reporting requirements in the scope of ATG UE Demod;

- References

[1] R4-2313875, “WF for ATG demodulation requirements”, 3GPP TSG-RAN WG4 Meeting # 108, Toulouse, FR, August 21 – 25, 2023