3GPP TSG-RAN WG4 Meeting #108bis	R4-2316755

# Xiamen, China, October 9 – 13, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for DL RSCPD

Agenda item:	5.22.2.6

Document for:	Information

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including:

- Support of NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

NR carrier phase positioning (NR CPP) has been studied in the FS_NR_pos_enh2 study item by RAN1 with company contributions and outcome of the study being captured in TR 38.859 [2].  Based on Rel-16 and Rel-17 link level simulation assumptions for RSTD and PRS-RSRP, respectively, in [3] and [4], RAN4 has agreed NR CPP link level simulation assumptions for RSCPD in [5].

In this contribution we list simulation assumptions and then provide revised simulation results for DL RSCPD. It is noted that these results do not contain any RF/BB impairments. However, great potential of this positioning technology in terms of improved positioning accuracy can be seen.

Simulation Assumptions

Simulation assumptions for DL RSCPD captured in [5] are considered. The PRS configurations as listed in Table 1 were used in the simulation study. Note that the repetition factor relates to repetitions in adjacent slots.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | Sample rate (Tc) |
|  | 15 | 24 | 4 | 256 |
|  |  | 52 | 1 | 128 |
|  |  | 104 | 1 | 64 |
|  | 30 | 24 | 4 | 128 |
|  |  | 48 | 1 | 64 |
|  | 60, FR1 | 24 | 4 | 64 |
|  | 60, FR2 | 24 | 4 | 64 |
|  |  | 64 | 1 | 32 |
|  |  | 132 | 1 | 16 |
|  | 120 | 32 | 4 | 32 |
|  |  | 64 | 1 | 16 |
| PRS comb size | 4 |  |  |  |
| PRS symbol size | 4 |  |  |  |
| Number of samples | 1, 4 |  |  |  |
| PRS periodicity | 40 ms |  |  |  |
| NOTE 1: Repetition 4 is achieved by PRS in 4 consecutive slots with PRS-ResourceTimeGap=1. NOTE 2: Companies are encouraged to check suitability of the parameters and the values. |  |  |  |  |

Table 1: PRS configurations used for NR RSCPD study for SCS = 15kHz, 30kHz, 60kHz (FR1 and FR2), and 120 kHz.

As an additional note on the simulation assumptions, we focused on AWGN channel, and no RF/BB impairments are included. However, in the next contributions we plan to include simulation results using the two-tap channel and including RF/BB impairments.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the DL RSCPD error in degree for given Es/Iot for three cells [5].

FR1, SCS = 15kHz

The DL RSCPD error in degree for AWGN channel with SCS set to 15kHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB, respective, are listed in Table 2 and Table 3.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -6.5039 | 6.9018 | -6.6215 | 6.7083 |
|  |  |  |  | 4 | 4 | -3.1702 | 3.4064 | -3.4011 | 3.2462 |
|  |  | 52 | 128 | 1 | 1 | -10.9066 | 10.5215 | -10.2805 | 10.0292 |
|  |  |  |  | 1 | 4 | -5.3284 | 5.3493 | -5.2706 | 5.371 |
|  |  | 104 | 64 | 1 | 1 | -7.1177 | 7.2165 | -7.6338 | 7.4459 |
|  |  |  |  | 1 | 4 | -3.848 | 3.4137 | -3.4898 | 3.8946 |
|  | 4 | 24 | 256 | 4 | 1 | -6.9538 | 6.7731 | -6.8605 | 6.4308 |
|  |  |  |  | 4 | 4 | -3.1046 | 3.249 | -3.4129 | 3.4001 |
|  |  | 52 | 128 | 1 | 1 | -10.932 | 10.3939 | -9.9634 | 10.6902 |
|  |  |  |  | 1 | 4 | -5.2023 | 5.7741 | -5.1356 | 4.7041 |
|  |  | 104 | 64 | 1 | 1 | -7.3397 | 7.8895 | -7.8521 | 7.1818 |
|  |  |  |  | 1 | 4 | -3.7767 | 3.812 | -3.8232 | 3.6172 |

Table 2: DL RSCPD error for AWGN channel in degree for SCS = 15kHz. Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 24 | 256 | 4 | 1 | -3.2057 | 3.3885 | -3.4239 | 3.3723 |
|  |  |  |  | 4 | 4 | -1.6443 | 1.6998 | -1.7839 | 1.66 |
|  |  | 52 | 128 | 1 | 1 | -5.612 | 5.2553 | -5.2098 | 5.0327 |
|  |  |  |  | 1 | 4 | -2.7531 | 2.6694 | -2.6536 | 2.686 |
|  |  | 104 | 64 | 1 | 1 | -3.6009 | 3.5722 | -3.8295 | 3.7975 |
|  |  |  |  | 1 | 4 | -1.8766 | 1.7037 | -1.7601 | 1.9542 |
|  | 4 | 24 | 256 | 4 | 1 | -3.5475 | 3.3421 | -3.5367 | 3.395 |
|  |  |  |  | 4 | 4 | -1.5649 | 1.6051 | -1.7156 | 1.6728 |
|  |  | 52 | 128 | 1 | 1 | -5.3873 | 5.1885 | -5.0795 | 5.3701 |
|  |  |  |  | 1 | 4 | -2.614 | 2.8839 | -2.6879 | 2.4075 |
|  |  | 104 | 64 | 1 | 1 | -3.6875 | 3.9411 | -3.8859 | 3.611 |
|  |  |  |  | 1 | 4 | -1.8445 | 1.9545 | -1.9314 | 1.7574 |

Table 3. DL RSCPD error for AWGN channel in degree for SCS = 15kHz. Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

FR1, SCS = 30 kHz

The DL RSCPD error in degree for AWGN channel with SCS set to 30kHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB, respective, are listed in Table 4 and Table 5.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 30 | 2 | 24 | 128 | 4 | 1 | -6.6756 | 6.8632 | -6.9765 | 6.8815 |
|  |  |  |  | 4 | 4 | -3.4131 | 3.1816 | -3.3588 | 3.3825 |
|  |  | 52 | 64 | 1 | 1 | -10.5091 | 10.1851 | -10.4756 | 10.2386 |
|  |  |  |  | 1 | 4 | -5.2764 | 5.1353 | -5.5155 | 5.6665 |
|  | 4 | 24 | 128 | 4 | 1 | -6.6329 | 6.7307 | -6.9285 | 6.2093 |
|  |  |  |  | 4 | 4 | -3.6128 | 3.2721 | -3.6529 | 3.0515 |
|  |  | 52 | 64 | 1 | 1 | -10.3583 | 10.7014 | -10.8463 | 10.4353 |
|  |  |  |  | 1 | 4 | -5.1847 | 5.1297 | -5.3033 | 5.1623 |

Table 4: DL RSCPD error for AWGN channel in degree for SCS = 30kHz. Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 30 | 2 | 24 | 128 | 4 | 1 | -3.3634 | 3.4361 | -3.4953 | 3.3102 |
|  |  |  |  | 4 | 4 | -1.6573 | 1.586 | -1.7201 | 1.5824 |
|  |  | 52 | 64 | 1 | 1 | -5.3127 | 5.0536 | -5.1211 | 5.2211 |
|  |  |  |  | 1 | 4 | -2.606 | 2.4727 | -2.6313 | 2.8341 |
|  | 4 | 24 | 128 | 4 | 1 | -3.3317 | 3.3129 | -3.3893 | 3.1511 |
|  |  |  |  | 4 | 4 | -1.7698 | 1.6296 | -1.8025 | 1.5639 |
|  |  | 52 | 64 | 1 | 1 | -5.2012 | 5.2115 | -5.3325 | 5.1762 |
|  |  |  |  | 1 | 4 | -2.6038 | 2.5184 | -2.6545 | 2.5469 |

Table 5: DL RSCPD error for AWGN channel in degree for SCS = 30kHz. Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

FR1, SCS = 60 kHz

The DL RSCPD error in degree for AWGN channel with SCS set to 60kHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB, respective, are listed in Table 6 and Table 7.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 2 | 24 | 64 | 4 | 1 | -6.9381 | 6.8577 | -6.8323 | 6.292 |
|  |  |  |  | 4 | 4 | -3.096 | 3.2519 | -3.3217 | 3.3356 |
|  | 4 | 24 | 64 | 4 | 1 | -6.727 | 6.6945 | -6.8026 | 6.5479 |
|  |  |  |  | 4 | 4 | -3.3113 | 3.5173 | -3.3953 | 3.434 |

Table 6: DL RSCPD error for AWGN channel in degree for SCS = 60kHz. Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 2 | 24 | 64 | 4 | 1 | -3.5507 | 3.3397 | -3.5967 | 3.3267 |
|  |  |  |  | 4 | 4 | -1.5528 | 1.6287 | -1.6751 | 1.6486 |
|  | 4 | 24 | 64 | 4 | 1 | -3.5218 | 3.3115 | -3.4288 | 3.344 |
|  |  |  |  | 4 | 4 | -1.6401 | 1.7148 | -1.7013 | 1.7017 |

Table 7: DL RSCPD error for AWGN channel in degree for SCS = 60kHz. Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

FR2, SCS = 60 kHz

The DL RSCPD error in degree for AWGN channel with SCS set to 60kHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB, respective, are listed in Table 8 and Table 9.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 24 | 64 | 4 | 1 | -6.727 | 6.6945 | -6.8026 | 6.5479 |
|  |  |  |  | 4 | 4 | -3.3113 | 3.5173 | -3.3953 | 3.434 |
|  |  | 64 | 32 | 1 | 1 | -10.3892 | 10.0811 | -10.1826 | 9.5335 |
|  |  |  |  | 1 | 4 | -5.0121 | 4.7655 | -4.9435 | 4.5897 |
|  |  | 132 | 16 | 1 | 1 | -7.0447 | 7.437 | -7.056 | 7.3041 |
|  |  |  |  | 1 | 4 | -3.4473 | 3.7419 | -3.6629 | 3.7755 |

Table 8. DL RSCPD error for AWGN channel in degree for SCS = 60kHz. Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 24 | 64 | 4 | 1 | -3.5218 | 3.3115 | -3.4288 | 3.344 |
|  |  |  |  | 4 | 4 | -1.6401 | 1.7148 | -1.7013 | 1.7017 |
|  |  | 64 | 32 | 1 | 1 | -5.0517 | 4.9564 | -5.089 | 5.0229 |
|  |  |  |  | 1 | 4 | -2.3939 | 2.3793 | -2.3483 | 2.3851 |
|  |  | 132 | 16 | 1 | 1 | -3.5684 | 3.7543 | -3.5208 | 3.6913 |
|  |  |  |  | 1 | 4 | -1.7724 | 1.9339 | -1.8239 | 1.8464 |

Table 9: DL RSCPD error for AWGN channel in degree for SCS = 60kHz. Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

FR1, SCS = 120 kHz

The DL RSCPD error in degree for AWGN channel with SCS set to 120kHz, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells (-6, -13, -13) dB and (-3, -6, -6) dB, respective, are listed in Table 10 and Table 11.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 120 | 2 | 32 | 32 | 4 | 1 | -6.3099 | 6.1645 | -5.9209 | 6.3781 |
|  |  |  |  | 4 | 4 | -3.1569 | 3.0365 | -2.941 | 3.185 |
|  |  | 64 | 16 | 1 | 1 | -10.2138 | 10.1725 | -10.5072 | 10.2518 |
|  |  |  |  | 1 | 4 | -4.8494 | 4.7123 | -5.4578 | 4.9644 |
|  | 4 | 32 | 32 | 4 | 1 | -6.9274 | 6.1281 | -6.2839 | 6.5227 |
|  |  |  |  | 4 | 4 | -3.3695 | 3.1788 | -3.213 | 3.2335 |
|  |  | 64 | 16 | 1 | 1 | -10.315 | 10.1114 | -10.1496 | 10.6489 |
|  |  |  |  | 1 | 4 | -4.7868 | 5.0169 | -5.4526 | 5.3087 |

Table 10: DL RSCPD error for AWGN channel in degree for SCS = 120kHz. Es/Iot (cell-1, cell-2, cell-3) = (-6, -13, -13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB | Sampling rate (Tc) | Rep | Number of samples | RSCPD (cell-1, cell-2) (-6, -13) dB |  | RSCPD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 120 | 2 | 32 | 32 | 4 | 1 | -3.1103 | 3.1126 | -3.0142 | 3.1767 |
|  |  |  |  | 4 | 4 | -1.5756 | 1.5149 | -1.5195 | 1.5803 |
|  |  | 64 | 16 | 1 | 1 | -5.1133 | 5.1154 | -5.2902 | 5.2132 |
|  |  |  |  | 1 | 4 | -2.4764 | 2.4212 | -2.8063 | 2.5483 |
|  | 4 | 32 | 32 | 4 | 1 | -3.4836 | 3.1582 | -3.2305 | 3.1899 |
|  |  |  |  | 4 | 4 | -1.679 | 1.603 | -1.6171 | 1.5733 |
|  |  | 64 | 16 | 1 | 1 | -5.1272 | 5.0859 | -5.2153 | 5.4083 |
|  |  |  |  | 1 | 4 | -2.3628 | 2.5209 | -2.8003 | 2.6249 |

Table 11. DL RSCPD error for AWGN channel in degree for SCS = 120kHz. Es/Iot (cell-1, cell-2, cell-3) = (-3, -6, -6) dB.

Discussion

From the simulation results reported above for the AWGN channel without RF/BB impairments, we observed:

- The RSCPD error for all PRS BW with same (PRS repetition factor, number of samples) is not significantly deviated. Thus, the carrier phase accuracy dependency on the bandwidth for AWGN channel is not so significant.

- For a given bandwidth, the measurement accuracy improves with the increasing in number of measurement samples or the PRS repetition factor. For instance, when comparing carrier phase accuracy for 1 measurement sample against 4 measurement samples, the RSCPD error in the 90 percentile is roughly halved. Thus, a use of larger BW or smaller BW with increased PRS repetitions/measurement samples could be the trait depending on the latency requirement.

- For a given bandwidth, the measurement accuracy does not significantly deviate for different carrier frequencies. Thus, the carrier phase accuracy dependency on the carrier frequency for AWGN channel is not so significant.

Thus, following observations are made:

The carrier phase accuracy dependency on bandwidth is not so significant for AWGN channel.

The use of larger BW or smaller BW with increased PRS repetitions/measurements samples could be the trait depending on the latency requirement. Four measurement samples yield significant lower RSCPD error than one measurement sample.

The carrier phase accuracy dependency on carrier frequency is not so significant for AWGN channel.

Conclusion

In this contribution we have provided first simulation results for DL RSCPD error without RF/BB impairments based on simulation assumptions in [5].

Following observations are made.

- The carrier phase accuracy dependency on bandwidth is not so significant for AWGN channel.

The use of larger BW or smaller BW with increased PRS repetitions/measurements samples could be the trait depending on the latency requirement. Four measurement samples yield significant lower RSCPD error than one measurement sample.

The carrier phase accuracy dependency on carrier frequency is not so significant for AWGN channel.

- References

| [1] | RP-232670, "Revised WID on Expanded and Improved NR positioning", 3GPP TSG RAN#101 |
| --- | --- |
| [2] | TR 38.859, "Study on Expanded and Improved NR Positioning (Release 18)," V18.0.0 |
| [3] | R4-2002287, "Updated link level simulation assumption for RSTD and RSRP," Huawei |
| [4] | R4-2211061, "Simulation assumption for PRS-RSRPP," Huawei |
| [5] | R4-2310078, "Simulation assumptions for CPP measurement," CATT |