3GPP TSG-RAN WG4 Meeting # 108-bis									          R4-2316768

Xiamen, China, 9th October – 13th October, 2023

Title: 	Uplink CA_n5B BCS1 MSD

Agenda Item:	4.1.1.1

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Introduction

In RAN#100, a new BCS1 was requested for UL CA_n5B with a maximum 25 MHz aggregated uplink (UL) bandwidth (BW). This BCS1 request was initially analysed in RAN4 #100-e [1], and at meeting #108 in [2] and [3]. This contribution proposes MSD requirements based on power amplifier measurements of [1] for the test point agreed upon in Way Forward (WF) [4].

# Discussion

## 2.1 Power Amplifier Calibration

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR- PC3 ACLR -30 dBc

- Post PA losses: 4 dB

- Power Class 3 (PC3) operation

- Local Oscillator (LO) leakage: -28 dBc

- IQ Image rejection: -28 dB

## 2.2 Measurement Results and MSD Proposal

The Band n5 PCC/SCC MSD is evaluated using the following parameters for the test point agreed in WF [4]:

- PC3 operation, Tx power level: 23 dBm

- Tx-Rx duplexer isolation: 50 dB

- Tx-antenna duplex rejection in Rx range: 45 dB

- Post-PA insertion losses: 4 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1dB Signal-to-Noise Ratio (SNR)

The measured noise is -0.1 dBm/MBW and -9.1 dBm/MBW in the Band n5 PCC 5 MHz and SCC 20 MHz CBW, respectively.

Proposal: Adopt the MSD test point specific to UL CA_n5B BCS1 highlighted in green in Table 1 below. A new footnote is added to distinguish the test point applicable to BCS1 vs that previously agreed upon for BCS0.

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 26.2 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# Conclusion

In this contribution, we have presented measurements and calculations for CA_n5B intra-band UL CA MSDs, and make the following proposal.

Proposal: Adopt the MSD test point specific to UL CA_n5B BCS1 highlighted in green in Table 1 below. A new footnote is added to distinguish the test point applicable to BCS1 vs that previously agreed upon for BCS0.

Table 1 Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBSTART = 0) | 10 (RBSTART = 42) | 30.8 | 26.1 | FDD |
| CA_n5BX | 15/15 | 5MHz + 20MHz | 4 (RBSTART = 0) | 16 (RBSTART = 90) | 46.4 | 26.2 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 1. |  |  |  |  |  |  |  |

# References

- R4-2114579 CA_n5B MSD, 3GPP TSG-RAN WG4 Meeting # 100e Electronic meeting, Skyworks Solutions Inc.

- R4-2313380 MSD for UL CA_n5B, 3GPP TSG-RAN WG4 #108 Toulouse, France, Qualcomm France.

- R4-2311260 CA_n5B ULCA MSD for BCS 1 and 2, 3GPP TSG-RAN WG4 #108 Toulouse, France, Skyworks Solutions Inc.

- R4-2314855 WF on CA_n5B DL/UL BCS1 MSD test point, 3GPP TSG-RAN WG4 #108 Toulouse, France, Skyworks Solutions Inc.