3GPP TSG-RAN WG4 Meeting # 108-bis									         R4-2316775

Xiamen, China, 9th October – 13th October, 2023

Title: 	Correction to SUL_n41-n80 and guidelines for SUL MSD test points

Agenda Item:	5.1.3

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Introduction

At RAN4 meeting #108, we proposed a Change Request (CR) to cross-align the MSD test points due to cross-band isolation of several Supplementary Uplink (SUL) combinations with their NR-CA counter parts [1]. Following the meeting, it was discovered that the uplink configuration for SUL_n41-n80 was erroneous due to the SUL in Band n80 and its NR counterpart Band n3 of 50 MHz in CA_n3-n41. To prevent additional errors, we propose an updated set of guidelines when a 1:1 correspondence is not met between a SUL combination and its NR CA counterpart.

# Discussion

## 2.1 Background

At meeting #108, the CR [1] modified the SUL_n41-n80 MSD from the test points of Table 1 to the test points of Table 2.

Table 1: Rel 18.2.0 SUL_n41-n80 cross-band isolation MSD test point.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1780 | 10 | 15 | 50 (RBstart=0) | 2505 | 10 | 4.3 | >ACLR2 |
| n80 | n41 | 1780 | 10 | 15 | 50 (RBstart=0) | 2550 | 100 | 3.0 | >ACLR2 |

Table 2: Rel 18.3.0 SUL_n41-n80 updated cross-band isolation MSD test point.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2501 | 10 | 0.7 | >ACLR2 |
| n80 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2546 | 100 | 0.7 | >ACLR2 |

The objective of these changes is to align the MSD test point for SUL_n41-n80 with the MSD test points of its NR-CA counterpart, namely CA_n3-n41, as shown in Table 3.

Table 3: CA_n3-n41 PC3 cross-band isolation MSD test point

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2501 | 10 | 0.7 | >ACLR2 |
| n3 | n41 | 1760 | 50 | 15 | 50 (RBstart=220) | 2546 | 100 | 0.7 | >ACLR2 |

However, contrary to its NR counterpart, the maximum supported CBW for the SUL Band n80 in SUL_n41-n80 is 40 MHz (Table 5), while the Band n3 maximum supported CBW in CA_n3-n41 is 50 MHz for BCS 3, 4 and 5 (Table 4).

Table 4: n3 maximum CBW for CA_n3-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418,9 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 3 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | See n41 channel bandwidths in Table 5.3.5-1 |  |

Table 5: n80 maximum CBW for SUL_n41-n80

| SUL configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- |
| SUL_n41A-n80A | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  | n80 | 5, 10, 15, 20, 25, 30 |  |
|  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 1 |
|  | n80 | 5, 10, 15, 20, 25, 30, 40 |  |

Therefore, the SUL_n41-n80 MSD test point is erroneous and must be corrected at meeting #109.

To prevent additional errors when a full 1:1 correspondence is not met between the SUL combination and its NR counterpart, we propose a set of guidelines to capture the SUL MSD test points due to cross-band isolation.

## 2.2 Guidelines for SUL MSD test points due to cross-band isolation.

We only propose to update the guidelines on the UL band configurations. Guidelines for DL band configurations, and guidelines for cross-band interference source and MSD evaluations, remain identical to the NR-CA guidelines [2] – see Annex.

Proposal 1: Adopt the following guidelines for the SUL configuration for cross-band isolation MSD test points:

- For SUL band combinations, and for the first test point which evaluates the MSD for the lowest DL CBW, the SUL band should be configured with the highest supported CBW, as specified in Table 5.5C-1. This ensures that the SUL band lowest IMD order has a maximum reach towards the DL affected band.

- For the second test point, the choice of the SUL CBW remains open to account for exceptions or regional concerns, or to address a proponent’s request.

- The SUL SCS should be the lowest SCS that can be supported for the selected SUL CBW. For example, if the SUL CBW is 50 MHz, then SCS15 kHz should be specified.

- For the UL configuration "Lcrb" for the SUL band: The UL Lcrb of the NR band counterpart as defined in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the corresponding SUL band CBW is specified. A SUL-NR counterpart look-up is provided in Table 6.

Table 6: n80 maximum CBW for SUL_n41-n80

| SUL band | NR UL Band counterpart | FUL_low – FUL_high (MHz) |
| --- | --- | --- |
| n80 | n3 | 1710 – 1785 |
| n81 | n8 | 880 – 915 |
| n82 | n20 | 832 – 862 |
| n83 | n28 | 703 – 748 |
| n84 | n1 | 1920 – 1980 |
| n86 | n66 | 1710 – 1780 |
| n89 | n5 | 824 – 849 |
| n95 | n34 | 2010 – 2025 |
| n97 | n40 | 2300 – 2400 |
| n98 | n39 | 1880 – 1920 |
| n99 | n24 | 1626.5 – 1660.5 |

- The SUL RBstart should ensure that the UL RBs are positioned closest to the DL affected band.

- The SUL carrier center frequency should be configured closest to the affected DL band.

## 2.3 SUL_n41-n80 correction

Based on proposal 1, the corrected MSD test point for SUL_n41-n80 is captured in proposal 2:

- The UL BW is specified to 40 MHz, since this is the highest CBW specified for SUL n80 in SUL-n41-n80 Table 5.5C-1

- The lowest SCS for 40 MHz CBW is SCS 15 kHz

- the SUL Lcrb is copied from the NR counterpart Band n3 UL Lcrb specified in Table 7.3.2-3 for SCS 15 kHz for 40 MHz uplink CBW, ie. Lcrb=40RBs

Since the DL affected band is higher in frequency than the SUL band:

- The RBstart is specified to its highest value

- The SUL UL Fc is specified to its highest carrier frequency

Proposal 2: Adopt the corrected SUL_n41-n80 MSD test point of Table 7

Table 7: Corrected SUL_n41-n80 updated cross-band isolation MSD test point.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart = 166) | 2501 | 10 | 0.7 | >ACLR2 |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart = 166) | 2546 | 100 | 0.7 | >ACLR2 |

# Conclusion

In this contribution, we propose an updated set of guidelines to specify the SUL configuration for MSD due to cross-band isolation. These guidelines are used to correct the SUL_n41-n80 MSD test point agreed upon in [2].

Proposal 1: Adopt the following guidelines for the SUL configuration for cross-band isolation MSD test points:

- For SUL band combinations, and for the first test point which evaluates the MSD for the lowest DL CBW, the SUL band should be configured with the highest supported CBW, as specified in Table 5.5C-1. This ensures that the SUL band lowest IMD order has a maximum reach towards the DL affected band.

- For the second test point, the choice of the SUL CBW remains open to account for exceptions or regional concerns, or to address a proponent’s request.

- The SUL SCS should be the lowest SCS that can be supported for the selected SUL CBW. For example, if the SUL CBW is 50 MHz, then SCS15 kHz should be specified.

- For the UL configuration "Lcrb" for the SUL band: The UL Lcrb of the NR band counterpart as defined in Table 7.3.2-3 (UL configuration for UL Band REFSENS) for the corresponding SUL band CBW is specified. A SUL-NR counterpart look-up is provided in Table 6.

Table 6: n80 maximum CBW for SUL_n41-n80

| SUL band | NR UL Band counterpart | FUL_low – FUL_high (MHz) |
| --- | --- | --- |
| n80 | n3 | 1710 – 1785 |
| n81 | n8 | 880 – 915 |
| n82 | n20 | 832 – 862 |
| n83 | n28 | 703 – 748 |
| n84 | n1 | 1920 – 1980 |
| n86 | n66 | 1710 – 1780 |
| n89 | n5 | 824 – 849 |
| n95 | n34 | 2010 – 2025 |
| n97 | n40 | 2300 – 2400 |
| n98 | n39 | 1880 – 1920 |
| n99 | n24 | 1626.5 – 1660.5 |

- The SUL RBstart should ensure that the UL RBs are positioned closest to the DL affected band.

- The SUL carrier center frequency should be configured closest to the affected DL band.

Proposal 2: Adopt the corrected SUL_n41-n80 MSD test point of Table 7

Table 7: Corrected SUL_n41-n80 updated cross-band isolation MSD test point.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | X band interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart = 166) | 2501 | 10 | 0.7 | >ACLR2 |
| n80 | n41 | 1765 | 40 | 15 | 50 (RBstart = 166) | 2546 | 100 | 0.7 | >ACLR2 |

# References

[1]	R4-2311129 CR to TS 38.101-1 Rel-18 SUL MSD alignment, 3GPP TSG-RAN WG4 Meeting #108, Toulouse, France, Skyworks Solutions, Inc.

[2]	R4-2210565 WF on criteria on Rel-17 enhanced MSD table format, 3GPP TSG-RAN WG4 Meeting #103e, Electronic Meeting, Skyworks Solutions, Inc.

# Annex: NR-CA Guidelines for MSD Test Points due to Cross-band isolation [2]

P8: Guidelines for the DL band configuration:

- The affected DL band carrier centre-frequency shall be configured closest to the UL band.

- Up to two test points can be specified [1]:

  - the first test point should capture the lowest DL channel bandwidth (CBW) MSD [2],

  - the second test point is optional and may capture the highest DL channel bandwidth (CBW) MSD.

- It is not precluded that the second test point is specified for other DL CBW or DL carrier frequency to accommodate exceptions due to say, regional spectrum allocations specificities, or proponent concerns on specific CBW of interest etc.. For example, for CA_n18-n28, both the 1st and the 2nd MSD test point may be specified for the smallest n28 DL CBW of 5MHz, but it is currently discussed that:

  - the first MSD test point may specify the MSD for the n28B DL frequency range,

  - while the second MSD test point may specify the MSD for the n28A frequency range.

- The “lowest” and the “highest” CBW shall not exceed the CBW specified in the BCS configuration tables of clause 5.5A.

P10: Guidelines for cross-band interference source and MSD evaluation

- The interference source can be either “ACLR1”, “ACLR2”, or greater than ACLR2 (“>ACLR2”),

- For “ACLR1”, or “ACLR2” test points:

  - MSD may be neglected if the order of the IMD product of the modulated RB and its image that overlaps the affected DL band is greater than [9],

  - For RB/Image IMD order less or equal to [7], the MSD needs to be analysed taking into consideration RF-FE complexity (quadplexer, triplexer, etc..),

- For “>ACLR2” test points, aka “flat-band noise cases” / large separation distances:

  - MSD may need to be evaluated if the Tx noise rejection in the DL affected band is less than [40] dB. The underlying assumption is that the flat Tx noise PSD is -130dBm/Hz and DL affected CBW is 5MHz.