3GPP TSG-RAN WG4 Meeting # 108-bis									          R4-2316777

Xiamen, China, 9th October – 13th October, 2023

Title: 	PC5 NR-U CA_n96B A-MPR for NS_53 NS_54

Agenda Item:	4.1.1.2

Source: 	Skyworks Solutions, Inc.,

Document for:	Approval

# 1	Introduction

During the RAN4 meeting #108, Power Class 5 (PC5) A-MPR for uplink (UL) carrier aggregation (CA) class B in Band n96 (also known as CA_n96B), was evaluated using simulation results in [1, 2]. In this contribution, we present our perspective based on both measurements and results. Due to lack of time, we are unable to analyse the whole dataset prior to deadline. Data may be presented prior to meeting start if the request to revise this document is accepted by the RAN4 chairman.

# Discussion

## 2.1 Background

With regards to the NS_53 A-MPR levels: for aggregated bandwidth of up to 100 MHz, we observe similar A-MPR performance between [1] vs [2]. Evaluations for 120, 140 and 160 MHz are absent in [2]. Our conclusions are as follows:

- For cases where the A-MPR is dominated by the NS_53 in-band PSD requirement of -1 dBm/MHz, we propose that the PC5 A-MPR for UL CA be identical to that of the PC5 single UL carrier requirement.

- For cases where the A-MPR is driven by the NS_53 out-of-band (OOB) requirement of -27 dBm/MHz, it would be advisable to ensure that these OOB A-MPR requirements only affect the lower edge and the upper edge channels of Band n96. The inner channels would consequently be limited by the in-band PSD requirements.

## 2.2 Measurement results

#### PA calibration

- Power Class: PC5

- Calibration point: 1 dB MPR for QPSK DFT-s-OFDM, 20 MHz 100RB3 waveform at 27 dB ACLR

- Post PA losses of 4 dB

- ACLR: 27 dB

- IQ image: -28 dB

- Carrier leakage: -28 dBc

Raw PA output back-off (OBO) measurement results are captured in Table 1 below.

| WFM type | CC1 SB | CC2 SB | UL CA configuration | CC1 CBW | CC2 CBW | Agg BW | CC1 SCS | CC2 SCS | NS_53 -1dBm/MHz In-band PSD | NS_54 +17dBm/MHz In-band PSD | NS_53 -27dBm OBO PSD | NS_54 -27dBm OBO PSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DF | 1 | 1 | 1-1 | 20 | 20 | 40 | 15 | 15 | 5.5 | 0 |  |  |
| DF | 1 | 11 | 1-11 | 20 | 40 | 60 | 15 | 15 | 3.5 | 0 |  |  |
| DF | 1 | 111 | 1-111 | 20 | 60 | 80 | 15 | 30 | 2.5 | 0 |  |  |
| DF | 1 | 1111 | 1-1111 | 20 | 80 | 100 | 15 | 30 | 1.5 | 0 |  |  |
| DF | 1111 | 11 | 1111-11 | 80 | 40 | 120 | 30 | 15 | 0.5 | 0 |  |  |
| DF | 1111 | 111 | 1111-111 | 80 | 60 | 140 | 30 | 30 | 0 | 0 |  |  |
| DF | 1111 | 1111 | 1111-1111 | 80 | 80 | 160 | 30 | 30 | 0 | 0 |  |  |
| DF | 1 | 0 | 1-0 | 20 | 20 | 40 | 15 | 15 | 8.5 | 0 |  |  |
| DF | 1 | 00 | 1-00 | 20 | 40 | 60 | 15 | 15 | 8.5 | 0 |  |  |
| DF | 1 | 000 | 1-000 | 20 | 60 | 80 | 15 | 30 | 8.5 | 0 |  |  |
| DF | 1 | 0000 | 1-0000 | 20 | 80 | 100 | 15 | 30 | 8.5 | 0 |  |  |
| DF | 1000 | 00 | 1000-00 | 80 | 40 | 120 | 30 | 15 | 8.5 | 0 |  |  |
| DF | 1000 | 000 | 1000-000 | 80 | 60 | 140 | 30 | 30 | 8.5 | 0 |  |  |
| DF | 1000 | 0000 | 1000-0000 | 80 | 80 | 160 | 30 | 30 | 8.5 | 0 |  |  |
| CF | 1 | 1 | 1-1 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| CF | 1 | 11 | 1-11 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| CF | 1 | 111 | 1-111 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| CF | 1 | 1111 | 1-1111 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| CF | 1111 | 11 | 1111-11 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| CF | 1111 | 111 | 1111-111 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| CF | 1111 | 1111 | 1111-1111 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |
| CF | 1 | 0 | 1-0 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| CF | 1 | 00 | 1-00 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| CF | 1 | 000 | 1-000 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| CF | 1 | 0000 | 1-0000 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| CF | 1000 | 00 | 1000-00 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| CF | 1000 | 000 | 1000-000 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| CF | 1000 | 0000 | 1000-0000 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |
| DI | 1 | 1 | 1-1 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| DI | 1 | 11 | 1-11 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| DI | 1 | 111 | 1-111 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| DI | 1 | 1111 | 1-1111 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| DI | 1111 | 11 | 1111-11 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| DI | 1111 | 111 | 1111-111 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| DI | 1111 | 1111 | 1111-1111 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |
| DI | 1 | 0 | 1-0 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| DI | 1 | 00 | 1-00 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| DI | 1 | 000 | 1-000 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| DI | 1 | 0000 | 1-0000 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| DI | 1000 | 00 | 1000-00 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| DI | 1000 | 000 | 1000-000 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| DI | 1000 | 0000 | 1000-0000 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |
| CI | 1 | 1 | 1-1 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| CI | 1 | 11 | 1-11 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| CI | 1 | 111 | 1-111 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| CI | 1 | 1111 | 1-1111 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| CI | 1111 | 11 | 1111-11 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| CI | 1111 | 111 | 1111-111 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| CI | 1111 | 1111 | 1111-1111 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |
| CI | 1 | 0 | 1-0 | 20 | 20 | 40 | 15 | 15 |  |  |  |  |
| CI | 1 | 00 | 1-00 | 20 | 40 | 60 | 15 | 15 |  |  |  |  |
| CI | 1 | 000 | 1-000 | 20 | 60 | 80 | 15 | 30 |  |  |  |  |
| CI | 1 | 0000 | 1-0000 | 20 | 80 | 100 | 15 | 30 |  |  |  |  |
| CI | 1000 | 00 | 1000-00 | 80 | 40 | 120 | 30 | 15 |  |  |  |  |
| CI | 1000 | 000 | 1000-000 | 80 | 60 | 140 | 30 | 30 |  |  |  |  |
| CI | 1000 | 0000 | 1000-0000 | 80 | 80 | 160 | 30 | 30 |  |  |  |  |

## 2.2 A-MPR proposals

# Conclusion

# References

- R4-2311250 NR-U ULCA PC5 AMPR, 3GPP TSG-RAN WG4 #108 Toulouse, France, Apple.

- R4-2313823 Simulation results on UE RF NR-U UL CA A-MPR for PC5, 3GPP TSG-RAN WG4 #108 Toulouse, France, Charter.