3GPP RAN4 Meeting #108-bis	R4-2316791

# Xiamen, CN, October 9 – 13, 2023

Title: 	Sidelink PSSCH PSCCH MPR and A_MPR in unlicensed spectrum

Agenda Item:	5.30.2.1.2

Source: 	Qualcomm Incorporated

Document for:	Approval

# Introduction

In this paper we share PSSCH/PSCCH MPR simulation results using the example configurations that have been proposed in email after the May RAN4 meeting. We also include A_MPR for some NS cases.

All simulations in this paper are contiguous interlace combinations only, in other words non-contiguous interlace combinations are not included.

In each section we show existing NRU requirements for convenience and then simulation results.

# Power Class 5 MPR simulations

We have performed PSSCH/PSCCH simulations to determine the allowed MPR.

For convenience the MPR table for SS from 38.101-1 is shown below.

|  | case | Precoding | BW |  | RB Setup | SCS | QPSK and 16QAM MPR | 64 QAM MPR | 256 QAM MPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 |  | 105RB0 | 15 | 3.5 | 3.8 | 6.2 |
|  | 2 | CP-OFDM | 20 |  | 50RB0 | 30 | 3.5 | 3.8 | 6.0 |
|  | 3 | CP-OFDM | 40 |  | 216RB0 | 15 | 3.6 | 3.7 | 6.0 |
|  | 4 | CP-OFDM | 60 |  | 160RB0 | 30 | 3.5 | 3.6 | 6.0 |
|  | 5 | CP-OFDM | 80 |  | 216RB0 | 30 | 3.6 | 3.7 | 6.2 |
|  | 6 | CP-OFDM | 100 |  | 270RB0 | 30 |  |  |  |
| Interlaced Allocation Single CC | 7 | CP-OFDM | 20 |  | 1 RB0 every 10RBs | 15 | 3.5 | 3.7 | 6.2 |
|  | 8 | CP-OFDM | 20 |  | 1RB0 every 5RBs | 30 | 3.6 | 3.7 | 6.1 |
|  | 9 | CP-OFDM | 40 |  | 1RB0 every 10RBs | 15 | 3.6 | 3.9 | 6.4 |
|  | 10 | CP-OFDM | 60 |  | 1RB0 every 5RBs | 30 | 3.5 | 3.7 | 6.3 |
|  | 11 | CP-OFDM | 80 |  | 1RB0 every 5RBs | 30 | 3.7 | 3.9 | 6.5 |
|  | 12 | CP-OFDM | 100 |  | 1RB0 every 5RBs | 30 | 3.6 | 4.1 | 6.6 |
| Wide band Full operation | 13 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 3.6 | 3.7 | 6.0 |
|  | 14 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 3.1 | 3.6 | 6.0 |
|  | 15 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3.3 | 3.7 | 6.0 |
|  | 16 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 3.1 | 3.6 | 6.0 |
|  | 17 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3.3 | 3.7 | 6.0 |
|  | 18 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3.4 | 3.6 | 6.0 |
|  | 19 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.3 | 3.6 | 6.0 |
|  | 20 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.3 | 3.5 | 5.9 |
|  | 21 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 3.1 | 3.6 | 6.0 |
|  | 22 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3.3 | 3.7 | 6.0 |
|  | 23 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3.4 | 3.6 | 6.0 |
|  | 24 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3.4 | 3.7 | 6.2 |
|  | 25 | CP-OFDM | 100 |  | Bitmap 01000 | 30 |  |  |  |
|  | 26 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.4 | 3.5 | 5.9 |
|  | 27 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.5 | 3.8 | 6.5 |
|  | 28 | CP-OFDM | 100 |  | Bitmap 00100 | 30 |  |  |  |
| Wide band Interlaced operation | 29 | CP-OFDM | 40 |  | Bitmap 10 | 15 | 3.5 | 3.7 | 6.0 |
|  | 30 | CP-OFDM | 60 |  | Bitmap 100 | 30 | 3.2 | 3.8 | 6.1 |
|  | 31 | CP-OFDM | 60 |  | Bitmap 110 | 30 | 3.5 | 4.0 | 6.6 |
|  | 32 | CP-OFDM | 80 |  | Bitmap 1000 | 30 | 3.2 | 3.8 | 6.1 |
|  | 33 | CP-OFDM | 80 |  | Bitmap 1100 | 30 | 3.5 | 4.0 | 6.6 |
|  | 34 | CP-OFDM | 80 |  | Bitmap 1110 | 30 | 3.5 | 4.0 | 6.6 |
|  | 35 | CP-OFDM | 80 |  | Bitmap 0100 | 30 | 1.4 | 3.8 | 6.1 |
|  | 36 | CP-OFDM | 80 |  | Bitmap 0110 | 30 | 1.6 | 4.0 | 6.5 |
|  | 37 | CP-OFDM | 100 |  | Bitmap 10000 | 30 | 3.2 | 3.8 | 6.1 |
|  | 38 | CP-OFDM | 100 |  | Bitmap 11000 | 30 | 3.5 | 4.1 | 6.6 |
|  | 39 | CP-OFDM | 100 |  | Bitmap 11100 | 30 | 3.5 | 4.0 | 6.6 |
|  | 40 | CP-OFDM | 100 |  | Bitmap 11110 | 30 | 3.6 | 3.9 | 6.5 |
|  | 41 | CP-OFDM | 100 |  | Bitmap 01000 | 30 | 1.4 | 3.8 | 6.1 |
|  | 42 | CP-OFDM | 100 |  | Bitmap 01100 | 30 | 1.6 | 4.0 | 6.5 |
|  | 43 | CP-OFDM | 100 |  | Bitmap 01110 | 30 | 1.6 | 4.0 | 6.6 |
|  | 44 | CP-OFDM | 100 |  | Bitmap 00100 | 30 |  |  |  |

Table: Power class 5 PSSCH/PSCCH MPR simulation results

Proposal MPR: The power class 5 PSSCH/PSCCH simulation results in the table should be considered in the discussion of MPR requirements.

# n46 NS_31 A-MPR

In the last meeting we agreed that the NS values shown below would be included in rel18. Other NS A-MPR requirements are not precluded.

From the last meeting report, we have to following agreement, and our understanding is other NS cases are not precluded in rel18.

Issue 3-2: A-MPR NS values Discussions: LGE: fine with the proposal. We do not limit to these ones. Meta: fine with the reduced network signalling for simulation campaign. Other network signalling simulation should be delayed to Rel-19. Agreement: - Finalize the following NS values for each band in Rel-18: - n46 : NS_31 - n96 : NS_53, NS_60, NS_61 - n102 : NS_58

For convenience the existing NS_31 A_MPR is shown below. For SL we are interested in the CP-OFDM part.

We performed simulations for the full and partial allocations and computed the A-MPR for NS_31. The results are shown in the tables below.

| NS_31 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤ 5.2 | ≤ 5.7 | ≤ 8.3 | ≤ 8.6 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.1 |
| 16 QAM | ≤ 5.6 | ≤ 5.8 | ≤ 8.0 | ≤ 9.0 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.3 |
| 64 QAM | ≤ 5.5 | ≤ 6.0 | ≤ 8.0 | ≤ 9.3 | ≤ 8.1 | NA | ≤ 8.1 | ≤ 8.5 |
| 256 QAM | ≤ 5.6 | ≤ 5.9 | ≤ 8.0 | ≤ 9.5 | ≤ 8.0 | NA | ≤ 8.0 | ≤ 8.5 |

Table: Simulation results for NS_31 Full and partial allocations.

Proposal NS_31 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

# N102 NS_58

NRU spec information for the NS case is shown here.

| NS_58 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤5.2 | ≤5.4 | ≤5.9 | ≤6.0 | ≤5.6 | ≤5.8 | ≤5.4 | ≤5.7 |
| 16 QAM | ≤5.3 | ≤5.6 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |
| 64 QAM | ≤5.3 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.6 | ≤5.9 | ≤5.6 | ≤5.8 |
| 256 QAM | ≤5.4 | ≤5.8 | ≤5.8 | ≤6.2 | ≤5.7 | ≤5.9 | ≤5.6 | ≤5.8 |

Table: Simulation results for NS_58 Full and partial allocations.

Proposal NS_58 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

# N96 NS_53, NS_60, NS_61

NRU spec information for the NS case is shown here.

| NS_53 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤7.5 | ≤7.6 | ≤7.0 | ≤7.3 | ≤6.7 | ≤7.4 | ≤6.4 | ≤7.4 |
| 16 QAM | ≤7.5 | ≤7.8 | ≤6.9 | ≤7.4 | ≤6.8 | ≤7.6 | ≤6.4 | ≤7.6 |
| 64 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.6 | ≤6.8 | ≤7.8 | ≤6.5 | ≤7.8 |
| 256 QAM | ≤7.5 | ≤8.0 | ≤7.0 | ≤7.5 | ≤6.8 | ≤7.7 | ≤6.6 | ≤7.7 |

Table: Simulation results for NS_53,60,61 Full and partial allocations.

Proposal NS_53,60,61 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

# N46 NS_30 A-MPR

NRU spec information for the NS case is shown here.

| NS_30 A-MPR Allocation Note2 | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤10.7 | ≤10.9 | ≤10.4 | ≤11.0 | ≤9.3 | ≤10.3 | ≤9.1 | ≤10.4 |
| 16 QAM | ≤10.8 | ≤9.0 | ≤10.1 | ≤11.1 | ≤9.6 | ≤10.5 | ≤9.2 | ≤10.5 |
| 64 QAM | ≤10.9 | ≤11.4 | ≤10.1 | ≤11.4 | ≤9.6 | ≤10.6 | ≤9.3 | ≤10.6 |
| 256 QAM | ≤11.0 | ≤11.5 | ≤10.0 | ≤11.5 | ≤9.7 | ≤10.6 | ≤9.4 | ≤10.5 |

Table: Simulation results for NS_30 Allocation Note 2 Full and partial allocations.

| NS_30 A-MPR Allocation Note3 | 20M | 20M | 40M | 40M |
| --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial |
| QPSK | ≤8.3 | ≤8.4 | ≤5.9 | ≤6.2 |
| 16 QAM | ≤8.1 | ≤8.7 | ≤5.8 | ≤6.3 |
| 64 QAM | ≤8.3 | ≤9.0 | ≤5.8 | ≤6.3 |
| 256 QAM | ≤8.4 | ≤8.9 | ≤5.8 | ≤6.3 |

Table: Simulation results for NS_30 Allocation Note 3 Full and partial allocations.

Proposal NS_30 A-MPR: The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

# N46 NS_28 A-MPR

NRU spec information for the NS case is shown here.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

Table: Simulation results for NS_28 Allocation Full and partial allocations.

Proposal NS_28 A-MPR: The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

# Conclusions

Proposal MPR: The power class 5 PSSCH/PSCCH simulation results in the table should be considered in the discussion of MPR requirements.

Proposal NS_31 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

Proposal NS_58 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

Proposal NS_53,60,61 A-MPR: The power class 5 PSSCH A-MPR in the table should be considered in the discussion of MPR requirements.

Proposal NS_30 A-MPR: The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

Proposal NS_28 A-MPR: The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

# References

- 3GPP TS 38.101-1 v18.2.0