3GPP TSG RAN WG4 Meeting #108bis	R4-2316819

Xiamen, China, 09 Oct.  2023 – 13 Oct. 2023

Agenda Item:	5.14.4

Source: 	Ericsson

Title:	Discussion on RRM requirements for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we look at the RRM requirements that may be impacted due to introduction of NR in less than 5 MHz bandwidth.

# Discussion

Following issues are open after last meeting discussions.

## Impact on RLM

### Hypothetical PDCCH transmission parameters

In last meeting following is agreed.

PDCCH transmission parameters for SSB-based RLM and BFD:

- Issue 1-5: The BW in hypothetical PDCCH transmission parameters for SSB-based RLM

  - Define SSB-based RLM requirements for 12 PRBs [and 15 PRBs] hypothetical PDCCH transmission bandwidth.

    - FFS on applicability rules of RLM requirements for different bands

In last meeting it was discussed whether to define SSB based RLM requirements for 15 PRBs or not. As per RAN1 design, 15 PRB with puncturing is only applicable for CORESET 0 and not for other CORESET. For CORESET other than CORESET 0, the size should be multiple of 6 PRBs. Hence, RLM requirements is to be defined only for 12 PRBs.

- RAN4 to define SSB based RLM requirements for only 12 PRBs.

Out of sync parameters:

In last meeting following table is agreed for 12 and 15 PRBs. However, since only 12 PRBs can be possible form RAN1 design, we suggest removing 15 PRB from the table.

| Attribute | Value for BLER Configuration #0 |  |
| --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) |
| DCI format | 1-0 |  |
| Number of control OFDM symbols | [2] | [3] |
| Aggregation level (CCE) | [4] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |
| Bandwidth (PRBs) | 12 | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |
| DMRS precoder granularity | REG bundle size |  |
| REG bundle size | 6 |  |
| CP length | Normal |  |
| Mapping from REG to CCE | Distributed |  |

- RAN4 to agree on following table for OOS parameters.

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
|  | 3MHz (12 PRBs) |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 12 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

In sync parameters:

Similar to OOS, PDCCH for in sync also can be configured with only 12 PRBs. Since only 12 PRBs can be possible from RAN1 design, we suggest removing 15 PRB from the table. We suggest following table to be captured in the specification.

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
|  | 3MHz (12 PRBs) |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 2 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 12 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

- RAN4 to agree on following table for IS parameters

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
|  | 3MHz (12 PRBs) |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 2 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 12 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

## Link recovery procedures

Link recovery procedures mainly consists of beam failure detection and candidate beam detection.

### Beam failure detection (BFD)

Since the parameters and evaluation period principles are same as RLM, conclusion of RLM can be applied to BFD too.

- RLM OOS table for RLM can be used for BFD.

### Candidate beam detection (CBD)

As we agreed in previous meeting RLM IS parameters can be reused for CBD.

- RLM IS table can be reused for CBD.

## Impact on SSB Index reading

Based on the agreed simulation parameters we simulated changing different parameters and we present our observations in these sections.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: AWGN

Observations:

- For AWGN with 12 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -9 dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-A 30ns

Observations:

- For TDL-A 30ns, with 12 PRB punctured SSB, at least 3 SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-B 100ns

Observations:

- For TDL-B 100ns, with 12 PRB punctured SSB, at least 2 SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-C 300ns

Observations:

- For TDL-C 300ns, with 12 PRB punctured SSB, at least 2 SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -6 dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Observations:

- For HST single tap model with 2*416.67Hz constant Doppler shift, with 12 PRB punctured SSB, at least one SSB is required to achieve 99% successful decoding of the PBCH SSB Index at SNR = -8 dB.

Based on the observations 1 to 4, we make following observation.

- For 12 PRB punctured SSB, at least 2 SSBs are required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -6dB.

Based on the observation 5, we make following observation.

- For 12 PRB punctured SSB for HST scenario, at least 1 SSB is required to achieve 99% successful decoding of the SSB Index reading under SNR side condition of -8dB.

## Impact on MIB reading delay

We consider following simulation parameters as baseline and simulated changing different parameters and we present our observations in these sections.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: AWGN

Observations:

- For AWGN, with 12 PRB punctured SSB, at least two SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-A 30ns

Observations:

- For TDL-A 30ns, with 12 PRB punctured SSB, at least 16 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB and 8 SSBs for SNR=-4dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-B 100ns

Observations:

- For TDL-B 100ns, with 12 PRB punctured SSB, at least 16 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB and 8 SSBs for SNR=-4dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 3km/h; Channel model: TDL-C 300ns

Observations:

- For TDL-C 300ns, with 12 PRB punctured SSB, at least 14 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB and 8 SSBs for SNR=-4dB.

Simulation assumption: PRB 12, SNR -10:0db; Speed: 500km/h; Channel model: HST single tap with 2*416.67Hz constant Doppler shift

Observations:

- For HST single tap with 2*416.67Hz constant Doppler shift, with 12 PRB punctured SSB, at least 2 SSBs are required to achieve 99% successful decoding of the MIB at SNR = -6 dB.

Based on the observations 8 to 11, we make following observation.

- For 12 PRB punctured SSB, at least 8 SSB are required to achieve 99% successful decoding of the MIB under SNR side condition of -4dB.

Based on observation 12, we make following observation.

- For 12 PRB punctured SSB for HST scenario, at least 2 SSBs are required to achieve 99% successful decoding of the MIB reading under SNR side condition of -6dB.

Based on Observations 6, 7, 13, 14, we summarise the delay requirement in the following table

|  | SSB index reading delay | MIB reading delay |
| --- | --- | --- |
| Normal speed | 2 SSB are needed at -6dB | 8 SSB at -4dB |
| HST | 1 SSB at -8dB | 2 SSB at -6dB. |

- RAN4 to agree on following table as the delay requirements for SSB Index reading and MIB reading

|  | SSB index reading delay | MIB reading delay |
| --- | --- | --- |
| Normal speed | 2 SSB at -6dB | 8 SSB at -4dB |
| HST scenario | 1 SSB at -8dB | 2 SSB at -6dB. |

## Impact of SSB Index reading on HO requirements

In last meeting there was some discussion on whether SSB index reading should be included in the cell search delay component of the HO. We agree that UE need SSB index reading to identify the RACH occasion and RACH preamble. However, we are not sure this SSB index reading would impact the cell search delay. Because cell search delay can be 0 for known cell irrespective of L3 measurement is configured with SSB Index reading or not. Our understanding is that SSB index reading is performed during the fine timing acquisition and one SSB is enough for that. In our understanding SSB index reading do not impact the HO delay.

- RAN4 to agree that SSB index reading delay do not impact the HO delay.

## Applicability of requirements which are not impacted

At the start of the WI RAN4 identified the set of requirements, that are impacted or not impacted (i.e. legacy requirements can be reused). We think there can be two types of UEs supporting this feature.

- UE supporting other Channel Band width (CBW) along with less than 5 MHz CBW

- UE supporting only less than 5 MHz

For 1st type of UE, even if we do capture the requirements which are impacted separately, since UE supports other bands also, the requirements applicability may not have any issue. However, for the second type of UE which only support less than 5 MHz, if we do not mention each requirement applicability explicitly, there could be confusion at UE side which requirements to satisfy. To avoid this, we think we need to add an applicability sentence for each of the requirement that are not impacted also.

- RAN4 to add applicability rule of existing requirements to less than 5 MHz UE explicitly for each of the requirements applicable for the less than 5 MHz UE.

# Summary and Conclusion

In this contribution we have analysed RAN4 aspects for NR less than 5 MHz and made following proposals.

- RAN4 to define SSB based RLM requirements for only 12 PRBs.

- RAN4 to agree on following table for OOS parameters.

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
|  | 3MHz (12 PRBs) |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 12 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

- RAN4 to agree on following table for IS parameters

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
|  | 3MHz (12 PRBs) |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 2 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 12 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

- RLM OOS table for RLM can be used for BFD too.

- RLM IS table to be reused for CBD too.

- RAN4 to agree on following table as the delay requirements for SSB Index reading and MIB reading

|  | SIB reading delay | MIB reading delay |
| --- | --- | --- |
| Normal speed | 2 SSB at -6dB | 8 SSB at -4dB |
| HST scenario | 1 SSB at -8dB | 2 SSB at -6dB. |

- RAN4 to agree that SSB index reading delay do not impact the HO delay.

- RAN4 to add applicability rule of existing requirements to less than 5 MHz UE explicitly for each of the requirements applicable for the less than 5 MHz UE

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R4-2306244

- R4-2306346

- R4-2306400