3GPP RAN WG4 Meeting #108-bis		R4-2316835

Xiamen, China, Oct 09-13, 2023

Agenda item:	5.15.3

Source:	Telecom Italia, Vodafone

Title:	Further updates to the working procedure for TRP TRS Performance Test Campaign

Release:	Rel-18

Related WI:	NR_FR1_TRP_TRS_enh-Perf

Document for:	Discussion and Decision

# 1	Introduction

During the RAN4#108 meeting in Toulouse, an updated working procedure for Rel-18 Measurement Campaign to define requirements was approved in section 5 of [1]. Based on the agreements reported in the approved WF [2], this contribution aims at addressing some of the open points towards the finalization of such working procedure.

# 2	Updated Working procedure for Rel-18 Measurement Campaign to define requirements (for approval)

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: Complete full coverage of band n1, n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17;

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx  as phase 1, study how to specify EN-DC requirements based on SA measurement results. SA with 2Tx as phase 2.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm) as 1st priority; Size 2 as 2nd priority

  - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - Year of production: from second-half 2021 to 2023

  - The following selection criteria can also be considered:

- Year of production: [2020-2023]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with the selected test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - A test lab shall measure only one UE model in case different samples are provided

    - FFS for the same Same UE model with supporting different set of bands can be measured

  - Logistical aspects for devices provisioning to the labs are TBD]The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects

    - Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

- Test results submitting:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: laboratories use the spreadsheet in [TBD] to submit the device information At least all the supported bands information and production year should be shared

    - Other UE information disclosure (and thresholds) depends on further discussions, which can be added based on agreements.

  - Using Laboratories use the same worksheet template in [TBD] to submit the measurement results for Rel-18 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to the rapporteur

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per Power Class

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - [Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB ,GCF, and NAL/CTA (Chinese network access licensed test)]

    - Percentage of devices that are commercially available

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to be shared on the RAN4 reflector (for example - how many devices have been measured and on which bands)

    - Information of the devices that are going to be measured should be shared with the RAN4 Secretary as soon as available in order to monitor the achievement of the thresholds defined in point 8.b.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Only the results from aligned labs will be considered for specifying requirements

  - Requirements will not be specified if the following thresholds are not satisfied by the devices pool:

    - Minimum number of devices for each band, each device size, each power class: 40 (this number is in line with Rel-17)

    - Minimum number of device models: 30

    - Minimum number of devices' vendors: 5

    - Percentage of devices from second-half 2021 to 2023: 100%

    - Percentage of the devices that are certified by PTCRB / GCF / NAL-CTA: 100%

    - Percentage of devices that are commercially available: 100%

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-18.

  - Method of limits derivation: per-band Data driven approach

  - The value at [TBD] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, study how to specify PC3 requirement based on finalized PC2 requirements

- Test lab procedures

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

# 3	Conclusions

In this contribution it is proposed to further update the working procedure for TRP TRS Performance Test Campaign for Rel-18 TRP TRS WI to address some of the open points.

Proposal: Approve the updated working procedure for TRP TRS Performance Test Campaign for Rel-18 TRP TRS WI.

# 4	References

- R4-2313894, “Further updated working procedure for Rel-18 lab alignment and harmonization campaign”, vivo, 3GPP RAN4#108, August 2023

- R4-2313891, “WF for Rel-18 FR1 TRP TRS”, vivo, August 2023