3GPP TSG-RAN WG4 Meeting # 108bis					R4-2316847

Xiamen, China, October 09 – October 13, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on release independence aspects for n31 and n72 bands

Agenda Item:	4.34.1

Document for:	Discussion

# Introduction

During last RAN#101 meeting, new WI on Introduction of NR bands n31 and n72 was approved in [1].

In this contribution we provide brief analysis of the release independence aspects captured in the WID.

# Discussion

R3eferring to the WID in [1], the following objectives were captured:

One of the objectives is to introduce system parameters and RF characteristics for 3 and 5 MHz channel bandwidth for n31 and n72.

Referring to the above extracted notes, the intention was to introduce n31 and n72 in release independent way starting from Rel-15. However, concerns were raised during RAN#101 discussion on the feasibility of Rel-15 release independence for 3 MHz NR channel bandwidth for n31 and n72.

While there is no issue with having Rel-15 release independence for all the NR channel bandwidths larger than 5 MHz which were introduced for NR in Rel-15, it is not obvious that 3 MHz can be supported by legacy devices.

It shall be noted, that 3 MHz channel bandwidth was introduced for n100, n106, n26, n28 and n85 operating bands in Rel-18 under WI in [2], with the core part of the WI still ongoing due to unresolved issues for n100.

CR in [3] was agreed introducing new GSCN parameters for the global frequency for 3 MHz channel bandwidth, as well as applicable SS raster entries. Comparing introduced changes, one can notice that some RF requirements for above and below 3MHz channel do differ, e.g.

- MPR for 3 MHz edge RB was increased from 0.5 dB to 1 dB for Pi/2 BPSK.

- Spectrum emission mask for 3MHz is also more stringent (± 5-6 region) than for wider channel bandwidths.

- Etc.

It shall be also noted that layer 1 design was specifically adjusted for 3 MHz based on n100 constraints due to GSM-R refarming roadmap, leading to a specific PBCH design with reduce RB allocation.

Considering all the above, the following is observed:

Observation 1: 3 MHz NR RF requirements and system design differs from other existing Rel-15 requirements.

Observation 2: Legacy (Rel-15) UEs behaviour cannot be guaranteed when scheduled by the network to use 3MHz channel allocation for Rel-18 bands.

# Conclusions

Based on the above short discussion, the following proposal was formulated:

Proposal 1: Clarify in the WID, that 3 MHz channel bandwidth cannot be introduced in release independent way from Rel-15 onwards.

Related modification can be implemented during RAN#102 based on Rapporteurs input.

Proposal 2: Consired below modification of the Notes to implement clarifications on 3 MHz and 5 MHz channel bandwidths introduction for n31 and n72:

Note 1: 5 MHz channel bandwidth will be introduced in a REL-independent way starting from REL-15 for n31 and n72. Note 2: 3MHz channel bandwidth will be introduced for Rel-18 onwards.

# References

[1]	RP-232691	Introduction of NR bands n31 and n72, WID, RAN#101

[2]	RP-231425	NR support for dedicated spectrum less than 5MHz for FR1, NR_FR1_lessthan_5MHz_BW

[3]	R4-2314764	CR to 38.101-1 Introduction of dedicated spectrum less than 5MHz for FR1, RAN4#108