3GPP TSG-RAN WG4 Meeting #108bis	R4-2316858

Xiamen, China, Oct 09 – Oct 13, 2023

Agenda item:	5.20.4

Source: 	Qualcomm Incorporated

Title: 	Review of outcome of RAN1 studies related to RRM

Document for:	Discussion

# Introduction

During RANP#101, the SID on Rel-18 Low Power Wake Up Signal (LP-WUS) was revised to include the following objective for RRM[1].

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - To review the outcome of RAN1 studies on serving cell RSRP/RSRQ measurement offloading to LP-WUR for IDLE/INACTIVE mode for feasibility verification [RAN4]. - Consider different LP-WUR architectures: - LP-SS based RRM measurement for envelop detection-based LP-WUR - SSS based RRM measurement for OFDM based LP-WUR - For each of above, to review: - SNR target X for LP-WUR RRM measurement considering the practical noise figure of LP-WUR

As a part of the SI, RAN1 has been discussing multiple aspects of the LP-WUR including Low power wakeup signal (LP-WUS) design, waveform generation, monitoring schemes and RRM measurements. RAN1 has identified that with a LP-WUR, UE achieves very little power savings without offloading at-least the serving cell measurements from the main radio (MR) to the LP-WUR. In that regard, determining RRM offloading mechanisms become an important aspect of the LP-WUR. In this contribution paper, we present our views on Radio resource management (RRM) measurements with LP-WUR considering the agreements from RAN1.

# RRM Measurement with LP-WUR

The primary use-case of LP-WUR is to enable the Main-Radio go to deep-sleep state and save significant power, where the LP-WUR monitors for a WUS and wakes up the MR to receive paging message when the WUS indicates so. Although this may provide some power savings, but the main bottleneck here is the RRM measurements in the IDLE/INACTIVE mode which requires the Main Radio to perform periodic measurements and cell-reselection criteria evaluation on the serving cells, as well as the neighbour cells.

A UE operating in RRC-IDLE and RRC-INACTIVE states performs RSRP and RSRQ measurements on the serving cell as well as multiple neighbour cells to ensure that the UE is camping on the best cell. For the serving cell measurement, the following is specified in TS 38.133 [2].

The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 for the serving cell at least once every M1*N1 DRX cycle; where: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second, otherwise M1=1.

The value of N1 is different for FR1 and FR2. For FR1, N1 is equal to 1. It means that NR UE should perform serving cell measurement every 1 or 2 DRX cycles. Therefore, RRM measurement is a significant contributor to the overall power consumption in RRC idle or inactive state.

Observation 1: During IDLE/INACTIVE mode, RRM measurements performed by the Main Radio, for serving cell and neighbour cell evaluation for cell-reselection limit the achievable power-savings with the LP-WUR.

Proposal 1: RAN4 agrees with RAN1 understanding/conclusion that LP-WUR operation without measurement offloading/relaxations limits the achievable UE power savings.

In Rel-16, relaxations for neighbor cell measurements was specified for UEs that are not at cell edge and/or with low mobility and additional measurement relaxation criteria for supporting stationary RedCap UEs were specified during Rel-17. It’s worth mentioning that no relaxations are allowed for the serving cell measurements and evaluation. So relaxing serving cell measurements could provide significant power savings with the LP-WUR.  But at the same time, we would like to highlight that serving cell measurements are critical for IDLE/INACTIVE state mobility and may break the system if not done appropriately. So utmost care should be taken in defining the conditions/criteria for serving cell measurement relaxations. Such conditions may limit the achievable power savings only to certain scenarios, e.g., a stationary UE that is not at the cell edge. To extend the power savings to other possible scenarios, offloading the RRM measurements from the MR to the WUR need to be considered.

Although both the serving cell and the neighbor cell measurements could potentially be offloaded to the WUR, we need to evaluate the trade-off between the power savings and increased LP-WUR complexity. Neighbour cells configured for measurements/cell-reselection may include intra-frequency, inter-frequency and inter-RAT cells. In order to measure inter-frequency and inter-RAT cells, the UE needs to perform RF re-tuning in most cases which adds a lot of complexity to the WUR, if it performs such measurements. The power-savings gain on the other hand may not be significant such measurements are typically triggered when the serving cell becomes bad and neighbor cell measurements are needed in such cases.

Observation 2: Relaxations are specified only for the neighbouring cells and not for the serving cells.

Observation 3: Specifying serving cell measurement relaxations provides power-savings but not in all cases

Proposal 2: RAN4 supports measurement offloading to the LP-WUR at-least for the serving cell. Relaxations for the serving cell measurements can be additionally considered.

- Further discuss the scenarios where measurements can be relaxed and/or offloaded

- The exact relaxations and offloading mechanism to be specified during the WI phase

RAN1 has studied the LP-WUR signal design and have identified two options for RRM measurements.

- LP-SS based measurements for envelop detection-based LP-WUR and

- SSS based RRM measurement for OFDM based LP-WUR

From RAN4 point of view, RRM measurement offloading and/or relaxation is possible with both the options, but since the nature of the two waveforms is different, the accuracy level may differ, and different number of samples may be needed to achieve the same target SNR level.

RAN4 specifies (absolute) SS-RSRP accuracy levels +/-6db for an FR1 cell configured for CA/DC Idle mode measurements. The requirements are applicable for a UE in state RRC_IDLE or RRC INACTIVE that is synchronised to the cell that is measured where the SSB Ês/Iot >=-4db under normal conditions. For the connected mode measurements, RAN4 specified accuracy levels of +/-4.5db for SSB Ês/Iot >=-6db. Compared to the MR, LP-WUR may have limited measurement capability (e.g., due to the simple receiver architecture), and LP-WUR measurement may not have the same accuracy as MR measurement. However, with filtering across sufficient number of samples and a careful design of the LP-SS/SSS periodicity, a desired accuracy of +/-4.5 to +/-6db can be targeted for the LP-SS/SS based measurements by the LP-WUR.

Observation 4: RAN4 specifies (absolute) SS-RSRP accuracy levels +/-6db for an FR1 cell configured for CA/DC Idle mode measurements with SSB Ês/Iot >=-4db.

Observation 5: RAN4 specifies (absolute) SS-RSRP accuracy levels +/-4.5db for an FR1 cell configured for Connected mode measurements with SSB Ês/Iot >=-6db.

Proposal 3: In RAN4’s view, a desired accuracy of +/-4.5 to +/-6db can be targeted for the LP-SS/SS based measurements by the LP-WUR.

- Note that the exact accuracy levels shall be specified during the WI phase.

Although the target SSB Ês/Iot >=-4db to -6db is typically used for the Main Radio, the LP-WUR may have higher noise figure, so the target SNRs can be lower than what was assumed for the MR if a similar cell coverage is assumed. This needs to be further studied and inputs from RF need to be considered.

Proposal 4: RAN4 to further study the SNR targets and the number of measurement samples needed to achieve the SNR target shall be specified during the WI phase.

# Conclusion

Observation 1: During IDLE/INACTIVE mode, RRM measurements performed by the Main Radio, for serving cell and neighbour cell evaluation for cell-reselection limit the achievable power-savings with the LP-WUR.

Proposal 1: RAN4 agrees with RAN1 understanding/conclusion that LP-WUR operation without measurement offloading/relaxations limits the achievable UE power savings.

Observation 2: Relaxations are specified only for the neighbouring cells and not for the serving cells.

Observation 3: Specifying serving cell measurement relaxations provides power-savings but not in all cases

Proposal 2: RAN4 supports measurement offloading to the LP-WUR at-least for the serving cell. Relaxations for the serving cell measurements can be additionally considered.

- Further discuss the scenarios where measurements can be relaxed and/or offloaded

- The exact relaxations and offloading mechanism to be specified during the WI phase

Observation 4: RAN4 specifies (absolute) SS-RSRP accuracy levels +/-6db for an FR1 cell configured for CA/DC Idle mode measurements with SSB Ês/Iot >=-4db.

Observation 5: RAN4 specifies (absolute) SS-RSRP accuracy levels +/-4.5db for an FR1 cell configured for Connected mode measurements with SSB Ês/Iot >=-6db.

Proposal 3: In RAN4’s view, a desired accuracy of +/-4.5 to +/-6db can be targeted for the LP-SS/SS based measurements by the LP-WUR.

- Note that the exact accuracy levels shall be specified during the WI phase.

Proposal 4: RAN4 to further study the SNR targets and the number of measurement samples needed to achieve the SNR target shall be specified during the WI phase.

# References

[1] RP-232672, “Revised WID: Study on low-power Wake-up Signal and Receiver for NR”, Vivo, RAN#101