3GPP TSG-RAN WG4 Meeting # 108-bis									          R4-2316859

Xiamen, China, 9th October – 13th October, 2023

Title: 	On UL1-DL4 harmonic mixing and table templated for 1UL-CC and 2UL-CC MSD studies

Source: 	Skyworks Solutions Inc.

Agenda Item:	5.1.3	Simplification of specification and reduction of test burden [FS_SimBC]

Document for:	Approval

# 1	Background

In RAN4#108, a discussion on whether UL1/DL4 harmonic mixing should be specified was raised from a contribution proposing an UL1/DL4 MSD for a low band victim. It was also questioned whether the answer should be different depending on the UL aggressor power class. In this contribution, we provide technical background on the even harmonic mixing implementations aspects, its dependency versus DL frequency, and UL power class. Additionally, we provide an updated template for MSD analysis of 1UL band/1CC and 2ULband/1CC per band cases for discussion.

# Discussion

## 2.1 Even harmonic mixing implementation aspects

As a first step, we provide in Table 1 the current even harmonic mixing cases specified in 38.101-1.

Table 1: Specified even harmonic mixing cases for PC3, PC2 and PC1.5

| UL band | Power Class | DL band | DL band range | UL/DL collision |
| --- | --- | --- | --- | --- |
| n3 | PC3 | n26 | <1GHz | UL1/DL2 |
| n7 | PC3 | n71 | <1GHz | UL1/ DL4 |
| n40 | PC3 | n77/78 | >3GHz | UL3/DL2 |
| n41 | PC3 | n39 | >1GHz and <3 GHz | UL3/DL4 |
| n41 | PC3 | n77/78 | >3GHz | UL3/DL2 |
| n46 | PC5 | n7 | >1GHz and <3 GHz | UL1/DL2 |
| n77 | PC3/2/1.5 | n2 | >1GHz and <3 GHz | UL1/DL2 |
| n77 | PC3/2/1.5 | n25 | >1GHz and <3 GHz | UL1/DL2 |
| n77 | PC2/PC1.5 | n3 | >1GHz and <3 GHz | UL1/DL2 |
| n77 | PC2/PC1.5 | n5 | <1GHz | UL1/DL4 |
| n77 | PC3 | n8 | <1GHz | UL1/DL4 |
| n77 | PC3 | n70 | >1GHz and <3 GHz | UL1/DL2 |
| n78 | PC2 | n3 | >1GHz and <3 GHz | UL1/DL2 |
| n78 | PC2 | n8 | <1GHz | UL1/DL4 |

Observations:

- Even if the question raised in RAN4#108 was about UL1/DL4, there other even harmonic mixing cases that are specified even for PC3:

  - UL1/DL2 down to DL band <1GHz

  - UL3/DL2 for DL band >3GHz

  - UL1/DL4 down to DL band <1GHz

  - UL3/DL4 down to DL band >1GHz and <3GHz

- When the same harmonic mixing case is specified for different power classes, the specified MSD values corresponds to an interference level raising dB per dB, which is consistent with the theory as harmonic mixing relates to a parasitic mixing gain/loss at harmonics. For example, CA_n2-n77 MSD value for n2 is 6.7, 9.1 and 11.8dB for PC3, PC2 and PC1.5, respectively.

- Note that some cases are PC5 in the PC3 table as they pertain to NR-U bands

Therefore, in order to address which even harmonic mixing MSD should be specified and under which conditions, some implementations aspects should be noted:

- Balanced mixer should cancel H2 and H4 harmonic response, but cancellation level depends on the 180-degree error

- Quadrature mixers should further cancel H4 harmonic response, but cancellation level depends on the 90-degree errors

- If the error is dominated by a fixed delay, thus, the cancellation degrades with increased mixer LO frequency

- Being a cancellation mechanism, the BB interference level will rise dB per dB with the UL band power class.

- DL4 harmonic mixing response is lower than DL2 Harmonic response as there is some filtering benefits and use of quadrature mixers further improves it.

Based on the above analysis, the question is not whether UL1/DL4 MSDs should be specified, but rather for which DL frequency range and power level should the different even harmonic mixing MSD be specified:

- UL1/DL2 then UL1/DL4 have the strongest response since the interferer can reach PCmax and the UL filter does not attenuate the interferer level, only the DL filter does. Also, the PCB isolation is critical

- UL3/DL2 then UL3/DL4 have much lower response since the interferer is several tens of dB lower than PCmax and the UL filter will attenuate the interferer level on top of the DL filter does. Also, the PCB isolation is less critical.

- As a first approach (empirical) the following rules with simple frequency boundaries could be used:

  - UL1/DL2 MSD should be investigated and specified if necessary, for all FR1 DL bands and all power classes

  - UL1/DL4 MSD should be investigated and specified if necessary, for all FR1 DL bands for PC2 and PC1.5 and for DL bands >1GHz for PC3

  - UL3/DL2 MSD should be investigated and specified if necessary, for all FR1 DL bands >3GHz for PC3 and >1GHz for PC2 and PC1.5

  - UL3/DL4 MSD should be investigated and specified if necessary, for all FR1 DL bands >5GHz for PC5 and PC3 and >3GHz for PC2 and PC1.5

  - Depending on the available filter and PCB isolations, the MSD level is calculated and if above 0.5dB, specified.

Proposal: Even harmonic mixing MSD should be investigated and specified if necessary, for:

- UL1/DL2 for all FR1 DL bands and all power classes

- UL1/DL4 for all FR1 DL bands for PC2 and PC1.5 and for DL bands >1GHz for PC3

- UL3/DL2 for all FR1 DL bands >3GHz for PC3 and >1GHz for PC2 and PC1.5

- UL3/DL4 for all FR1 DL bands >5GHz for PC5 and PC3 and >3GHz for PC2 and PC1.5.

Based on the above proposal, Table 1 can be re-assessed.

Proposal on correction of current harmonic mixing cases in 38.101-1 in the last column of the table below:

| UL band | Power Class | DL band | DL band range | UL/DL collision | Case specified |
| --- | --- | --- | --- | --- | --- |
| n3 | PC3 | n26 | <1GHz | UL1/DL2 | Yes for all power classes |
| n7 | PC3 | n71 | <1GHz | UL1/ DL4 | No for PC3 |
| n40 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n41 | PC3 | n39 | >1GHz and <3 GHz | UL3/DL4 | No for all power classes |
| n41 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n46 | PC5 | n7 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n2 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n25 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC2/PC1.5 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n77 | PC2/PC1.5 | n5 | <1GHz | UL1/DL4 | Yes for PC2/PC1.5 |
| n77 | PC3 | n8 | <1GHz | UL1/DL4 | No for PC3 |
| n77 | PC3 | n70 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n78 | PC2 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n78 | PC2 | n8 | <1GHz | UL1/DL4 | Yes for PC2 |

## 2.2 Proposed template covering MSDs for 1UL(1CC) and IMDs for 2UL(1CC per band)

Based on the discussion above and questions raised about which cases should be investigate for harmonic mixing and cross band isolation, we would like to discuss an update of tabular calculations used for MSD studies covering UL harmonics, Harmonic mixing and cross-band isolation MSDs for 1UL(1CC) and IMDs for 2UL(1CC per band). This update can be built on top of the already well-used IMD calculation table in TP to TR for 2 and 3 band DL with 2 band UL and 1CC per UL band. For the two band DL and 1UL, 1CC case, additional rows are added to provide calculations for harmonic ranges, harmonic mixing ranges and cross-band ACLR ranges.

A preliminary version of such update is presented in Table 2 with different sections across rows with, in top to bottom order:

- The input frequencies and maximum UL channel BW section provide:

  - The two band names and types

  - The two bands UL frequencies

  - The two bands DL frequencies

  - The maximum UL channel bandwidth per band

  - A preliminary input on near miss ranges but that still requires more work but uses the maximum UL channel bandwidth per band.

- The UL harmonics frequencies section is to be used for 2DL band TP only for 1UL harmonic calculations provides the H2 to H5 harmonic ranges

- The DL harmonics frequencies section is to be used for 2DL band TP only for 1UL harmonic mixing calculations and provides the H2 to H5 harmonic ranges

- The Cross band ACLR range section is to be used for 2DL band TP only for cross band calculations and provides the ACLR1 to 4 based on the maximum UL channel bandwidth per band. Note that this approach may also be applicable to intra-band CA cases and a specific template developed.

- Then the usual IMD frequency range calculations section to be used for 2 and 3 DL bands with 2UL bands with 1CC per band TPs.

- Additionally, a notes section is provided for guidance on which cases should be investigated and other pertinent details. This part also requires additional work to integrate as many guidance and rules as possible.

- This table is populated with the CA_n18-n77 case as an example.

Table 2: Template for 1UL(1CC) and 2UL(1CC per band) frequency analysis

| UE carriers |  | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- | --- |
| Band number / type |  | n18 | FDD | n77 | TDD |
| Input frequencies2 | UL | 815 | 830 | 3400 | 4200 |
|  | DL | 860 | 875 | 3400 | 4200 |
|  | near miss | 760 | 975 | 3385 | 4215 |
| Maximum UL CBW |  | 15 |  | 100 |  |
| UL harmonics frequencies | H2 | 1630 | 1660 | 6800 | 8400 |
|  | H3 | 2445 | 2490 | 10200 | 12600 |
|  | H4 | 3260 | 3320 | 13600 | 16800 |
|  | H5 | 4075 | 4150 | 17000 | 21000 |
| DL harmonics frequencies | H23 | 1720 | 1750 | 6800 | 8400 |
|  | H34 | 2580 | 2625 | 10200 | 12600 |
|  | H45 | 3440 | 3500 | 13600 | 16800 |
|  | H56 | 4300 | 4375 | 17000 | 21000 |
| Cross band ACLR range | ACLR1 | 845 | 890 | 3300 | 4300 |
|  | ACLR2 | 830 | 905 | 3200 | 4400 |
|  | ACLR3 | 815 | 920 | 3100 | 4500 |
|  | ACLR47 | 800 | 935 | 3000 | 4600 |
| 2nd order IMD8 | products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
|  | frequency | 2570 | 3385 | 4215 | 5030 |
| 3rd order IMD8 | products | ¦f2_low – 2*f1_high¦ | ¦f2_high – 2*f1_low¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
|  | frequency | 1740 | 2570 | 5970 | 7585 |
|  | products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
|  | frequency | 5030 | 5860 | 7615 | 9230 |
| 4th order IMD8 | products | ¦3*f1_low – f2_high¦ | ¦3*f1_high – f2_low¦ | ¦3*f2_low – f1_high¦ | ¦3*f2_high –f1_low¦ |
|  | frequency | 910 | 1755 | 9370 | 11785 |
|  | products | ¦3*f1_low +f2_low¦ | ¦3*f1_high + f2_high¦ | ¦3*f2_low+f1_low¦ | ¦3*f2_high +f1_high¦ |
|  | frequency | 5845 | 6690 | 11015 | 13430 |
|  | products | ¦2*f1_low –2*f2_high¦ | ¦2*f1_high –2*f2_low¦ | ¦2*f1_low +2*f2_low¦ | ¦2*f1_high +2*f2_high¦ |
|  | frequency | 5140 | 6770 | 8430 | 10060 |
| 5th order IMD8 | products | ¦f1_low –4*f2_high¦ | ¦f1_high –4*f2_low¦ | ¦f2_low –4*f1_high¦ | ¦f2_high –4*f1_low¦ |
|  | frequency | 12770 | 15985 | 80 | 940 |
|  | products | ¦f1_low +4*f2_low¦ | ¦f1_high +4*f2_high¦ | ¦f2_low+4*f1_low¦ | ¦f2_high +4*f1_high¦ |
|  | frequency | 14415 | 17630 | 6660 | 7520 |
|  | products | ¦2*f1_low –3*f2_high¦ | ¦2*f1_high –3*f2_low¦ | ¦2*f2_low -3*f1_high¦ | ¦2*f2_high -3*f1_low¦ |
|  | frequency | 8540 | 10970 | 4310 | 5955 |
|  | products | ¦2*f1_low +3*f2_low¦ | ¦2*f1_high +3*f2_high¦ | ¦2*f2_low+3*f1_low¦ | ¦2*f2_high +3*f1_high¦ |
|  | frequency | 11830 | 14260 | 9245 | 10890 |
| Note 1: All frequencies are in MHz |  |  |  |  |  |
| Note 2: Region or country based frequency restriction can be applied if the band combination is identifiable to a single region or country |  |  |  |  |  |
| Note 3: UL1/DL2 harmonic mixing collisions should be investigated for DL bands > 3GHz, UL3/DL2 may be considered for DL bands >5GHz |  |  |  |  |  |
| Note 4: UL1or2or3/DL3 harmonic mixing collisions should be investigated |  |  |  |  |  |
| Note 5: UL1/DL4 harmonic mixing collisions should be investigated for DL bands > 1GHz and UL2or3/DL4 for bands > 5GHz |  |  |  |  |  |
| Note 6: UL1or2/DL5 harmonic mixing collisions should be investigated |  |  |  |  |  |
| Note 7: In the case were ACLR3 range does not overlap with the other band DL range, there could still be a residual MSD due to transmitter noise floor at low filter rejection |  |  |  |  |  |
| Note 8: IMD section is needed for 2UL case only, only the lowest even and odd order overlapping a DL band should be specified |  |  |  |  |  |

To ease a systematic review of the potential MSD cases, another table is added to the TP where each MSD source is discussed and the possible need for specification is reported. Table 3 is a preliminary version populated with the CA_n18-n77 MSD sources analysis.

Table 3: Template for 1UL(1CC) and 2UL(1CC per band) MSD analysis

| 1UL harmonics | UL4 and UL5 of band n18 fall into DL1 of band n77 |
| --- | --- |
| 1UL Harmonic mixing | DL4 of n18 overlaps with UL1 of n77, this can be ignored as it a DL4 of a band <1GHz |
| 1UL cross band | no cross band range overlaps with the other DL band and the distance is large so no noise floor related MSD should be investigated |
| 2UL IMDs | IMD2 falls in band n77 DL but can be ignored as it is a TDD band IMD4 falls in band n18 DL and should be specified IMD5 falls in band n18 DL and should be specified |
| Note 1: for MSD analysis, any harmonic or IMD product overlapping a DL band or DL harmonic overlapping an UL band or its harmonics should be investigated and commented in this Table. For cases where there is no overlap but the product falls within the maximum UL CBW distance of a DL band, near miss MSD should be considered for low order products. |  |

Proposal: The 2 band and 3 band DL TP to TR templates are updated to add the 1UL harmonic, harmonic mixing and cross-band isolation calculations tables with Notes for guidance and associated MSD analysis reporting table. Example tables provided in the contribution can be used as a starting point.

# Conclusion

In this contribution, we discussed the question raised about the need to specify UL1/DL4 harmonic mixing MSD and make the following proposal for even harmonic mixing cases in general.

Proposal: Even harmonic mixing MSD should be investigated and specified if necessary, for:

- UL1/DL2 for all FR1 DL bands and all power classes

- UL1/DL4 for all FR1 DL bands for PC2 and PC1.5 and for DL bands >1GHz for PC3

- UL3/DL2 for all FR1 DL bands >3GHz for PC3 and >1GHz for PC2 and PC1.5

- UL3/DL4 for all FR1 DL bands >5GHz for PC5 and PC3 and >3GHz for PC2 and PC1.5.

Proposal on correction of current harmonic mixing cases in 38.101-1 in the last column of the table below:

| UL band | Power Class | DL band | DL band range | UL/DL collision | Case specified |
| --- | --- | --- | --- | --- | --- |
| n3 | PC3 | n26 | <1GHz | UL1/DL2 | Yes for all power classes |
| n7 | PC3 | n71 | <1GHz | UL1/ DL4 | No for PC3 |
| n40 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n41 | PC3 | n39 | >1GHz and <3 GHz | UL3/DL4 | No for all power classes |
| n41 | PC3 | n77/78 | >3GHz | UL3/DL2 | Yes for all power classes |
| n46 | PC5 | n7 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n2 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC3/2/1.5 | n25 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n77 | PC2/PC1.5 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n77 | PC2/PC1.5 | n5 | <1GHz | UL1/DL4 | Yes for PC2/PC1.5 |
| n77 | PC3 | n8 | <1GHz | UL1/DL4 | No for PC3 |
| n77 | PC3 | n70 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes |
| n78 | PC2 | n3 | >1GHz and <3 GHz | UL1/DL2 | Yes for all power classes (PC3 missing?) |
| n78 | PC2 | n8 | <1GHz | UL1/DL4 | Yes for PC2 |

We also looked into augmenting the coverage of calculation tables for 1UL and 2UL MSDs for the TP to TR templates with the following proposal based on preliminary templates shown in this contribution.

Proposal: The 2 band and 3 band DL TP to TR templates are updated to add the 1UL harmonic, harmonic mixing and cross-band isolation calculations tables with Notes for guidance and associated MSD analysis reporting table. Example tables provided in the contribution can be used as a starting point.

# References