3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316869

Xiamen, China, 9th Oct. – 13th Oct. 2023

Agenda Item:	4.1.1.2

Source:	Charter Communications, Inc.

Title:	A-MPR tables for CA_n96 NR-U CA_NS_53 and CA_NS_54

Document for:	Approval

# Introduction

In RAN4#106 meeting, a WF for introducing Carrier aggregation to Power class 5 (PC5) in NR-unlicensed spectrum for was approved [1].

“In Release 17 the CA_n96B/C UL CA and configurations are specified. Even so, it was clarified in RAN that although NR-U ULCA requirements were completed, NR-U ULCA could not be deployed since the A-MPR for n96B/C UL was not specified, especially for the U.S. NS_53 and NS_54.” [1]

Then in RAN4#108, two papers were submitted with simulation results [4] and [5].

From Charter’s paper [4], we got the following proposed tables: Table 1: A-MPR for NS_53 power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B Pre-coding; Modulation; Channel bandwidth (Sub-band allocation) / RB Allocation. 40 MHz; 60 MHz; 80 MHz; 100MHz. Full (dB); Partial (dB); Full (dB); Partial (dB); Full (dB); Partial (dB); Full (dB); Partial (dB). DFT-s-OFDM; PI/2 BPSK2; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [4.5]; ≤ [5.5]; ≤ [3.0]; ≤ [5.5]. QPSK; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [4.5]; ≤ [5.5]; ≤ [3.0]; ≤ [5.5]. 16 QAM; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [4.5]; ≤ [5.5]; ≤ [4.0]; ≤ [5.5]. 64 QAM; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [5.5]; ≤ [6.0]; ≤ [5.0]; ≤ [5.5]. 256 QAM; ≤ [6.5]; ≤ [9.0]; ≤ [5.5]; ≤ [7.0]; ≤ [5.5]; ≤ [6.0]; ≤ [5.5]; ≤ [7.0]. CP-OFDM; QPSK; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [4.5]; ≤ [5.5]; ≤ [5.0]; ≤ [5.5]. 16 QAM; ≤ [6.5]; ≤ [9.0]; ≤ [4.5]; ≤ [7.0]; ≤ [5.0]; ≤ [5.5]; ≤ [5.0]; ≤ [5.5]. 64 QAM; ≤ [6.5]; ≤ [9.0]; ≤ [5.5]; ≤ [7.0]; ≤ [6.0]; ≤ [6.0]; ≤ [5.5]; ≤ [6.0]. 256 QAM; ≤ [7.0]; ≤ [9.0]; ≤ [7.0]; ≤ [7.0]; ≤ [7.0]; ≤ [7.0]; ≤ [7.0]; ≤ [7.0]. NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0.. Table 2: A-MPR for NS_54 power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B Pre-coding; Modulation; RB Allocation (Note 2); RB Allocation (Note 3). Full/Partial; Full (dB); Partial (dB). DFT-s-OFDM; PI/2 BPSK4; See Table 6.2F.2A.2-1; ≤ [4.5]; ≤ [5.0]. QPSK; ≤ [4.5]; ≤ [5.5]. 16 QAM; ≤ [5.0]; ≤ [5.5]. 64 QAM; ≤ [5.0]; ≤ [5.5]. 256 QAM; ≤ [6.0]; ≤ [6.0]. CP-OFDM; QPSK; ≤ [5.5]; ≤ [6.0]. 16 QAM; ≤ [5.5]; ≤ [6.0]. 64 QAM; ≤ [6.0]; ≤ [6.5]. 256 QAM; ≤ [7.0]; ≤ [7.0]. NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz], and 100 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0.. From Apple’s paper [5], we got the following proposed tables: Table 3: A-MPR for NS_53 power class 5 Pre-coding; Number of active 20MHz sub-bands; Modulation / RB Allocation. PI/2 BPSK2; QPSK; 16 QAM; 64 QAM; 256 QAM. Full (dB); Partial (dB); Full (dB); Partial (dB); Full (dB); Partial (dB); Full (dB); Partial (dB); Full (dB); Partial (dB). DFT-s-OFDM; 1; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0. 2; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5. 3; ≤ 4.5; ≤ 6.5; ≤ 4.5; ≤ 6.5; ≤ 4.5; ≤ 6.5; ≤ 4.5; ≤ 6.5; ≤ 6.0; ≤ 6.5. 4; ≤ 4.0; ≤ 5.5; ≤ 4.0; ≤ 5.5; ≤ 4.5; ≤ 5.5; ≤ 5.0; ≤ 5.5; ≤ 6.0; ≤ 6.0. 5; ≤ 4.0; ≤ 4.5; ≤ 4.0; ≤ 4.5; ≤ 4.5; ≤ 4.5; ≤ 5.0; ≤ 5.0; ≤ 6.0; ≤ 6.0. 6; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.5; ≤ 4.5; ≤ 5.0; ≤ 5.0; ≤ 6.0; ≤ 6.0. 7; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.5; ≤ 4.5; ≤ 5.0; ≤ 5.0; ≤ 6.0; ≤ 6.0. 8; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.0; ≤ 4.5; ≤ 4.5; ≤ 5.0; ≤ 5.0; ≤ 6.0; ≤ 6.0. CP-OFDM; 1; N/A; N/A; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0; ≤ 9.0; ≤ 12.0. 2; N/A; N/A; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5; ≤ 6.5; ≤ 8.5; ≤ 7.0; ≤ 8.5. 3; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. 4; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. 5; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. 6; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. 7; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. 8; N/A; N/A; ≤ 5.5; ≤ 6.5; ≤ 6.0; ≤ 6.5; ≤ 6.5; ≤ 6.5; ≤ 7.0; ≤ 7.0. NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0.. Table 4: A-MPR for NS_54 power class 5 Pre-coding; Modulation; RB Allocation (Note 2); RB Allocation (Note 3). Full/Partial; Full (dB); Partial (dB). DFT-s-OFDM; PI/2 BPSK4; See Table 6.2F.2-1; ≤ 3.5; ≤ 5.0. QPSK; ≤ 3.5; ≤ 5.0. 16 QAM; ≤ 3.5; ≤ 5.0. 64 QAM; ≤ 4.5; ≤ 5.0. 256 QAM; ≤ 6.0; ≤ 6.0. CP-OFDM; QPSK; ≤ 4.5; ≤ 6.0. 16 QAM; ≤ 4.5; ≤ 6.0. 64 QAM; ≤ 6.0; ≤ 6.5. 256 QAM; ≤ 7.0; ≤ 7.0. NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0..

# Discussion

In this paper, we are proposing the following two merged A-MPR tables, where the two companies results have been merged using the mean average and then rounded upward to the closed x.5 or x.0 backoff value.

Table 5: A-MPR for NS_53 power class 5

| Pre-coding | Modulation | Aggregated channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  | 120MHz, 140MHz, 160MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.5] | ≤ [5.5] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] | ≤ [4.0] |
|  | QPSK | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.5] | ≤ [5.5] | ≤ [4.0] | ≤ [4.5] | ≤ [4.0] | ≤ [4.0] |
|  | 16 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.5] | ≤ [5.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] | ≤ [4.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [5.5] | ≤ [6.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] | ≤ [5.0] |
|  | 256 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [6.0] | ≤ [7.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] | ≤ [6.0] |
| CP-OFDM | QPSK | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [5.0] | ≤ [6.0] | ≤ [5.5] | ≤ [6.5] | ≤ [5.5] | ≤ [6.5] |
|  | 16 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [6.0] | ≤ [7.0] | ≤ [5.5] | ≤ [6.0] | ≤ [6.0] | ≤ [6.5] | ≤ [6.0] | ≤ [6.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [9.0] | ≤ [6.0] | ≤ [7.0] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] | ≤ [6.5] |
|  | 256 QAM | ≤ [9.0] | ≤ [12.0] | ≤ [7.0] | ≤ [9.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 1: Consider Table 5 when specifying ULCA A-MPR for NS_53.

Table 6: A-MPR for NS_54 power class 5

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table 6.2F.2A.2-1 | ≤ [3.5] | ≤ [5.0] |
|  | QPSK |  | ≤ [3.5] | ≤ [5.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] |
|  | 64 QAM |  | ≤ [4.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [6.0] | ≤ [6.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] |
|  | 16 QAM |  | ≤ [4.5] | ≤ [6.0] |
|  | 64 QAM |  | ≤ [6.0] | ≤ [6.5] |
|  | 256 QAM |  | ≤ [7.0] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz aggregated channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz aggregated channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz aggregated channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

Proposal 2: Consider Table 6 when specifying ULCA A-MPR for NS_54.

# Conclusion

In this paper, we at Charter are proposing following new A-MPR tables for shared spectrum access.

Proposal 1: Consider Table 5 when specifying ULCA A-MPR for NS_53.

Proposal 2: Consider Table 6 when specifying ULCA A-MPR for NS_54.

# Reference

[1] R4-2303484, WF on NR-U enhancement, Apple

[2] R4-2306481, WF on NR-U contiguous ULCA A-MPR, Charter Communications, Inc., Apple

[3] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.8.0 (2022-12)

[4] R4-2313823, Simulation results on UE RF NR-U UL CA A-MPR for PC5, Charter Communications Inc

[5] R4-2311250, On NR-U ULCA A-MPR for NS_53 and NS_54, Apple Inc