3GPP TSG-RAN WG4 Meeting #108bis	R4-2316902

Xiamen, China, October 09 – October 13, 2023

Title: 	Offline Minutes for [108bis][315] 256 QAM BS Demod (NR_RF_FR2_req_Ph3_Demod)

Agenda Item:	5.6.4.1

Source: 	Moderator (Nokia, Nokia Shanghai Bell)

Document for:	Information

# 1 PUSCH requirements

### Sub-topic 1-1: Tx EVM

Issue 1-1: Tx EVM

- Agreements from RAN4#108 [R4-2313946]

  - Issue 2-1: EVM Impact

    - Agreement: The impact of EVM can be considered in impairment results, and interested companies are encouraged to provide ideal simulations with and without TxEVM for alignment at RAN4#108bis, TxEVM value to be 3.5%.

    - Note: Test requirement parameters do not capture TxEVM and TEs don’t add additional TxEVM during test.

- Proposals:

  - Option 1: Companies use no Tx EVM results for alignment (Ericsson, Keysight)

  - Option 2: FFS on Tx EVM modelling (Samsung)

- Recommended WF

  - For discussion at meeting.
Note that several companies (Eri, Nok, HW, Xiaomi) have provided simulation results with and without TxEVM and have observed minimal EVM impact.

Offline Tentative Agreement: Option 1 can be agreed, impact for TxEVM can be included in impairment result.

### Sub-topic 1-2: SNR Limit

Issue 1-2: SNR Limit

- Agreements from RAN4#108 [R4-2313946]

  - Issue 2-2: SNR Limit

  - RAN4 will attempt to keep simulation parameters to reach an SNR operating point less than 20dB, but will await simulation results from interested companies at RAN4#108-bis.

  - Request input from TE vendor on maximum testable SNR at RAN4#108bis

- Proposals:

  - Option 1: RAN4 will reuse existing limit value of 20dB SNR for UL256QAM demod testing. (Keysight)

- Recommended WF

  - Agree Option 1.

Offline Tentative Agreement: Agree Option 1, if impairment result for MCS 20 is over 20dB check with TE vendors if this is testable. Request companies to include results with impairment for next meeting.

  - Note: Confirm with TE vendors that if 20dB is exceeded with impairment results whether this is feasible.

### Sub-topic 1-3: Phase Noise Model

Issue 1-3: Tx Phase Noise Model

- Proposals:

  - Option 1: Include Tx phase noise impact

    - Option 1a: Specify model ()

    - Option 1b: Do not explicitly specify model ()

  - Option 2: Do not consider Tx phase noise model, only in impairment factor ()

  - Option 3: Do not consider Tx phase noise impact (Ericsson, Samsung, Nokia, Huawei, ZTE)

- Recommended WF:

  - For Discussion at meeting

Offline Tentative Agreement: Do not consider Tx phase noise model or impact for requirement derivation.

Issue 1-3a: Rx Phase Noise Model

- Proposals:

  - Option 1: Include Rx phase noise impact

    - Option 1a: Specify model (ZTE)

    - Option 1b: Do not explicitly specify model (Huawei)

  - Option 2: Do not specify Rx phase noise model but consider Rx phase noise impact for impaired results (Ericsson, Samsung, Nokia, Huawei)

  - Option 3: Do not consider Rx phase noise impact ()

- Recommended WF:

  - For Discussion at meeting

Offline Tentative Agreement: Do not specify Rx phase noise model but consider Rx phase noise impact for impaired results.

### Sub-topic 1-4: Channel

Issue 1-4: Channel

- Proposals:

  - Option 1: TDLD 30-35 (Ericsson, Nokia, Xiaomi, Huawei)

- Recommended WF

  - Proposed agreement: TDLD 30-35.

Offline Tentative Agreement: TDLD 30-35

Note: Review channel if impaired results create testability issues.

### Sub-topic 1-5: Antenna Configuration

Issue 1-5: Antenna Configuration

- Proposals:

  - Option 1: 1T2R (Ericsson, Nokia, ZTE, Samsung, Xiaomi)

    - Option 1a: FFS on other number of Rx (ZTE)

- Recommended WF

  - Proposed agreement: 1T2R as baseline, FFS on other number of Rx.

Offline Tentative Agreement: 1T2R

### Sub-topic 1-6: Carrier BW and SCS

Issue 1-6: 60 kHz SCS and corresponding carrier BW

- Proposals:

  - Option 1: Have 60kHz with 50MHz CBW (Ericsson, Samsung, Nokia, Xiaomi)

  - Option 2: Do not define 60kHz requirements (Huawei, Samsung)

  - Option 3: SCS and related channel bandwidths should be chosen based on operators’ request (Xiaomi)

- Recommended WF

  - For discussion at meeting, if agreed to proceed with 60 kHz SCS, 50 MHz can be agreed.

Offline Tentative Agreement: Companies to use 50 MHz for 60 kHz, however decision whether to include requirements for SCS of 60 kHz to be deferred to RAN4#109.

Issue 1-7: 120 kHz SCS and corresponding carrier BW

- Proposals:

  - Option 1: 50MHz (Ericsson, Samsung, Nokia)

  - Option 2: 100MHz

  - Option 3: 50, 200 MHz (Huawei, Nokia, Xiaomi)

  - Option 4: 50, 100, 200 MHz (Xiaomi)

  - Option 4a: 100, 200 MHz (ZTE)

  - Option 5: SCS and related channel bandwidths should be chosen based on operators’ request. (Xiaomi)

- Recommended WF

  - Proposed agreement: Have 120kHZ SCS requirements.
CBW for discussion at meeting.

Offline Tentative Agreement:

Companies to use 50 MHz initially for requirements derivation, with FFS on 100 MHz and 200 MHz.

Companies to provide simulations for 50, 100, 200 with a priority in order of 50, 200, 100, with a view on testable SNR.

Note: Could use existing applicability rule to extend 50 MHz to other CBW.

### Sub-topic 1-7: DMRS

Issue 1-8: DMRS

- Proposals:

  - Option 1: 1+1 (Ericsson, Samsung)

    - Option 1a: 1+1, FFS on 1+0 (Samsung)

  - Option 2: Both 1+1 and 1+0, cases selected for tests to be based on BS Manufacture Declaration and applicability rule (Huawei, Nokia, Samsung, Ericsson, ZTE)

  - Option 3: Choose a single DMRS configuration between 1+0 and 1+1 (ZTE)

  - Option 4: 1+0 (Nokia)

- Recommended WF:

  - For Discussion at meeting

Offline Tentative Agreement: Both 1+1 and 1+0, cases selected for tests to be based on BS Manufacture Declaration and applicability rule

### Sub-topic 1-8: PTRS

Issue 1-9: PTRS

- Agreements from RAN4#108 [R4-2313946]

  - Issue 2-10: PTRS

    - Interested companies to provide simulation results with both enabled and disable for RAN4#108-bis.

    - Configuration: K_PTRS: 2 and L_PTRS =1

- Proposals:

  - Option 1: Do not use PTRS (ZTE)

  - Option 2: Use PTRS (Nokia, Xiaomi, Samsung, ZTE)

  - Option 3: Define with both, cases selected for tests to be based on BS Manufacture Declaration and applicability rule (Huawei)

- Recommended WF

  - For Discussion at meeting

Offline Tentative Agreement: Tentatively Agree Option 2, Huawei to check before online session

### Sub-topic 1-9: MCS

Issue 1-10: MCS

- Proposals:

  - Option 1: MCS 20 (Huawei, Ericsson, Nokia, ZTE, Xiaomi, Samsung)

  - Option 2: Initially focus on MCS 22 (Xiaomi)

- Recommended WF

  - For Discussion at meeting

Offline Tentative Agreement: MCS 20

# Topic #2: Work Split

## Open issues summary

### Sub-topic 2-1: Work Split

Issue 2-1: Work Plan

- Proposals

| Task ID |  | Responsible Company |
| --- | --- | --- |
| Simulation result summary for 256 QAM BS Demodulation |  | Huawei |
| CR split |  |  |
| Specification | Content part | Responsible Company |
| 38.104 | PUSCH requirement | Nokia |
| 38.141-2 | PUSCH requirement | Ericsson |
| 38.104, 38.141-2, 38.141-1 (for alignment) | FRC tables | ZTE |
| 38.104 | Applicability rule, manufactory declaration, measurement uncertainty and test tolerance, delay profiles for FR2 | Huawei |
| 38.141-2 | Applicability rule, manufactory declaration, measurement uncertainty and test tolerance, delay profiles for FR2 | Samsung |

- Recommended WF

  - Work split for demodulation to be agreed at RAN4#108bis.

## A.4 Reference

[1] R4-2317946, Topic summary for [108][321] NR_RF_FR2_req_Ph3_Demod, Nokia, Nokia Shanghai Bell, RAN WG4 #108bis