3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316915

Xiamen, China, October 09 – October 13, 2023

Source:	China Telecom

Title:	WF on the advanced receiver for MU-MIMO scenario

Agenda Item:	5.18.4

Document for:	Approval

Introduction

This document provides the way forward on advanced receiver for MU-MIMO scenario of Rel-18 NR demodulation requirement evolution WI.

The previous approved WFs are listed as below:

  - R4-2302929 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106

  - R4-2305914 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#106bis-e

  - R4-2309892 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#107

  - R4-2313993 - WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4#108

<Topic #1: Assumptions for the advanced receiver for MU-MIMO>

Sub-topic 1-2 Reference receiver assumptions

Issue 1-2-1: Selection of reference receiver

  - Not to revisit the previous decision on the selection of the reference receiver.

  - Discuss the detailed test parameters and test case design in the test setup part.

Issue 1-2-2: Additional assumptions to the R-ML receiver

- Candidate options on maximum number of layers need to be handled with R-ML receiver:

  - Option 1: Different types of UEs that defines the minimum total layer number across target and co-scheduled UEs with R-ML processing based on UE declaration

    - Option 1A:

- Type 1: R-ML with enhanced inter-stream interference suppression for MU-MIMO transmissions with rank 2 with 2 Rx

- Type 2: R-ML with enhanced inter-stream interference suppression for MU-MIMO transmissions with rank 2,3,4 with 4 Rx

    - Option 1B:

- For R-ML receiver without modulation order detection for MU-MIMO

- Type 1: 2Rx UEs can process up to 2 layers across target and co-scheduled UEs with R-ML receiver

- Type 2: 4Rx UEs can process up to 4 layers across target and co-scheduled UEs with R-ML receiver

- For R-ML receiver with modulation order detection for MU-MIMO

- Type 1: 2Rx UEs which can process up to 2 layers across target and co-scheduled UEs with R-ML receiver

- Type 2: 4Rx UEs which can process up to 2 layers across target and co-scheduled UEs with R-ML receiver

- Type 3: 4Rx UEs which can process up to 4 layers across target and co-scheduled UEs with R-ML receiver

  - Option 2: Introduce UE capability signalling for the following types

    - Type 1: 2Rx UEs which can process up to 2 layers across target and co-scheduled UEs with R-ML receiver

    - Type 2: 4Rx UEs which can process up to 2 layers across target and co-scheduled UEs with R-ML receiver

    - Type 3: 4Rx UEs which can process up to 4 layers across target and co-scheduled UEs with R-ML receiver

  - Option 3: Maximum 4 layer including target and co-scheduled UEs are required. When the assumptions are not fulfilled, UE is allowed to fall back to MMSE-IRC requirements

- Candidate options on supported DMRS configurations:

  - Option 1: Not to have additional restrictions on supported DMRS configurations

  - Option 2: Restrict R-ML for MU-MIMO to certain DMRS configuration and length or introduce UE capability on the supported DMRS configuration and lengths

Additional assumptions to the R-ML receiver

- If the target UE does not support Rel-18 DMRS. It is up to UE implementation whether the UE will perform R-ML to the co-scheduled layers associate with the DMRS ports which are not included in the Rel-15 DMRS.

Sub-topic 1-3 Discussion on the required information

Issue 1-3-1: The DMRS port information for the co-scheduled UE

- Candidate options on additional RRC based assistant signalling:

  - Option 1: No need to consider additional RRC signaling for DMRS port

  - Option 2: Introduce RRC signaling for upper bound on number of co-scheduled UE ports

  - Option 3: Introduce RRC signalling to indicate whether there is UE with Rel-18 DMRS configuration in the whole cell existing

Issue 1-3-2: Frequency domain resource allocation type for the co-UE and the target UE

- Candidate options:

  - Option 1: Introduce default assumption for resource allocation type for co-UE same as targe UE. Introduce dedicated RRC signaling to indicate if the default assumption is true or false

  - Option 2: Not to have the assumption on the frequency domain resource allocation type for the co-scheduled UE

Issue 1-3-3: Additional evaluation on modulation order blind detection

- Candidate options on additional RAN4 default assumptions to assist modulation order blind detection:

  - Proposal 1: RAN4 to consider default assumption of only type 1 FDRA allocation of co-UEs, and Further evaluate if UE blind MO detection capability can be extended to include

    - UE capable of blind MO detection with granularity of PRG =2/4

    - UEs capable of blind MO detection within each type 1 FDRA allocation.

    - UEs capable of single blind MO detection per layer.

    - UEs capable of only one blind MO detection across all layers in a slot.

Issue 1-3-4: New MAC-CE command to assist DMRS port blind detection

- Candidate options:

  - Option 1: Introduce the following new MAC-CE command to assist DMRS port blind detection

| New MAC-CE Command | Content |
| --- | --- |
| Joint signal power detection across multiple PRBs/PRGs with respect to one DMRS port | 1 bit: Target UE apply joint signal power detection across multiple PRBs/PRGs with respect to one DMRS port; 3 bits: Valid period for UE to apply joint signal power detection across multiple PRBs/PRGs with respect to one DMRS port. 2~16 ms |

    - Sub-topic 1-4 UE capability aspects

Issue 1-4-1: Capability signalling for advanced receiver for MU-MIMO

- UE advanced receiver to cancel inter-user interference for MU-MIMO is an optional feature with UE capability signalling

- Candidate options on capability definition for R-ML with modulation order blind detection:

  - Option 1: Blind modulation order detection is based on UE capability signaling

    - Option 1A: Define different capability in the scenarios indicated by DCI index 6 and 7 respectively

    - Option 1B: Introduce 3 level UE capabilities: 1) Low-end UE: Support DCI 0-5; 2) Medium-end UE supporting DCI 0-6; 3) High-end UE supporting DCI 0-7

  - Option 2: Blind modulation order detection is based on UE declaration

- Candidate options on capability definition for Maximum number of layers:

  - Option 1: Introduce UE capability for Maximum number of layers of co-UE or total number of layers for joint detection

  - Option 2: Not to introduce such capability definition

    - Option 2A: The maximum number of layers of co-UE can be derived by subtracting the scheduled MIMO layers for the target UE from maxNumberMIMO-LayersPDSCH

- Candidate options on capability definition for Maximum number of DMRS ports:

  - Option 1: Introduce UE capability signalling for maximum DMRS ports to be detected

  - Option 2: Not to introduce such capability definition

- Candidate options on capability definition for Maximum modulation orders of interfering DMRS ports supported:

  - Option 1: UE capability signaling to inform network of the maximum modulation orders of interfering DMRS port supported

  - Option 2: Not to introduce such capability definition

Issue 1-4-2: Capability granularity and details for the R-ML capability signalling

- Candidate options:

  - Option 1: Align with the Rel-17 MMSE-IRC for MU-MIMO, i.e., per UE, no FDD/TDD difference, FR1 only

  - Option 2: Introduce per CC per band per band combination (Per-FSPC) UE capability

<Topic #2: Test parameters and simulation results >

Issue 2-1: Test cases for scenarios where R-ML receiver is not applicable

- Candidate options:

  - Option 1: Do not introduce test cases for scenarios where R-ML receiver is not applicable

  - Other options are not precluded

Issue 2-2: Test scope

- Candidate options:

  - Option 1: Reuse the same test scope for Rel-17 MMSE-IRC for MU-MIMO

- Both FDD 15kHz SCS with 10MHz CHBW and TDD 30kHz SCS with 40MHz CHBW

- 2Tx-2Rx with rank 1 for both target and co-scheduled UE on each PRB.

- 2Tx-4Rx with rank 1 for both target and co-scheduled UE on each PRB.

- 4Tx-4Rx, FFS the rank number for target and co-scheduled UE on each PRB.

  - Option 2: Reuse the same test scope for Rel-17 MMSE-IRC for MU-MIMO except for tests for 2Tx-4Rx

Issue 2-3: Co-scheduled UE number

- Candidate options:

  - For the cases without modulation order blind detection:

- Option 1: 1 co-scheduled UE

- Option 2: In addition to 1 co-scheduled UE, define performance requirements based on multiple co-UEs using the same modulation order

  - For the cases with modulation order blind detection:

- Option 1: Model 2 co-scheduled UEs with different modulation orders and different FDRA

- Option 2: In addition to 1 co-scheduled UE, define performance requirements based on multiple co-UEs using the same modulation order

- Option 3: 1 co-scheduled UE

Issue 2-4: Frequency domain resource allocation

- Candidate options:

  - Option 1: Define requirements with full CHBW FDRA co-scheduled UE only

  - Option 2: Cover both full and partial CHBW resource allocation, and full CHBW resource allocation for the target UE

Issue 2-5: Test setting for the RAN4 agreed network default assumptions

  - For phase II tests, all the RAN4 agreed network default assumptions should be valid

  - FFS on the detailed RRC configuration details pending decisions on the signalling design

Issue 2-6: MCS Table

- Candidate options on the RRC assistant information configuration on the MCS table:

  - Option 1:

    - For the cases without modulation order blind detection (UE informed DCI index 1-5), no need for the network to inform such information to the UE

    - For the cases with modulation order blind detection (UE informed DCI index 6), FFS the RRC signaling configuration details after decisions are made

  - Other options are not precluded.

- Candidate options on MCS Table1 for the test configuration:

  - Option 1: The maximum MCS table is 256QAM or 64QAM MCS table, i.e., 1024QAM is not covered

  - Option 2: Use MCS Table1

Issue 2-7: Precoder selection for co-scheduled UE

- Candidate options

  - Option 1: Only consider orthogonal PMI selection with the target UE

  - Option 2: Use the randomized precoder for co-scheduled UE which is not equal to any column of the precoder matrix of target UE

  - Option 3: consider both random PMI and orthogonal PMI

    - Option 3A: Consider random PMI selection for rank 1+1, and consider orthogonal PMI selection for rank 2+2

Issue 2-8: Test setting for UEs not supporting modulation order blind detection

- Candidate options on Test with DCI index 1-5 configured (Tests #1-1):

  - Option 1: Define Tests #1-1 with 1 co-scheduled UE and full FDRA

  - Option 2: In addition to the Tests with 1 co-UE, consider cases with 2 co-UEs having same modulation order

- Candidate options on Test with DCI index 6 configured (Tests #1-2):

  - Option 1: In addition to Tests #1-1, define Tests #1-2 to verify UE E-IRC receiving process under the same test parameters with Tests #1-1

  - Option 2: Do not introduce test cases for scenarios where R-ML receiver is not applicable

Issue 2-9: Test setting for UEs supporting modulation order blind detection

- Candidate options on Tests with DCI index 6 configured (Tests #2-2):

  - Option 1: Define Tests #2-2 to verify UE R-ML process with modulation order blind detection

- Option 1A: Model 2-co-scheduled UEs with different modulation order and different FDRA

- Option 1B: Follow test settings from test without modulation order blind detection except DCI signalling

- Option 1C: Model 1-co-scheduled UE with partial FDRA and single modulation order

- Option 1D: Only consider rank 1+1 with QPSK for the co-UE

- Candidate options on Test with DCI index 1-5 configured (Test #2-1):

  - Option 1: In addition to Tests #2-1, Define Tests #2-1 to verify UE R-ML receiving process with modulation order information with 1 co-scheduled UE and full FDRA

  - Other options are not precluded.

- Candidate options on Test with DCI index 7 configured (Test #2-3):

  - Option 1: Introducing tests for R-ML with modulation order blind detection, with DCI index 7

  - Other options are not precluded.

Issue 2-10: Modulation order for the co-scheduled UE

- Candidate options:

  - For the test cases without modulation order blind detection (for Tests#1-1 and Tests#2-1 if defined):

- Option 1: QPSK for rank 1+1, and 16QAM for rank 2+2 tests

- Option 2: QPSK for both rank 1+1 and rank 2+2 tests

- Option 3: 16QAM for both rank 1+1 and rank 2+2 tests

- Option 4: Cover both QPSK and 16QAM for rank 1+1, and QPSK for rank 2+2 tests

  - For the cases with modulation order blind detection (for Tests#1-2 and Tests#2-2 if defined):

- Option 1: Follow test settings from test without modulation order blind detection

- Option 2: Model 1 co-scheduled UEs with QPSK, for both rank 1+1 and rank 2+2 tests

- Option 3: QPSK only

- Option 4: Model 2 co-scheduled UEs with QPSK and 16QAM respectively, for both rank 1+1 and rank 2+2 tests

- Option 5:

- For rank 1+1: Co-scheduled UE1 with Partial CHBW allocation and QPSK, co-scheduled UE2 with Partial CHBW allocation and 16QAM

- For rank 2+2: Co-scheduled UE1 with Partial CHBW allocation and 16QAM, co-scheduled UE2 with Partial CHBW allocation and 64QAM

Issue 2-11: Detailed test parameters

- Candidate options on rank 1+1 tests with 2T2R:

  - Option 1

- Target MCS: 13 (Table 1)

- MIMO configuration: ULA medium

- Channel: TDLC300-100

  - Other options are not precluded.

- Candidate options on rank 1+1 tests with 2T4R:

  - Option 1

- Target MCS: 13 (Table 1)

- MIMO configuration: ULA Low

- Channel: TDLA30-10

  - Option 2

- MIMO configuration: ULA medium

- Channel: TDLC300-100

- Candidate options on rank 2+2 tests with 4T4R:

  - Option 1

- Target MCS: 17 (Table 1)

- MIMO configuration: 4T4R ULA Low

- Channel: TDLA30-10

  - Other options are not precluded.

Issue 2-12: Other parameters

- Candidate options:

  - Option 1: Reuse the phase I simulation assumptions as a start point

  - Other options not precluded