3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316920

Xiamen, China, October 09 ‒ October 13, 2023

Title: 	WF on NR_MIMO_evo_DL_UL_demod

Agenda Item:	5.29.5

Source: 	Samsung

Document for:	Approval

Topic #1 General Scope

Sub-topic 1-1 Scope of UE demodulation performance and CSI requirements

Issue 1-1-1: clarify if CSI requirements are needed for codebook enhancement for UE predicated PMI

Agreement:

- Study the feasibility of introducing PMI reporting requirements with ‘typeII-Doppler-r18’ (FR1 only).

      - FFS for test metric

      - FFS for simulation assumptions

Issue 1-1-2: clarify if demodulation or CSI requirements are needed for TDCP

Agreement:

- Do not introduce PDSCH demodulation requirements for TDCP measurement.

- FFS on CSI requirements for TDCP measurement

Issue 1-1-3: clarify performance impact for Unified TCI enhancement on multi-TRP

Agreement:

- No performance impact on UE demodulation and CSI reporting for Unified TCI enhancement on multi-TRP.

Issue 1-1-4: clarify if CSI requirements are needed for codebook enhancement for CJT

Agreement:

- Study the feasibility of introducing PMI reporting requirements with ‘typeII-CJT-r18’ (FR1 FDD only).

      - Focus on 2TRP only

      - FFS for test metric

      - FFS for simulation assumptions

Issue 1-1-5: clarify if demodulation requirements are needed for codebook enhancement for CJT

Agreement:

- No demodulation requirements for codebook enhancement for CJT are needed.

Issue 1-1-6: clarify if demodulation requirements are needed for increased number of orthogonal DMRS ports

Agreement:

- Introduce PDSCH demodulation requirements for Rel-18 enhanced DMRS for SU-MIMO scenario. Select limited case(s) from the legacy PDSCH cases to reuse the test configurations, but change the DRMS port configuration to configure the new defined DMRS ports for no more than 4 DMRS ports

      - FFS for applicability rule to be defined that UE can skip the legacy case if UE has passed the case with the same configuration but using the new DMRS ports

      - Companies are encouraged to propose the test case(s) to be reused.

Sub-topic 1-2 Scope of BS demodulation performance requirements

Issue 1-2-1: clarify if BS demodulation requirements are needed for unified TCI framework extension

Agreement:

- No BS demodulation requirements introduced for unified TCI framework extension

Issue 1-2-2: clarify if BS demodulation requirements are needed for two TAs enhancement

Agreement:

- No BS demodulation requirements introduced for two TAs enhancement

Issue 1-2-3: clarify if BS demodulation requirements are needed for increased number of orthogonal DMRS ports

Agreement:

- Introduce PUSCH demodulation requirements for Rel-18 enhanced DMRS ports. Select limited case(s) from legacy PUSCH cases to reuse the test configurations, but change the DRMS port configuration to configure the new defined DMRS ports for no more than 2 DMRS ports with introducing test applicability rule.

      - Applicability rule to be defined that BS can skip the legacy case if BS has passed the case with the same configuration but using new DMRS ports

      - Companies are encouraged to propose the test case(s) to be reused.

Issue 1-2-4: clarify if BS demodulation requirements are needed for Uplink 8Tx transmission in Rel-18

Agreement:

- Postpone the discussion on BS performance requirement introduction for Uplink 8Tx transmission to future release

Issue 1-2-5: clarify if BS demodulation requirements are needed for FR2 STxMP

Agreement:

- FFS on introducing BS demodulation requirements for FR2 STxMP

- Companies are encouraged to provide the view about testability of FR2 STxMP

Topic #2: Test set-up and simulation assumptions for UE demodulation performance and CSI

Sub-topic 2-1 Initial simulation assumptions for TypeII Doppler

Issue 2-1-1: Propagation channel

Way forward:

- For channel model: TDLA30

- For maximum Doppler frequency:

      - Option 1: 240Hz

      - Option 2: 100Hz

- Other options are not precluded

Issue 2-1-2: Correlation configurations

Way forward:

- Option 1: XP Medium as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-3: N1, N2, O1, O2 and the number of CSI-RS ports

Way forward:

- Option 1: Use the number of CSI-RS ports 16 with (N1, N2) = (4, 2), (O1, O2) = (4, 4) as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-4: paramCombination-Doppler-r18

Way forward:

- Option 1: Set paramCombination-Doppler-r18 as 7 (L=4, pυ=1/2, β=1/2) as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-5: RI restriction (typeII-Doppler-RI‑Restriction-r18)

Way forward:

- Option 1: Set RI restriction as 0010 for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-6: N4 configuration

Way forward:

- Option 1: N4=4

- Option 2: N4=1

- Other options are not precluded

Issue 2-1-7: K (number of NZP CSI-RS resources)

Way forward:

- Option 1: Set K=4 as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-8: m (separation between two consecutive CSI-RS resources) and d (DD unit duration (in slots)

Way forward:

- Depending on N4

  - Option 1: For N4>1, set m and d as 2 as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

- Note: Not configure m and d if N4=1

Issue 2-1-9: delta (slot associated with CSI report)

Way forward:

- Option 1: Set delta as 1 as a starting point for Rel-18 TypeII Doppler PMI test.

- Other options are not precluded

Issue 2-1-10: MCS

Way forward:

- Option 1: MCS20 (64QAM, 0.55)

- Other options are not precluded

Issue 2-1-11: CSI-RS configuration

Way forward:

- Option 1: Periodic with periodicity 5 slots and offset 1 slot

- Other options are not precluded

Issue 2-1-12: other parameters

Way forward:

- Option 1: follow below table

| Parameter | Value |
| --- | --- |
| Channel bandwidth and subcarrier spacing | For FDD, 10MHz/15kHz For TDD, 40MHz/30kHz |
| TDD DL-UL configuration | FR1.30-1 as specified in 38.101-4 Annex A. |
| Number of UE receiver antennas | 2 and 4 |
| R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18) | 1 |

Issue 2-1-13: Test metric

Way forward:

- Option 1: For PMI reporting tests with Type II codebook for predicted PMI, use the test metric defined as γ=tue,follow1,follow2trnd1,rnd2 as the starting point, where tue,follow1,follow2 is TBD % of the maximum throughput obtained at SNRfollow1,follow2 using the precoders configured according to the UE reports, and trnd1,rnd2 is the throughput measured at SNRfollow1,follow2 with random precoding.

- Other options are not precluded

Sub-topic 2-2 Initial simulation assumptions for TypeII for CJT

Issue 2-2-1: Propagation channel and correlation configuration

Way forward:

- Option 1: Use TDLA30-10 with XP high as the propagation channel and correlation configuration for Rel-18 TypeII for CJT test.

- Other options are not precluded

Issue 2-2-2: K (numberOfCSI-RS-Resources), NTRP (Number of TRPs) and restrictedCMR-Selection

Way forward:

- Option 1: Set K=2 CSI-RS resources, NTRP=2 TRPs and configure parameter restrictedCMR-Selection to restrict the number of selected CSI-RS resources is N=NTRP for Rel-18 TypeII for CJT PMI test.

- Other options are not precluded

Issue 2-2-3: N1, N2, O1, O2 and the number of CSI-RS ports

Way forward:

- Option 1: Set PCSI-RS=8 CSI-RS ports per TRP with (N1, N2) = (4, 1), (O1, O2) = (4, 1) as a starting point for Rel-18 TypeII for CJT PMI test.

- Other options are not precluded

Issue 2-2-4: paramCombination-CJT-L-r18

Way forward:

- Option 1: Set paramCombination-CJT-L-r18 as 7 ({4, 4})

- Option 2: Set paramCombination-CJT-L-r18 as 4 ({2, 2})

- Other options are not precluded

Issue 2-2-5: paramCombination-CJT-r18

Way forward:

- Option 1: Set paramCombination-CJT-r18 as 4 (pν=14and18, β=12) or 7 (pν=12and12, β=12) for paramCombination-CJT-L-r18 = 7

- Option 2: Set paramCombination-CJT-r18 as 1 (pν=18, β=14) for paramCombination-CJT-L-r18 = 4

- Other options are not precluded

Issue 2-2-6: RI restriction (typeII-CJT-RI‑Restriction-r18)

Way forward:

- Option 1: Set RI restriction as 0001 for Rel-18 TypeII for CJT PMI test.

- Other options are not precluded

Issue 2-2-7: codebookMode

Way forward:

- Option 1: Set codebookMode as Mode2 for Rel-18 TypeII for CJT test.

- Other option (i.e., Mode1) is not precluded

Issue 2-2-8: other parameters

Way forward:

- Option 1: follow below table

| Parameter | Value |
| --- | --- |
| Channel bandwidth and subcarrier spacing | For FDD, 10MHz/15kHz |
| TDD DL-UL configuration | FR1.30-1 as specified in 38.101-4 Annex A. |
| Number of UE receiver antennas | 2 and 4 |
| R (numberOfPMI-SubbandsPerCQI-Subband-Doppler-r18) | 1 |

Issue 2-2-9: Test metric

Way forward:

- Option 1: For PMI reporting test with Type II codebook for CJT, the test metric is defined as γ=tue,follow1,follow2trnd1,rnd2, where tue,follow1,follow2 is TBD % of the maximum throughput obtained at SNRfollow1,follow2 using the precoders configured according to the UE reports, and trnd1,rnd2 is the throughput measured at SNRfollow1,follow2 with random precoding.

- Other options are not precluded

Sub-topic 2-3 Test set-up and simulation assumptions for Rel-18 DMRS

Issue 2-3-1: DMRS configuration type and length

Way forward:

- Option 1: Rel-18 DMRS configuration Type 1 with length 1

- Other options are not precluded

Issue 2-3-2: DMRS ports

Way forward:

- Option 1: DMRS ports introduced by Rel-18

      - {1008} if Rank 1 test is selected

      - {1008, 1009} if Rank 2 test is selected

      - {1008-1010} if Rank 3 test is selected

      - {1008-1011} if Rank 4 test is selected

- Other options are not precluded

Issue 2-3-3: Propagation channel

Way forward:

- Option 1: RAN4 to consider low delay spread channels such as TDLA30 for PDSCH demodulation performance requirements.

- Other options are not precluded

Topic #3: Test set-up and simulation assumptions for BS demodulation performance

Sub-topic 3-1 Test set-up and simulation assumptions for Rel-18 DMRS

Issue 3-1-1: DMRS ports

Tentative Agreement:

- Rank 1 for 1Tx: {8}

- Rank 2 for 2Tx: {8,9}

Issue 3-1-2: other parameters

Way forward:

- Option 1:

      - SCS&CBW: 15KHz SCS, 5MHz; 30KHz SCS, 10MHz

      - Antenna configuration: 1Tx2Rx, 2Tx2Rx

      - MCS:  MCS 16

      - Channel Model: TDLC300-100

      - Mapping type: Type A

      - Number of DMRS symbol: 2

      - Waveform: CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {8}, {8, 9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

- Other options are not precluded

Sub-topic 3-2 Initial simulation assumptions for FR2 STxMP (if introduced)

Issue 3-2-1: Layer combination and DMRS port

Way forward:

- Option 1: Specify PUSCH requirements with layer combination 2+2 for SDM scheme single-DCI based STxMP transmission, DMRS port {0, 1} and DMRS port {2, 3} for each panel

- Other options are not precluded

Issue 3-2-2: PTRS

Way forward:

- Option 1: Specify PUSCH requirements with layer combination 2+2 for SDM scheme single-DCI based STxMP transmission, DMRS port {0, 1} and DMRS port {2, 3} for each panel

- Other options are not precluded

Issue 3-2-3: Number of DMRS

Way forward:

- Option 1: Configure 2 symbol of DMRS for specifying PUSCH requirement with SDM scheme

- Other options are not precluded

Issue 3-2-4: Timing and Frequency offset assumption

Way forward:

- FFS on Timing and Frequency offset modelling for specify PUSCH requirement with SDM scheme

- Other options are not precluded

Issue 3-2-5: Channel Model

Way forward:

- Option 1: Introduce TDLA30-300 for PUSCH requirement with SDM scheme

- Other options are not precluded

Issue 3-2-6: MCS

Way forward:

- Option 1: Introduce MCS 16 for PUSCH requirement with SDM scheme

- Other options are not precluded

Issue 3-2-7: SCS&BW

Way forward:

- Option 1: Introduce 120KHz SCS and 100MHz for PUSCH requirement with SDM scheme

- Other options are not precluded

Issue 3-2-8: Other test parameters

Way forward:

- Option 1:

The exiting test parameters for PUSCH requirements in FR2 can be reused as following

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS: 3D1S1U, S=10D:2G:2U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0, 1} for TRP1 {2,3} for TRP 2 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain | PUSCH mapping type | B |
| resource | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain | RB assignment | Full applicable test bandwidth |
| resource | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission per each TRP |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns |  |  |

- Other options are not precluded