3GPP TSG-RAN WG4 Meeting # 108bis											R4-2316924

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.14.7

Source:	Nokia, Nokia Shanghai Bell

Title:	WF on NR_FR1_lessthan_5MHz_BW_demod

Document for:	Approval

# Topic#1: UE Demod

## Sub-topic#1-1: PDSCH

Issue 1-1-1: Introduction of new requirements

Way forward

FFS, whether new PDSCH demodulation performance requirements needs to be introduced:

- FFS, whether to introduce a set of requirements for PDSCH for UE supporting only less than 5 MHz:

  - Consider minimum requirements for RedCap (5.2.2.1.17) as a reference

  - Further evaluate a need for optional features and excluding advance features which is out of scope of the RAN1 WI

  - FFS, a need for SDR requirements

- FFS, a need to introduce the limited set or no new requirements for PDSCH for UE supporting multiple CBWs (i.e., less than and more than 5 MHz)

Issue 1-1-2: CA requirements

Agreement:

- Not to define PDSCH CA demodulation requirements with CBW less than 5 MHz.

Issue 1-1-3: PDSCH parameters for further evaluation (non-HST scenario)

Agreement:

- Consider the following parameters for further evaluation of PDSCH performance in less than 5MHz:

  - Duplex: FDD, CBW: 3MHz, SCS: 15kHz, Number of PRBs: 15

  - Parameters from RedCap PDSCH minimum requirements (5.2.2.1.17) can be used for reference

    - Use 2 RX as a starting point, FFS 4 RX

Issue 1-1-4: PDSCH requirements in HST scenario

Way forward

- FFS, whether to introduce PDSCH requirements for less than 5MHz CBW in HST conditions with the speed up to 500 km/h:

  - Option 1: Use HST DPS propagation conditions (B3.3) and test 5.2.2.1.10 as a reference

  - Option 2: Use TS 38.101-4 Table 5.2.2.1.1-3 Test 1-6 [single-tap propagation conditions B.3.1] as a reference

    - 64QAM, 0.43, HST-972, Rank 1, 1Tx, 2Rx/4Rx.

  - Other options are not precluded

- FFS, the paramters (Dmin, Ds, f_d) of high speed propogation conditions

## Sub-topic#1-2: PDCCH

Issue 1-2-1: Introduction of new requirements

Way forward:

FFS, whether new PDCCH demodulation performance requirements needs to be introduced in less than 5 MHz CBW:

- Consider only 15KHz SCS, FDD, 2Rx, FFS for 4Rx

- Option1: Don’t define PDCCH requirements for channel bandwidth less than 5MHz

- Non punctured PDCCH:

  - Option 2: Define (non-punctured) PDCCH demodulation requirements with 15PRBs, 3MHz CBW, for UE supporting NR_FR1_lessthan_5MHz_BW:

    - 12 PRB CORESET

    - 2 symbols, AL2, DCI 1_0 (35 bits for 15 PRBs)

  - Option 3: Define (non-punctured) PDCCH demodulation requirements with 24 PRB PDCCH, for 5 MHz CBW with AL 4.

- Punctured PDCCH:

  - FFS, how to address the testability issue, i.e., no ACK/NACK for the SIB1 scheduled by PDCCH in CORESET#0.

  - Option 4: If the testability issue can be resolved than consider PDCCH demodulation requirements with punctured PDCCH for CORESET#0 (FFS, testing punctured PDCCH mapped to USS in CORESET#0)

      - with 3 symbols AL 8 PDCCH with 3 MHz CBW, interleaved

      - Other options are not precluded

Issue 1-2-2: Propagation conditions

Way forward:

- Consider the following propagation conditions:

  - TDLA30-10 for 1Tx,

  - TDLC300-100 for 2Tx antenna,

  - HST propagation conditions for 500km/h speed

    - FFS, Single-tap or HST DPS propagation conditions

  - Other options are not preluded.

## Sub-topic#1-3: PBCH

Issue 1-3-1: Need for new requirement

Agreement

- Define requirements for 12 PRB PBCH with 3 MHz CBW, 15kHz SCS, FDD, unknown SSB/PBCH index

- No requirement for known SSB index

Issue 1-3-2: Test metric

Agreement

- Reuse the Rel-15 PBCH demodulation test metric for punctured PBCH demodulation requirements.

Issue 1-3-3: PBCH parameters

Agreement

- Use the follwoing parameters as a starting point

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, [FFS, 1x4] | 1 | TBD |

Issue 1-3-4: Whether to consider HST conditions for PBCH

Way forward

The issue requires further discussion:

  - Option 1: Not to consider HST conditions for PBCH

  - Option 2: Further check the impact of HST conditions on PBCH performance.

    - Interested companies are encouraged to bring simulations results at the next meeting.

## Sub-topic#1-4: CSI reporting

Issue 1-4-1: A need for new requirements

Way forward:

Further discussion is needed whether to introduce new CSI reporting requirements for channel bandwidth less than 5MHz:

- Option1: Define CSI requirements for CBW=3MHz and SCS=15kHz: PMI, CQI and RI.

- Option 2: Do not CSI requirements.

FFS, requirements and applicability rules if UE supports only less then 5MHz CBW

## Sub-topic#1-5: General aspects

Issue 1-5-1: Common HST propagation conditions and parameters

Way forward:

Further discuss which HST propagation conditions (500 km/h) and parameters to use:

- Option 1: Single-tap propagation conditions ( based on B.3.1)

  - Ds = [300] m, Dmin = [2] m, f_d =[972] Hz

- Option 2: HST DPS propogation conditions (based on B.3.3.)

  - Ds = [700] m, Dmin = [150] m, f_d = [TBA]

Issue 1-5-2: Applicability of requirements

Way forward:

FFS, how to introduce applicability rules for UE Demodulation and CSI reporting requirements in less than 5 MHz channel BW:

- Option 1: Create requirements’ applicability table for UE supporting NR_FR1_lessthan_5MHz_BW.

- Option 2: The new requirements are only applicable to the specific bands for this WI, instead of being band-agnostic.

- Other options are not precluded.

# Topic#2: BS Dedmod

## Sub-topic#2-1: PUSCH

Issue 2-1-1: Introduction of requirements

Way forward:

- Further discuss the scope of PUSCH requirements:

  - FFS, whether to introduce full set of Rel-15 requirements for PUSCH for BS supporting only less than 5 MHz CBW, e.g.,

    - PUSCH with precoding disabled.

    - PUSCH with precoding enabled.

    - UCI multiplexed on PUSCH.

  - FFS, the limited set of requirements for PUSCH for BS supporting multiple CBWs (i.e., less and more than 5MHz CBW)

  - FFS, on applicability rules for PUSCH depending on the supported BW.

Issue 2-1-2: Performance evaluation/simulations

Way forward:

Use the following parameters as a starting point for performance evaluation:

- PUSCH with precoding disabled (CP-OFDM)

- Use parameters from Rel-15 5MHz CBW requirements as a baseline by updating the CBW to 3MHz:

  - Number of PRBs: 12

  - MCS: 16

  - 1T2R, 1 layer

- Note: Other parameters for simulations and requirements are not precluded

Issue 2-1-3: PUSCH requirements in HST conditions

Way forward:

FFS,

- Which test to consider for HST conditions in less than 5MHz CBW, e.g.,

  - UL timing adjustment

  - PUSCH for high-speed train: 350km/h and/or 500km/h

- which propagation conditions to consider for HST scenario (500km/h speed):

  - Option 1: The maximum Doppler is 815Hz

  - Option 2: TDLC 300-600

  - Other options are not precluded

## Sub-topic#2-2: PUCCH

Issue 2-2-1: Performance evaluation/simulations

Agreement:

- Evaluate PUCCH demodulation performance for 3MHz with 15kHz SCS ahead of defining requirements:

  - Enable Frequency Hopping for PUCCH

  - Number of PRBs:

    - 15, 12 for 3MHz CBW

    - 25 as a baseline for 5MHz CBW

  - Channel: TDLC300-100

  - Antenna configuration: 1T2R as a starting point

  - Use the following PUCCH Formats and paramters as a astarting point :

    - Format 0: UCI bits = 1; RB = 1; OFDM Symbols = 1,2

    - Format 1: UCI bits = 2; RB = 1; OFDM Symbols = 1

    - Format 2: UCI bits = 4 (when OFDM==1), 22 (when OFDM==2); RB = 4 (when OFDM==1), 9 (when OFDM==2); OFDM Symbols = 1,2

    - Format 3: UCI bits = 16; RB = 1 (when OFDM==14), 3 (when OFDM==4); OFDM Symbols = 4,14

    - Format 4: UCI bits = 22; RB = 1; OFDM Symbols = 14

  - Note: Other paramters are not precluded

Issue 2-2-2: Introduction of requirements

Way forward

- Based on the performance evaluation:

  - Introduce new requirements for all formats/cases if the performance difference is observed in any of the formats

- FFS, on applicability rules for PUCCH depending on the supported BW

## Sub-topic#2-3: RACH

Issue 2-3-1: Introduction of requirement

Agreement

- No new PRACH requirement need to be introduced for Less than 5MHz channel bandwidth.

- FFS whether applicability rule or note for long RACH sequences in less than 5 MHz bandwidths are needed.