3GPP TSG-RAN WG4 Meeting #108-bis	R4-2316963

Xiamen, CN Oct 9th ‒ Oct 13th, 2023

Title: 	WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception

Agenda Item:	5.7.4

Source: 	Qualcomm

Document for:	Approval

Moderator’s note: For all the open Issues listed in the WF below, other options are not precluded unless explicitly specified.

# General Aspects for FR2 Multi-Rx Demod

Issue 1-1-1: UE processing for sDCI

- Agreement:

- For sDCI SDM the UE processing types are defined as:

- Separate processing - MIMO detector processing 2Rx and li layers per TRP

- Joint processing - MIMO detector processing 4Rx and l1 + l2 layers across both TRP

Issue 1-1-2: UE processing for mDCI

- Agreement:

  - For mDCI with overlapping PDSCH the UE processing types are defined as:
- 	Separate processing - MIMO detector processing 2Rx and li layers per TRP 
- 	Joint processing - MIMO detector processing 4Rx and l1 + l2 layers across both TRP

Issue 1-1-3: MMSE-IRC Processing.

- Agreement:

  - Confirm receiver assumption of MMSE-IRC with both joint and separate UE processing.

Issue 1-1-4: Receiver assumption for mDCI case.

- Agreement:

- Define requirement with separate processing as a baseline for fully overlapping (1+1) and non-overlapping (2+2) scenarios.

  - Other scenarios not precluded, such as fully overlapping (2+2)

- FFS: Joint processing

  - Companies to evaluate performance of both separate and joint processing for the next meeting with the objective to make a decision next meeting. A minimum set of simulation scenarios may be agreed to manage the workload for the next meeting.

- Baseline case is prioritized.

Issue 1-1-5: Receiver assumption for sDCI SDM case.

- Agreement:

- Evaluate joint and separate processing for sDCI.

- For sDCI, consider separate processing as baseline.  Joint processing is FFS.

Issue 1-1-6: MCS and layer selection for mDCI fully overlapping case.

- Agreement:

  - Separate processing

    - 1+1: MCS 17, ρ = -12dB

    - Other scenarios are not precluded, such as 2+2 configuration.

Issue 1-1-7: MCS and layer selection for mDCI non overlapping case.

- Agreement:

  - Separate processing

    - 2+2: MCS13, ρ = -12dB

Issue 1-1-8: TxEVM.

- Agreement:

  - If considered, then use Tx EVM at 6% since we are considering up to 64QAM modulation.

Issue 1-1-9: Whether to consider ρ to be cross-talk power ratio.

- Agreement:

  - Consider to be the cross-talk power ratio.

Issue 1-1-10: TR update.

<way forward>

  - FFS whether to consider expanding the scope of TR 38.751 751 to include demodulation related evaluation and study.

# PDSCH Demodulation Requirements

Issue 2-1-1: Channel model

- Agreement:

  - Only use TDLA30-75 for 100 MHz/120 kHz as a baseline.

Issue 2-1-2: PTRS Port for sDCI schemes

- Agreement:

- Define both requirements. If UE can pass the test for one PTRS port per TRP configuration, UE would not need to be tested for one PTRS port across TRP test.

Issue 2-1-3: PT-RS EPRE Ratio

- Tentative agreement:

  - Default for sim assumption is EPRE ratio set to state 0 (also captured in 38.101-4 Table 7.2-1)

Issue 2-1-4: Test cases and simulation parameters for mDCI fully overlapping.

- Agreement:

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1+1: {1000} 2+2: {1000,1001} | 1+1: {1002} {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Fully-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Issue 2-1-5: Test cases and simulation parameters for mDCI non-overlapping

- Agreement:

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 1 |  |
|  | Length (L) |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 2+2: {1000,1001} | 2+2: {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | non-overlapping |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Issue 2-1-6: Whether to consider a scaling factor for transmitted signal in mDCI fully-overlapping case

- Agreement

  - Don’t consider SNR scaling for mDCI fully overlapping case.

# PMI Reporting Requirements

Issue 3-1-1: TDD Pattern

- Agreement:

  - Use DDDSU TDD pattern with 1.75ms delay.

Issue 3-1-2: Channel Model.

- Agreement:

  - Use TDLA30-35 propagation channel for PMI reporting requirement for sDCI SDM scheme.

Issue 3-1-3: Spatial Correlation Model for PMI Reporting Test Cases

- Agreement:

  - Follow the agreements in general section for spatial correlation matrix definition.

Issue 3-1-4: UE Processing for PMI Reporting

- Agreement:

  - Use the same agreement on UE processing as PDSCH demod for sDCI for PMI reporting.

Issue 3-1-5: PTRS Port for PMI Reporting Test Cases

- Agreement:

  - Use the same agreement as PDSCH demod for sDCI PTRS port configuration.

Issue 3-1-6: Simulation parameters for PMI reporting requirement for sDCI SDM scheme

- Agreement:

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1 (Note 1) | TRxP #2 (Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-1 as specified in Annex A of TS 38.101-4 |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-35 |  |
| Antenna configuration per TRxP |  |  | XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.75 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |

Issue 3-1-7: Performance Metric

- Agreement:

  - Set test metric as γ=t_ue/t_rnd , where t_ue is [70, 90] % of the maximum throughput obtained at SNR_ue using the precoders configured according to the UE reports, and t_rnd is the throughput measured at SNR_ue with random precoding.

Issue 3-1-8: Reference Channel

- Agreement:

  - Define new reference channel for PMI reporting requirement for sDCI SDM.

    - FFS on the exact details.

Issue 3-1-9: MCS

- Agreement:

- MCS: [11,13], MCS Table 1

# CR Split

| Section number | Section title | Responsible company |
| --- | --- | --- |
| TS 38.101-4 |  |  |
|  | Big CR | Qualcomm |
| 5.1.1.8 | Applicability of requirements (if any) | Samsung |
| B.2.5 | FR2 multi-rx correlation model | Qualcomm |
| 7.2.2.2.5 | Minimum requirements for sDCI SDM | Huawei |
| 7.2.2.2.6 | Minimum requirements for mDCI non-overlapping | Nokia |
| 7.2.2.2.7 | Minimum requirements for mDCI fully-overlapping | Apple |
| 8.3.2.2.2 | Minimum requirements for PMI reporting requirement for sDCI SDM | MTK |
| A3.2.2.9 | Reference measurement channels for sDCI SDM | Huawei |
| A3.2.2.10 | Reference measurement channels for mDCI non-overlapping | Nokia |
| A3.2.2.11 | Reference measurement channels for mDCI fully-overlapping | Apple |
| A.4 | Reference measurement channels for PMI reporting requirement for sDCI SDM (if any) | MTK |