3GPP TSG-RAN WG4 Meeting # 108-bis	R4-2316988

# Xiamen, China, Oct.9th – Oct.13th, 2023

Title:	Reply LS on required DCI signalling for advanced receiver on MU-MIMO scenario

Response to:	R1-2308598

Release:	Rel-18

Work Item:	NR_demod_enh3-Core

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN WG2

Contact person:	Ke Li, like54@hisilicon.com

Jingzhou,Wu, wujingzhou@chinatelecom.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN4 thanks for the Reply LS (R1-2308598) on DCI signalling for advanced receiver on MU-MIMO and appreciate that RAN1 agreed to implement the proposed DCI signalling in TS 38.212. For the questions included in the reply LS (R1-2308598), RAN4 has the following response:

Question 1: Whether this new signaling in DCI is introduced in DCI format 1_2 in addition to format 1_1?

Answer: The understanding in RAN4 is that URLLC is not a common scenario for MU-MIMO, but if there are relevant use cases with MU-MIMO scheduling with DCI format 1_2, the signalling in DCI can be introduced in DCI format 1_2, otherwise not.

Question 2: Whether this new signaling in DCI is supported for one or more DL multi-TRP schemes?

Answer: The understanding in RAN4 is that there are limited scenarios for MU-MIMO with mTRP operation. RAN4 suggests that this new signalling in DCI is not supported for multi-TRP schemes.

Question3: Whether this new signaling in DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2?

Answer:  This new DCI is supported if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and target UE is only scheduled 1 codeword.

Question 4: Whether the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured?

Answer: This new DCI signalling can be supported if there are relevant use cases with MU-MIMO scheduling when the RRC codeBlockGroupTransmission is configured, otherwise not.

Question 5: Whether the new signaling in DCI is supported when Rel-18 DMRS is configured?

Answer: Yes. The new signaling can be supported for the UE with Rel-18 DMRS configured, and co-scheduled UE mentioned in DCI signaling includes both co-scheduled UEs on R15 DMRS ports and co-scheduled UEs on R18 DMRS ports

Question 6: In the content corresponding to “Bit field mapped to index” =6, whether or not the phrase “In each individual PRB allocated to the target UE, the following condition is satisfied” should be replaced by “In each individual PRB PRG allocated to the target UE, the following condition is satisfied”?

Answer: RAN4 has agreed that the revision suggested by RAN1 is not needed.

Question 7: For “Bit field mapped to index” =1/2/3/4/5, does “empty PRB without co-scheduled UE” is allowed “in all the PRBs” of the target UE.

Answer: [Yes, “For bit field mapped to index”=1/2/3/4/5”, empty PRB without co-scheduled UE is allowed in all the PRB” of the target UE]

2	Actions

To RAN WG1

ACTION:

RAN4 kindly request RAN1 to take the above RAN4 answers into consideration.

3	Dates of next RAN WG 4 meetings

TSG RAN WG4 Meeting #109		              13rd  – 17th Nov. 2023   	    	Chicago, USA

TSG RAN WG4 Meeting #110		              TBA		   	    	Athens, GR