3GPP TSG-RAN WG4 Meeting #108bis												R4-2317004

Xiamen, China, October 09 – October 13, 2023

Agenda item:			5.15.4

Source:	vivo

Title:	WF on NR_FR1_TRP_TRS_enh

Document for:	Approval

Topic #1: Test methodology

Sub-topic 1-1 Single-layer UL-MIMO TRP test method

Issue 1-1-2: Test Methods for fully Coherent UE support multiple TPMI index 2~5

Agreements:

- Capture common test procedure of O1 and O2 into TR, final measured EIRPs processing can be further decided. The performance metric for each approach should also be further discussed.

  - Option 1 (averaging TRPs)

  - Option 2 (Max EIRPs)

- Test mode is captured in the TR which can be considered as one test method, only if the phase variation issue can not be resolved in O1 and O2.

  - Test mode option 1: Single antenna transmission each time

  - Test mode option 2: Two antenna transmission simultaneously

  - Other Test modes are not precluded

Issue 1-1-3: Performance metric definition for Option 1 and Option 2 methodology

Agreements:

If we cannot agree this meeting to the metric, we can identify the areas we would like to study for the next meeting to further the progress.

Topics to study for next meeting

- Identify names and metrics for option 1 and option 2

- Come up with impact to regulatory and industry for both option 1 and option 2

Issue 1-1-6: phase variation for single-layer UL-MIMO

Agreements:

- phase variation is a general issue for all 2Tx test cases, no matter TxD and UL MIMO.

- Further study the TRP results impacts based on simulation and real measurements. The impact should also be considered when developing 2Tx TRP requirements in future.

Issue 1-1-7: Measurement grid analysis for UL-MIMO

Agreements:

- In RAN4, measurement grid and preliminary MU assessment to take options 1 and 2 into account

- Final grids and corresponding MU value for 2Tx conformance testing can be decided in RAN5.

- If any measurements are presented as part of this WI, use at least the legacy Dq=Df=15° measurement grids

Sub-topic 1-4 RC test method

Issue 1-4-1: RC test method for 2Tx test cases

Agreements:

- RC test method for 2Tx test cases can be studied based on contribution-driven approach.

Topic #2: Rel-18 TRP TRS requirements

Sub-topic 2-1 Rel-18 AC lab alignment activity

Issue 2-1-1: LAD information update

Agreements:

  - Confirm the following updated LADs for Rel-18 AC lab alignment activity.

| Rel-18 Lab Alignment Device (LAD) |  |  |  |
| --- | --- | --- | --- |
| Volunteer lab | Owner | Contact | Note |
| LAD1 (Rel-17) | vivo | Ruixin ruixin. wang@vivo.com | For n78 testing. The lab alignment has been started |
| LAD2 (Rel-17) | vivo | Ruixin ruixin. wang@vivo.com | For n78 testing. The lab alignment has been started |
| LAD3 | vivo | Ruixin ruixin. wang@vivo.com | For n28 testing. The lab alignment has been started |
| LAD4 | TIM | Alex alessandro.trogolo@telecomitalia.it | For n28 testing. The lab alignment has been started |
| Note: To keep the test burden, most 4 test cases, i.e., two devices per band. |  |  |  |

Issue 2-1-2: Updated Working procedure for Rel-18 AC lab alignment activity

Agreements:

  - Approve the following working procedure for Rel-18 AC lab alignment activity:

Working procedure for Rel-18 AC Lab Alignment Campaign

- The purpose of Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support testing based on 3GPP TS 38.161.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TS 38.161;

- Test cases for Lab Alignment Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band: BHHL and BHHR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE configured with 1Tx

- Lab Alignment Device (LAD): in total 4 devices, to avoid delay of lab alignment activity, the LADs confirmed by RAN4#108 meeting will not be further added

  - Rel-17 LADs (LAD1 and LAD2) for band n78;

  - 2 more devices (LAD 3 and LAD 4) for band n28;

  - UE information listed in section 6

- Test results submitting:

  - Using the same worksheet template in [R4-2313892] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#109] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the expanded MU for BHH (defined in Annex of TR38.870).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of AC alignment activity is in [R4-2315843]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

Issue 2-1-3: Lab alignment for 2Tx in Rel-18

Agreements:

  - Skip lab alignment campaign and performance requirement for 2Tx TRP in Rel-18, this can be considered as Rel-19 scope.

Sub-topic 2-2 RC Harmonization and lab alignment

Issue 2-2-1: updated Working procedure for Rel-18 RC harmonization

Agreements:

  - Approve the following working procedure for Rel-18 RC harmonization activity:

Working procedure for Rel-18 RC harmonization Campaign

- The purpose of RC harmonization activity is to ensure there is no unexpected lab deviation among different RC systems, and the RC harmonization results will be compared with reference AC results to demonstrate the gap between different methodologies.

- RC Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should have Reverberation chamber(s) ready to support testing based on latest version of 3GPP TR 38.870.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: latest version of 3GPP TR 38.870;

- Test cases for RC harmonization Campaign:

  - Test bands: n28, n78; one low and one high band is sufficient

  - Number of test cases per band for each scenario: left and right at low/mid/high channel, total 6 test cases;

  - Use scenario: Both Head and Hand phantom (Talk mode, BHHL and BHHR) and Hand phantom only (Browsing mode, HL and HR);

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 1Tx.

- Harmonization Devices:

  - Same devices as Rel-18 AC lab alignment activity, listed in section 6

- Test results submitting:

  - Using the same worksheet template in [R4-2313893] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

- Harmonization RC alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of harmonization data pool from ≥3 labs submitted before end of [RAN4#109] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for RC lab alignment should be defined as X*MU (X is TBD) as baseline. MU value is the preliminary expanded MU for talk mode and/or browsing mode (to be defined in RAN5).

  - The timeline of RC harmonization activity is in [R4-2315843]. How to treat late submission results and confirm the alignment: TBD

- RC vs AC test methods harmonization criteria:

  - How to compare the RC measurement results with AC measurements results is FFS, e.g.;

    - Option 1: comparison of each reference value of RC and AC as starting point (per-band per-PC averaging approach (linear average with dBm) of each method from all test labs those are not apparent outliers)

      - The stand Dev of each test method should be studied and potentially considered

    - Option 2: other approaches are TBA

  - The pass/fail criteria are defined as TBD

- Test lab procedures:

- LADs delivery scheme

- LAD delivery scheme will be decided after the confirmation of all the RC volunteer labs.

- LAD delivery will also consider the parallel Rel-18 AC lab alignment activity to ensure efficiency

- LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

- Encourage test labs to share resulting combined MU based on their own systems

Issue 2-2-2: RC vs AC harmonization criteria

Agreements:

  - Encourage companies to further discuss reasonable criteria and pass/fail limits for RC vs AC harmonization.

Sub-topic 2-3 Rel-18 TRP TRS measurement campaign

Issue 2-3-1: updated working procedure for Rel-18 TRP TRS Performance Test Campaign to define requirements

Agreements:

  - Approve the following working procedure for Rel-18 TRP TRS Performance Test Campaign.

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: Complete full coverage of band n1, n28, n41, and n78 requirements, including performance objectives which were not concluded in Rel-17;

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx, study how to specify EN-DC requirements based on SA measurement results.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - Year of production: from [second-half 2021 to 2024] this will be confirmed next meeting

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with the selected test labs to provide devices

    - A test lab shall measure only one UE model in case different samples are provided

    - Same UE model supporting different set of bands can be measured. For this case, the UE model should be marked as different model, e.g., model A-1, model A-2. (guidance on how to manage this case are provided in the spreadsheet in [TBD])

  - The 3GPP member providing the DUTs should contact one of the selected labs to check their availability to receive the DUTs and define together the related provisioning aspects

    - Any issue should be reported to the rapporteur in a timely manner to discuss for an alternative solution

    - To plan properly the measurement campaign, the following actions are requested for the RAN4 Nov meeting:

      - The rapporteur check with the volunteer labs the number of DUTs (minimum 3, maximum 15) they expect to be able to measure AND how many DUTs they can accommodate from 3GPP members

      - The 3GPP member providing the DUTs check how many samples they intend to provide (in terms of maximum number)

      - Planning of the measurement campaign could be reviewed based on the above points

- Test results submitting:

  - RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure

  - UE information disclosure: laboratories use the spreadsheet in [TBD] to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

  - Laboratories use the worksheet template in [R4-2315844] to submit the measurement results for Rel-18 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed):

    - The minimum number of submitted devices from each lab is 3

    - Volunteer labs provide the device information sheet ONLY to the RAN4 Secretary and the sheet used to submit measurement results to the rapporteur

  - RAN4 Secretary ONLY publishes to 3GPP RAN4 the following summary of statistical information after anonymizing the sensitive UE information data, i.e., UE model name and vendor name:

    - Total number of devices

    - Total number of models

    - Total number of devices vendors

    - Percentage of devices per vendor

    - Percentage of devices per Power Class

    - Percentage of devices per each supported band

    - Percentage of devices per year of production

    - Percentage of the devices that are certified by at least one of certification bodies as following: PTCRB ,GCF, and NAL/CTA (Chinese network access licensed test)] /FCC/CE

      - Once the device gets the above certification, for RAN4 discussion that means the device is commercially available

    - Percentage of devices that are commercially available

  - The progress in each lab are encouraged to be shared on the RAN4 reflector (for example - how many devices have been measured and on which bands)

    - Information of the devices that are going to be measured should be shared with the RAN4 Secretary as soon as available in order to monitor the achievement of the thresholds defined in point 8.b.

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Only the results from aligned labs will be considered for specifying requirements

  - Requirements will not be specified if the following thresholds [further check in RAN4#109] are not satisfied by the devices pool:

    - Minimum number of devices for each band, each device size, each power class: [40]

    - Minimum number of device models: [25~30]

    - Minimum number of devices' vendors: 5

    - Percentage of devices from [second-half 2021 to 2024]: TBD%:

    - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD%

    - Percentage of devices that are commercially available: [100] %

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-18.

  - Method of limits derivation: per-band Data driven approach

  - The value at [TBD] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, e.g.,with [X]dB offset

- Test lab procedures

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

Issue 2-3-2: Rel-18 device information template for Device information collection and statistical analysis

Agreements:

  - Further check and confirm the UE information template based on updated working procedure next meeting.

Sub-topic 2-4 Rel-18 TRP TRS requirements work

Issue 2-4-1: Timeline for Rel-18 AC lab alignment and RC harmonization activity

Agreements:

  - Approve the updated schedule for Rel-18 TRP TRS measurement activities and requirement development in R4-2315843.

Issue 2-4-2: number of receive antennas for n1 or n3 requirements

Agreements:

  - Further discuss whether reduced number of device for 2Rx of n1/n3 should be considered.

  - Further discuss and decide 2Rx or 4Rx requirements for n1/n3 in Rel-18.