3GPP TSG-RAN WG4 Meeting #108bis	R4-2317007

Xiamen, China, 9th Oct-13th Oct, 2023

Title: 	WF on FR2 HST demodulation requirements

Agenda Item:	8.12.5

Source: 	Samsung

Document for:	Approval

# Topic #1: Deployment and Channel Modelling

## Sub-topic 1-1: Deployment and Channel Model for Demodulation requirement with simultaneous Rx reception in open space scenario

Issue 1-1-1:  RRH and (initial train) positioning of channel model for PDSCH requirements with multi-Rx Reception

Agreement:

- The location of RRH k is given as x_k=k*D_s+j*D_min, where:	k∈[-∞,∞], j=sqrt(-1) and D_minis the distance between the RRHs and railway track, while D_s is the distance of two RRHs, both in meters

- The train location is denoted as: y=a+j*0, a∈[-D_(s_offset),∞] and a means distance in meters, which means the train is right on the track. a=a_0+vt where v (m/s) is the moving speed of the train and a_0=-D_(s_offset).

- Update the initial UE position (t=0) in the figure describing beam serving coverage for multi-Rx reception in Bi-directional scenario from RAN4#107 WF [R4-2309825] as follows:

- Update the FR2 HST-DPS Channel Profile in B.3.4 in 38.101-4 in the CR draft stage

  - a=a_0+vt where v (m/s) is the moving speed of the train

    - a_0 = - D_(s_offset) for B.3.4.1 Unidirectional Deployment Channel Profile

    - a_0 =0   for B.3.4.2 Bidirectional Deployment Channel Profile

Issue 1-1-2:  Whether need to include relative power for channel model for PDSCH requirements with Multi-Rx Reception

Agreement

- Do not consider relative power profile modelling based on FR2 HST UE location for HST FR2 scenario to specify PDSCH requirement with multi-Rx simultaneous reception

## Sub-topic 1-2 Deployment and Channel Model in tunnel scenario

Issue 1-2-1: Channel Model in Tunnel Scenario for UE demodulation and BS demodulation

Agreement

- No need to introduce a new channel model for HST FR2 Tunnel deployment scenario for both UE and BS demodulation requirements in Rel-18

# Topic #2: UE demodulation for CA requirement

## Sub-topic 2-1: Intra-band CA requirement

Issue 2-1-1:  FRC for CA requirement

Agreement

- Confirm the FRC tables shown in Appendix for FR2 HST-DPS CA PDSCH demodulation requirements.

  - Uni-directional HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | UNI-1 TDD | UNI-2 TDD | UNI-3 TDD |  |
| Channel bandwidth | MHz | 50 | 100 | 400 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 32 | 66 | 264 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For slot i = 2 |  | 13 | 13 | 13 |  |
| For slot i = 3 |  | 9 | 9 | 9 |  |
| Allocated slots per 2 frames |  | 126 | 126 | 126 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 17 | 17 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 18 | 18 | 18 |  |
| For Slot i = 3 |  | 18 | 18 | 18 |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 13832 | 28680 | 114776 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 21504 | 45096 | 180376 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 21504 | 45096 | 180376 |  |
| For Slot i = 3 |  | 13832 | 28680 | 114776 |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 24 | 24 | 24 |  |
| For Slot i = 3 |  | 24 | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 2 | 4 | 14 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 3 | 6 | 22 |  |
| For Slot i = 1 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 3 | 6 | 22 |  |
| For Slot i = 3 |  | 2 | 4 | 14 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 41856 | 90360 | 406368 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 41856 | 90360 | 406368 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 84, 87, …, 159} | Bits | 33408 | 68904 | 275616 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 51072 | 105336 | 421344 |  |
| For Slot i = 1 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slot i = 2 |  | 51072 | 105336 | 421344 |  |
| For Slot i = 3 |  | 33408 | 68904 | 275616 |  |
| Max. Throughput averaged over 2 frames | Mbps | 123.2 | 257.839 | 1031.409 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1 with periodicity 20ms |  |  |  |  |  |

  - Bi-directional HST-DPS with CA scenario

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | BI-1 TDD | BI-2 TDD | BI-2 TDD |  |
| Channel bandwidth | MHz | 50 | 100 | 400 |  |
| Subcarrier spacing | kHz | 120 | 120 | 120 |  |
| Allocated resource blocks | PRBs | 32 | 66 | 264 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 9 | 9 | 9 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 13 | 13 | 13 |  |
| For Slot i=1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Allocated slots per 2 frames |  | 124 | 124 | 124 |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |
| MCS index |  | 17 | 17 | 17 |  |
| Modulation |  | 64QAM | 64QAM | 64QAM |  |
| Target Coding Rate |  | 0.43 | 0.43 | 0.43 |  |
| Number of MIMO layers |  | 2 | 2 | 2 |  |
| Number of DMRS REs |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} |  | 18 | 18 | 18 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} |  | 18 | 18 | 18 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Overhead for TBS determination |  | 6 | 6 | 6 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 13832 | 28680 | 114776 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 21504 | 45096 | 180376 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | Bits | 24 | 24 | 24 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | Bits | 24 | 24 | 24 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |  |
| For Slot i, if mod(i, 5) = 3 for i from {4,…, 159} | CBs | 2 | 4 | 14 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {5,…,159} | CBs | 3 | 6 | 22 |  |
| For Slot i = 1,2,3 |  | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| For Slots i = 5 and 85 (Note 3) | Bits | 33408 | 76632 | 392640 |  |
| For Slots i = 6 and 86 (Note 3) | Bits | 33408 | 76632 | 392640 |  |
| For Slot i, if mod(i, 5) = 3 for i from {7,…, 94,87, …, 159} | Bits | 33408 | 68904 | 275616 |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {7,…,84,87,…,159} | Bits | 51072 | 105336 | 421344 |  |
| For Slot i = 1,2,3 | Bits | N/A (Note 4) | N/A (Note 4) | N/A (Note 4) |  |
| Max. Throughput averaged over 2 frames | Mbps | 121.433 | 254.15 | 1016.651 |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Binary Channel Bits are calculated under assumption of 52 PRBs TRS allocation when the number of allocated resource blocks are more than 52. Note 4: SS/PBCH block is transmitted in slot #1, slot #2 and slot #3 with periodicity 20ms |  |  |  |  |  |

Issue 2-1-2: Release independent for CA requirement

Agreement

- Do not define FR2 HST CA requirement to be release independent from Rel-17 according to the UE capability with PC6 UE.  FR2 HST CA requirement is applicable from Rel-18

Issue 2-1-3: CA requirements derivation

Agreement

- Companies are encouraged to provide both ideal and impairment results for PDSCH CA requirements derivation in RAN4#109 meeting

# Topic #3: PDSCH requirement with multi-Rx reception

## Sub-topic 3-1: General for PDSCH requirement with multi-Rx reception

Issue 3-1-0: Test requirement to be defined

Way forward:

  - Test cases to be introduced

    - Option 1: one case with RTD larger than CP

    - Option 2: two cases based on UE declaration on supported baseband processing with RTD larger than CP or not

      - Case 1: RTD = 1.0 CP

      - Case 2: RTD larger than CP

Issue 3-1-1: UE processing assumption for the FFT window

Issue 3-1-2: Necessity to introduce RTD into channel model for CPE FFT processing test

Issue 3-2-1: Number of MCS for PDSCH requirement for multi-Rx reception

Issue 3-2-2: Number of SNR for PDSCH requirement for multi-Rx reception

Way forward:

- Introduce RTD in the FR2 HST PDSCH requirement between the different RX panels. Discuss RTD value based on evaluation.

  - Note RTD: Timing offset of TRP2 from TRP1

- Define one fixed MCS value per each Panel for PDSCH requirement with multi-Rx reception with fixed FRC

- Encourage companies to evaluate the performance difference under assumption on FFT (single FFT across Rx chains, and separate FFT per RF chain) with the following test setup and test metric, make a decision in RAN4#109 meeting

  - RTD for evaluation with the following priority order

    - Option 1: (2CP) 1.2 us

    - Option 2: (1 CP): 0.57us

    - Option 3a (1.2CP): 0.7us and Option 3b: (2.5CP) 1.5us

  - MCS

    - Set MCS 19 for TRP1 and Set MCS Y for TRP2 for FR2 HST simultaneous multi-Rx scenario.

      - Candidate MCS Y = {MCS 11, MCS 13}

      - Other feasible MCS are not precluded.

      - Expected power imbalance value and RTD value according to the deployment model (depending on UE Location) can be considered to derive suitable MCS pair (MCS 19, MCS Y). For example

        - Step 1: Derive the UE location based on given RTD value;

        - Step 2: Derive the expected power imbalance value x dB based on the UE location

        - Step 3: Run simulation to find SNR pair (SNR1, SNR2) for MCS pair (MCS 19, MCS Y) derived based on test metric 70% of TP for each PDSCH.

        - Choose the highest MCS Y according to the following condition:

          - SNR1(MCS19) – SNR2(MCS Y) =< x dB

          - To be decided whether SNR based on averaged submitted simulation results or individual SNR values reported.

  - Test metric for SNR derivation

      - Option 1: 70% Tput for each PDSCH as baseline.

      - Option 2: 70% Tput across all PDSCH

        - Step 1: Run simulation to find one SNR = y dB so that SNR pair {y dB, y-x dB} derived based on MCS pair (MCS 19, MCS Y) is fulfilled the following test metric

          - (Tput1 from TRP1 (MCS19) + Tput2 from TRP2 (MCS Y)) > 70% * (Tput1_max from TRP1 + Tput2_max from TRP2)

Issue 3-2-3:  Power scaling for two served RRH

Agreement:

- Do not consider the power scaling factor 1/sqrt(2) to transmitted PDSCH signal from each TRxP

## Sub-topic 3-2: Test setup for PDSCH requirement with multi-Rx reception

Issue 3-2-4: PDSCH allocation timeline in the UE Demod Test

Agreement

- The overview period after receiving MAC CE activate TCI switching for each panel from the through statistic is specified as

  - THARQ+TMAC Proc+[TfirstSSB + TSSB proc +TfirstTRSafterSSB]+ TTRS pro

  - THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK;

  - TMAC proc = 24 is the number of slots to process MAC-CE;

  - TTRS proc = 16 is the number of slots for TRS processing;

  - FFS on

    - TfirstSSB

    - TfirstTRSafterSSB

Issue 3-2-5: Applicability rule for PDSCH requirement with simultaneous multi-Rx reception

Agreement:

- Introduce the following test applicability rule for PDSCH requirement with multi-Rx reception

  - UE Power Class 6 (PC6);

  - simultaneousReceptionFR2HST-r18;

  - maxNumberActiveTCI-PerBWP > 1

  - multiDCI-MultiTRP-r16;

  - overlapPDSCHsFullyFreqTime-r16;

# Topic #4: Demoduation requirements for tunnel scenario

## Sub-topic 4-1: Test Scope of demodulation requirements of tunnel scenario

Issue 4-1-1: Whether to define UE demodulation requirements for tunnel deployment scenario in FR2 HST

Agreement:

- No new UE demodulation requirement introduced for Tunnel scenario in Rel-18 FR2 HST WI

Issue 4-1-2: Whether to define BS demodulation requirements for tunnel deployment scenario in FR2 HST

Agreement:

- No new BS demodulation requirement introduced for Tunnel scenario in Rel-18 FR2 HST WI

# Topic #5: Work split for Rel-18 FR2 HST demodulation

| Work split | Company |
| --- | --- |
| Big CR for FR2 HST demodulation | Samsung |
| Simulation result summary for FR2 HST demodulation | Samsung |
| PDSCH requirement with CA | Nokia |
| PDSCH requirement with multi-Rx reception | Huawei, QC |
| Channel Model for Multi-Rx | Samsung |
| Applicability rule of PDSCH requirement with multi-Rx | QC |
| FRC for PDSCH requirement with CA and multi-Rx | Ericsson |