3GPP TSG-RAN WG4 Meeting #108bis				   				            	            R4-2317322

Xiamen, China, October 9 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for requirements of inter-f L1-RSRP measurement with MG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4#108, it was agreed to introduce RRM requirement for inter-frequency L1-RSRP measurement in section 9.x.y. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introudce RRM requirement for inter-frequency L1-RSRP measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirement for inter-frequency L1-RSRP measurement would still be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 9.x.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 9.x	NR inter-frequency L1 measurement

### 9.x.1	Introduction

A L1-RSRP measurement is defined as an inter-frequency L1-RSRP measurement provided it is not defined as an intra- frequency L1-RSRP measurement according to clause [9.x].

When measurement gaps are provided, the UE shall be able to perform inter-frequency L1-RSRP measurements of SSBs from inter-frequency neighbor cells.

The UE shall be able to measure all SSB resources within the CSI-ResourceConfig settings configured for L1-RSRP measurement on inter-frequency neighbor cells with measurement gaps. If the number of resources exceeds the UE capability indicated by [TBD], it is up to UE implementation on how to choose resources to measure.

The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the higher layer parameter reportConfigType of each reporting setting CSI-ReportConfig.

### 9.x.2	Requirements Applicability

The requirements in clause 9.x.z apply if the following conditions are met:

-	FFS: The SSB configured for inter-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for RRM measurement in one of the measurement objects configured in the measObjectNR.

-	The inter-frequency neighbor cell is known.

-	The SSB resources configured for inter-frequency L1-RSRP measurements are measurable.

An SSB resource configured for inter-frequency L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in [clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2], respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to [Annex B.2.4.1] for a corresponding band.

The inter-frequency neighbor cell is considered as known if the following conditions are met in this requirement:

[TBA]

Otherwise, the cell is unknown.

### 9.x.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports for report configurations configured for the inter-frequency L1 measurement on neighbor cell.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.6. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting-r17 is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause 10.1.6. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in 10.1.6.

In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

#### 9.x.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.x.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.x.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement report if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

### 9.x.4	Number of cells and number of SSB

[TBA]

### 9.x.5	L1-RSRP measurement requirements

#### 9.x.5.1	Inter-frequency SSB based L1-RSRP Reporting

If an inter-frequency neighbor cell is known according to [9.x.z.2], the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_Inter. The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	[Concurrent gaps are not configured, and]

-	SSBs of the neighbor cell configured for inter-frequency L1-RSRP measurement are fully or partially overlapped with measurement gaps.

The value of TL1-RSRP_Measurement_Period_SSB_Inter is defined in Table 9.x.5.1-1 for FR1. The value of TL1-RSRP_Measurement_Period_SSB_Inter is defined in Table 9.x.5.1-2 for FR2, where

-	M=[TBD] if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=[TBD] otherwise

-	N= 8.

-	CSSFinter: it is a carrier specific scaling factor and is determined according to [CSSFwithin_gap] in clause 9.1.5.x for L1-RSRP measurement conducted within measurement gaps.

Table 9.x.5.1-1: Inter-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_Inter for known cells in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_Inter (ms) |
| --- | --- |
| non-DRX | max(TReport, M*max(TSSB, MGRP)*CSSFinter) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K*M)*max(TDRX,TSSB, MGRP)*CSSFinter) |
| DRX cycle > 320ms | M*TDRX*CSSFinter |
| Note 1: TSSB is the periodicity of the SSB-Index configured for inter-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

Table 9.x.5.1-2: Inter-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_Inter for known cells in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_Inter (ms) |
| --- | --- |
| non-DRX | max(TReport, M*N*max(TSSB, MGRP)*CSSFinter) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*N)*max(TDRX,TSSB_CDP, MGRP)*CSSFinter) |
| DRX cycle > 320ms | ceil(1.5*M*N)*TDRX*CSSFinter |
| Note 1: TSSB is the periodicity of the SSB-Index configured for inter-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

End of Change 1