3GPP TSG-RAN WG4 Meeting #108-bis	R4-2317323

Xiamen, China, October 09 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for LTM cell switch delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2023-10-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 agreements on LTM cell switch delay have not been captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capturing LTM cell switch delay requirements for PCell switch and the related known target cell and known TCI state conditions in section 6 of TS 38.133. Revision 1: Agreements reached during RAN4#108bis regarding TLTM-RRC-processing, TLTM-processing and TfirstSSB/TSSB-proc are captured. Agreement about SFN acquision can be captured later if RAN4 agrees a scenario where SFN acquisition is included in the cell switch delay. Parts that are still under discussion are marked as FFS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for LTM is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change 1>>

## 6.X	L1/L2-Triggered Mobility

### 6.X.1	LTM Cell Switch delay

#### 6.X.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell [or PSCell] to another cell.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighboring cell.

  - FR1 cell to FR1 cell

  - FR1 cell to FR2 cell

  - FR2 cell to FR2 cell

  - FR2 cell to FR1 cell

PCell switch to a serving SCell in MCG

  - FR1 cell to FR1 cell

  - FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighboring cell

  - FR1 cell to FR1 cell

PCell switch to a serving SCell in MCG

  - FR1 cell to FR1 cell

- Editor’s note: FFS PSCell switch to a neighboring cell

- Editor’s note: FFS PSCell switch to a serving NR SCell in SCG

#### 6.X.1.2	LTM Cell Switch delay

LTM cell switch delay DLTM is the delay from the end of the last TTI containing the MAC-CE command for cell switch until the time the UE transmits the first UL message on the target cell.

When the target cell and the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, the LTM cell switch delay is defined as:

DLTM = Tcmd [+ TLTM-RRC-processing or Texecution_time] + TLTM-interrupt

Where:

Tcmd equals to THARQ + 3ms, where THARQ is the timing between cell switch command and acknowledgement as specified in TS 38.213.

[TLTM-RRC-processing or Texecution_time] is the time for early ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

[TLTM-RRC-processing or Texecution_time]  = 0, if the the UE supporting capability [earlyDecodingAndValidityCheck] has performed early ASN.1 decoding and validity/compliance check of the complete LTM candidate cell configuration prior to the cell switch command for the LTM target cell indicated in the cell switch command.

Editor’s note: FFS the conditions under which the UE with new capability can work with early ASN.1 decoding and validity/compliance check

Otherwise [TLTM-RRC-processing or Texecution_time] = 10 ms.

Editor’s note: FFS whether [TLTM-RRC-processing or Texecution_time] is part of TLTM-interrupt or not.

TLTM-interrupt is as stated in section 6.X.1.2.1.

The target cell in the LTM cell switch command is known if the following conditions are met:

- Editor’s note: Conditions FFS.

Otherwise, the cell is unknown.

Editor’s note: FFS the conditions under which the target joint UL/DL TCI state or separate UL and DL TCI states in the LTM cell switch command are known, and whether to define common or separate conditions for DL and UL TCI state.

#### 6.X.1.3 Interruption time

The interruption time TLTM-interrupt is the time between end of the last TTI containing the MAC-CE command for LTM cell switch until the time the UE transmits the first UL message on the target cell, excluding Tcmd [and TLTM-RRC-processing] stated in section 6.X.1.2.

TLTM-interrupt  = TLTM-processing [+ Tfirst-RS + TRS-proc] + TLTM-IU ms,

- Where:

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- The value of TLTM-processing is 20 ms for FR1 to FR1 and FR2 to FR2 LTM cell switch.

- The value of TLTM-processing is 40 ms for FR1 to FR2 and FR2 to FR1 LTM cell switch.

Editor’s note: FFS whether a smaller value can be considered in some scenarios or under certain conditions, or whether to introduce a new UE capability to support smaller value of TLTM-processing.

Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

TRS-proc is the time for SSB processing.

Tfirst-RS = 0 and TRS-proc= 0 under the following conditions:

- The target TCI state indicated in the LTM cell switch command is in the LTM candidate cell active TCI state list, and

- L1-RSRP measurement period is not larger than 160 ms.

- Editor’s note: FFS other conditions.

Otherwise,

Tfirst-RS is the time to the first SSB transmission on the target cell [after Tcmd and [TLTM-RRC-processing or Texecution_time]].

Editor’s note: FFS whether TRS transmission is also considered.

TRS-proc = 2 ms.

Editor’s note: FFS whether Tfirst-RS and Tfirst-RS are part of TLTM-interrupt or not.

TLTM-IU is the interruption uncertainty during LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

For RACH-less LTM cell switch, TLTM-IU_is the uncertainty on transmitting the first uplink transmission on the target cell.

<< End of change 1>>