3GPP TSG-RAN WG4 Meeting # 108bis	R4-2317356

Xiamen, China, October 09 – October 13, 2023

Title: 	WF on R18 NR SL RRM requirements (part 1)

Agenda Item:	5.30.6

Source: 	LG Electronics

Document for:	Approval

- General

Issue 1-1: Specification structure

<Agreement>

- Define RRM requirements for sidelink unlicensed spectrum in clause 12 with suffix A with below example

12.2A UE Transmit Timing under CCA 12.3A Initiation/Cease of SLSS Transmissions under CCA 12.4A Selection / Reselection of Sidelink Synchronization Reference Source under CCA

Issue 1-2: The term of SyncRef UE is not available

<Agreement>

- The term SyncRef UE in SL-U is not available at the UE refers to when all the candidate S-SSB positions [monitored] in every S-SSB period are not available during the last 1280 ms; otherwise the SyncRef UE in SL-U is considered as available at the UE.

- Transmit timing requirements

Issue 2-1: Transmit timing error requirement

< Agreement>

- In R18, transmit timing requirements will be specified for 15kHz and 30kHz.

- Initiation / Cease of SLSS transmission

Issue 3-1: Requirements for SyncRef UE as a synchronization reference source (x_max)

< Agreement>

- Single value for x_max = [4]

- Selection / Reselection of V2X synchronization Reference source

Issue 4-1: Requirement for selection/reselection of SL synchronization reference source

<Agreement>

- Reuse existing requirement with adding condition below

  - “all the SSB periods are available during the Tdetect,SyncRef UE_V2X seconds.”

    - The wording can be further polished in the CR wrok.

    - E.g.,

      - UE shall not drop any sidelink data transmission for the purpose of selection/reselection to the SyncRef UE. The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2] and all the SSB periods are available during the Tdetect,SyncRef UE_V2X seconds.

Issue 4-2: Requirement for Tmeasure,PSBCH-RSRP (y_max)

<Agreement>

- Single value for y_max is specified.

  - y_max = [2]

Issue 4-3: Requirement when exceeding the maximum allowed LBT failures

<FFS>

- Option 1: Discuss UE behaviour when y exceeding y_max for measuring the PSBCH-RSRP of the current selected SyncRef UE

Issue 4-4: Requirements for fast sync SyncRef UE detection

<FFS>

- Option 1: Since both the sync SyncRef UE and async SyncRef UE detection times (1.6+x*1.6; 8+x*8 seconds) can be long with LBT failures of the newly detectable SyncRef UEs, we can speed up sync SyncRef UE search by allowing more SLSS Tx drop, e.g. allow 50% SLSS Tx drop and require 0.96+x*0.96 seconds detection time when the following conditions are satisfied

No detected SyncRef UE is available, or

The RSRP of the current SyncRef UE as sync source is lower than a threshold of z.

z can follow the SLSS evaluation threshold, or a separately configured threshold.

- Option 2: Open to discuss whether further updates are required with respect to RAN1 agreements on the number of additional SSB occasions in single RB set and more than one RB set

Issue 4-5: Additional Tx dropping for SyncRef UE detection

<FFS>

- Option 1: When GNSS is the highest priority and the sync source SyncRef UE is not directly or indirectly synchronized to GNSS, or when gNB is the highest priority,

  - an SL-U UE should be allowed to drop SLSS transmission besides data transmission to search for new SyncRef UE, at least when the current synchronization source is weak or experiencing too many consecutive LBT failures.

  - in addition to allowing 6% data Tx dropping, allowing 30% SLSS Tx dropping and the requirement for sync SyncRef UE detection applies.

- Option 2: When GNSS is the highest priority and the sync source SyncRef UE is not directly or indirectly synchronized to GNSS, or when gNB is the highest priority, reuse the legacy Tx dropping rate for Selection / Reselection of SL synchronization Reference source.

- L1 SL-RSRP measurement

Issue 5-1: SL-RSRP measurement requirement

<Agreement>

- The legacy requirements for L1 SL-RSRP measurement could be reused for SL-U based on NW configuration or pre-configuration.

- Congestion control measurement

Issue 6-1: RSSI measurement

<Agreement>

- The legacy requirements for SL RSSI measurement could be reused for SL-U.

- Interruption

Issue 7-1: Interruption requirements

<Agreement>

- The existing interruption requirements for legacy sidelink communications are reused

  - For common understanding in RAN4, on how to handle interruption requirements to WAN options like SL-DRX reconfiguration and MCSt configuration of SL transmission are already covered in the existing interruption requirements

- Others

Issue 8-1: Impact on RRM requirement due to new S-SSB design

<Agreement>

- The legacy requirement will be reused for the case with repeated S-SSB in frequency domain.

- FFS on the side condition with repetition.

- For multiple RB sets, UE is assumed to measure S-SSB on anchor RB sets only.